Arch HDL — AI Reference Card

Compact AI context for hardware generation · v0.24.0 · Put this in context, add design intent, paste compiler errors to self-correct.

1. Universal Block Schema — Every Construct Uses This

keyword Name

param NAME: const = value; // untyped int → parameter int (32-bit)

param NAME[hi:lo]: const = value; // width-qualified → parameter [hi:lo] (use to avoid WIDTHEXPAND)

param NAME: type = SomeType; // compile-time type parameter

port name: in TypeExpr;

port name: out TypeExpr;

port name: initiator BusName; // bus port (initiator perspective)

port name: target BusName; // bus port (directions flipped)

generate for i in 0..N-1 // generated ports / instances

port p[i]: in UInt<8>;

end generate for i

generate if PARAM > 0 // conditional ports

port opt: out Bool;

end generate if

assert name: expression; // (planned)

cover name: expression; // (planned)

end keyword Name

SIGNAL ASSIGNMENT:

comb y = expr; // one-line combinational (no end comb needed)

comb y = expr; end comb // equivalent multi-line form

reg r: T reset rst => 0 sync high; // register decl with reset (reset value after =)

reg r: T init 0 reset rst => 0; // optional init sets SV declaration initializer

reg p: T reset none; // register decl without reset

reg default: reset rst => 0; // wildcard default for all regs in scope

reg r: UInt<8>; // inherits reset from reg default

port reg q: out UInt<8> reset rst => 0; // output port + register combined

port reg q: out UInt<8>; // inherits reset from reg default

pipe_reg delayed: source stages 3; // N-stage delay chain, type inferred

default seq on clk rising; // set default clock for all seq blocks

seq on clk rising // clocked process — uses <=

r <= expr; // compiler auto-generates if(rst) guard

p <= expr; // no reset guard (reset none)

end seq

seq r <= expr; // one-line seq (uses default clock, no end seq)

let x: UInt<32> = a + b; // combinational wire (explicit type required)

CONDITIONALS: use elsif (one word), NOT else if (two words):

if cond_a r <= val_a; elsif cond_b r <= val_b; else r <= val_c; end if

unique if / unique match — assert mutual exclusivity; emits SV unique if / unique case:

unique if sel == 0 y = a; else y = b; end if

unique match opcode 0 => r <= a; 1 => r <= b; _ => r <= 0; end match

MATCH in comb uses = (not <=); in seq uses <=. Exhaustive enum match (all variants listed) satisfies the latch check without needing _ wildcard:

comb match color Color::Red => out = 1; Color::Green => out = 2; Color::Blue => out = 3; end match

FOR LOOPS (runtime, in comb/seq blocks):

for i in 0..7 out[i] = data[7 - i]; end for // inclusive range, emits SV for loop

for i in {0, 3, 7, 15} mask[i] = true; end for // value-list, compile-time unrolled

// Range for = runtime SV loop; value-list for = compile-time unroll; generate for = ports/instances

2. Types

UInt<N> SInt<N> Bool Bit

Clock<Domain> Reset<Sync|Async, High|Low> (polarity defaults High)

Clock<Domain> may be out — use for passthrough (comb clk_out = clk_in;), gating (comb clk_out = clk_in & en;), or division (reg toggle; seq toggle <= ~toggle; comb clk_out = toggle;). For integrated latch-based gating use the clkgate construct.

SysDomain is built-in — no domain SysDomain end domain SysDomain needed

Vec<T,N> struct S { f: T, } enum E { A, B, }

Bool and UInt<1> are identical — freely assignable, bitwise ops on 1-bit return Bool

Token Future<T> Token<T, id_width: N> (planned — TLM only)

Width conversions (always explicit): x.trunc<N>() x.zext<N>() x.sext<N>() x[hi:lo]

trunc<N>() → lowest N bits (SV: N’(x)); x[hi:lo] → bit-slice (SV: x[hi:lo])

Signedness reinterpret (same width, no N needed): signed(x) unsigned(x)

signed(UInt<8>) → SInt<8> (SV: $signed(x)); unsigned(SInt<8>) → UInt<8> (SV: $unsigned(x))

Use signed() to enter signed arithmetic chains: signed(a) + signed(b) → SInt<9>

Arithmetic: UInt<8> + UInt<8> → UInt<9> (auto-widen); must .trunc<8>() to assign back

$clog2(expr) supported in type args: UInt<$clog2(DEPTH)>

Bit concat: {a, b} → SV {a, b} (MSB first). Bit repeat: {N{expr}} → SV {N{expr}}.

Nestable: {{8{sign_bit}}, byte_val} → 16-bit sign-extended value.

3. Expressions & Operators

Arithmetic: + - * / %

Comparison: == != < > <= >=

Logical: and or not

Bitwise: & | ^ ~ << >>

Ternary: cond ? a : b (right-associative; chains for priority muxes)

Match expression: match x { E::A => val1, E::B => val2, _ => default }

Set membership: expr inside {val1, val2, lo..hi} — returns Bool, emits SV inside operator

Field access: s.field Array index: a[i]

Function call: FnName(arg1, arg2) (overload-resolved by argument types)

Enum variant: E::Variant Struct literal: S { f: val }

Sized literals: 8’hFF 16’d1024 4’b1010 (Verilog-style)

todo! — compilable placeholder; warns at compile, aborts at sim runtime

4. Construct Cards

module — combinational or registered logic

module Name ¦ No implicit latches: comb block ¦ ¦ signals must be assigned on ALL ¦ param W: const = 8; ¦ paths (missing else = error). ¦ ¦ Single driver per signal (error) ¦ local param ADDR: const = $clog2(W); ¦ localparam: not overridable ¦ ¦ ¦ param CFG: Mode = Mode::Fast; ¦ Enum-typed param (type-safe) ¦ ¦ ¦ port clk: in Clock<D>; ¦ All ports must be connected ¦ ¦ ¦ port rst: in Reset<Sync>; ¦ ¦ ¦ ¦ port a: in UInt<W>; ¦ ¦ ¦ ¦ port y: out UInt<W>; ¦ ¦ ¦ ¦ reg default: reset rst => 0; ¦ Wildcard default for all regs ¦ ¦ reg r: UInt<W>; ¦ Inherits reset from default ¦ ¦ ¦ port reg y: out UInt<W>; ¦ Output port + register combined ¦ ¦ ¦ pipe_reg d: r stages 2; ¦ 2-stage delay of r (read-only) ¦ ¦ ¦ default seq on clk rising; ¦ Set default clock for seq blocks ¦ ¦ ¦ seq on clk rising ¦ Compiler auto-generates ¦ ¦ ¦ r <= a; ¦ if(rst) reset guard from reg decl¦ ¦ ¦ y <= r; ¦ port reg assigned directly ¦ ¦ ¦ end seq ¦ ¦ ¦ ¦ seq r <= a; ¦ One-line seq (uses default clk) ¦ ¦ ¦ end module Name ¦ ¦ ¦ ¦ // Instantiation: ¦ ¦ ¦ ¦ inst u: Name ¦ ¦ ¦ ¦ param W = 16; ¦ ¦ ¦ ¦ clk <- clk; ¦ ¦ ¦ ¦ a <- sig; y -> out; ¦ ¦ ¦ ¦ // Port group member syntax: ¦ Flattened at parse time: ¦ ¦ ¦ write.en <- wr_en; ¦ write.en → write_en ¦ ¦ ¦ read[0].addr <- sel; ¦ read[0].addr → read0_addr ¦ ¦ ¦ read[1].data -> out_b; ¦ Index must be integer literal. ¦ ¦ ¦ // Whole-bus connection: ¦ Expands to per-signal connects ¦ ¦ using bus definition. Directions ¦ axi_rd -> m_axi_mm2s; ¦ derived from bus + perspective. ¦ ¦ ¦ // Indexed bus port in comb/seq: ¦ Flattened at codegen time: ¦ ¦ ¦ m_axi[0].ar_valid = true; ¦ → m_axi_0_ar_valid ¦ ¦ ¦ end inst u ¦ ¦ ¦ ¦ // Hierarchical refs FORBIDDEN: ¦ inst_name.port is a compile ¦ ¦ error. Use y -> wire ¦ // ✗ add.sum (compile error) ¦ in the inst block and reference ¦ ¦ wire in the enclosing scope. ¦ // ✓ sum -> my_sum; ¦ ¦ // then use my_sum ¦ ¦

function — pure combinational, overloadable

function AddSat(a: UInt<8>, ¦ Pure comb — no state, no clk ¦ b: UInt<8>) ¦ -> UInt<8> ¦ Overloading: same name, different¦ ¦ arg types (mangled in SV) ¦ let sum: UInt<9> = a.zext<9>() ¦ ¦ + b.zext<9>(); ¦ let bindings as temporaries ¦ ¦ ¦ return sum[8] ? 8’hFF ¦ Ternary / match in return ¦ : sum[7:0]; ¦ ¦ ¦ Emits SV: function automatic ¦ end function AddSat ¦ ¦

pipeline — staged datapath, compiler generates hazard logic

| pipeline Name ¦ Compiler generates: ¦ port clk: in Clock<D>; ¦ per-stage valid_r registers, ¦ port rst: in Reset<Sync>; ¦ stall chain (backpressure), ¦ stage Fetch stall when !in_valid ¦ flush masks, comb wire decls. ¦ reg r1: T reset rst => 0; ¦ Cross-stage refs rewritten: ¦ ¦ seq on clk rising ¦ Fetch.pc → fetch_pc ¦ r1 <= in; ¦ ¦ end seq ¦ valid_r accessible per-stage ¦ end stage Fetch ¦ for output gating: ¦ ¦ valid_r <= start; in stage0 ¦ seq overrides default (=1). ¦ stage Exec ¦ done = valid_r; in last stage ¦ reg r2: T reset rst => 0; ¦ ¦ ¦ seq on clk rising ¦ Explicit forwarding via comb ¦ r2 <= Fetch.r1; ¦ if/else mux inside stage. ¦ end seq ¦ ¦ inst alu0: Alu ¦ inst inside stages supported ¦ a <- Fetch.r1; ¦ (output wires auto-declared). ¦ ¦ end inst alu0 ¦ ¦ end stage Exec ¦ ¦ flush Fetch when mispredict; ¦ ¦ end pipeline Name ¦ |  |
| --- | --- |

fsm — finite state machine

| fsm Name ¦ Compiler checks exhaustive transitions. ¦ port clk: in Clock<D>; ¦ default state required (reset value). ¦ ¦ port rst: in Reset<Sync>; ¦ default ... end default block provides ¦ default comb/seq assignments emitted port active: out Bool; ¦ before the state case statement; states ¦ only override what differs. port fire_irq: out Bool; ¦ ¦ ¦ Undriven outputs are X (real HW). state [Idle, Running, Done] ¦ ¦ default state Idle; ¦ ¦ default ¦ comb ¦ active = false; ¦ fire_irq = false; ¦ end comb ¦ end default ¦ ¦ state Idle ¦ Transition syntax: -> Running when start; ¦ end state Idle ¦ -> Next when <expr>; ¦ state Running ¦ Multiple transitions are checked for ¦ mutual exclusivity; unique if emitted comb ¦ when exclusive, priority if otherwise. active = true; ¦ Implicit hold: if no transition fires, end comb ¦ FSM stays in current state. No catch-all ¦ -> Self when true needed. -> Done when all_done; ¦ ¦ -> Running when not all_done; ¦ Unconditional transition (no when): ¦ ¦ omit when to always advance. end state Running ¦ ¦ state Done ¦ ¦ comb ¦ fire_irq = true; ¦ end comb ¦ ¦ -> Idle; ¦ ¦ end state Done ¦ ¦ end fsm Name ¦ |  |
| --- | --- |

fsm datapath extension — reg, let, and seq inside FSMs:

FSMs may declare reg and let at scope level, and seq on clk rising ... end seq blocks inside state bodies. The compiler emits separate always_ff (state + datapath regs) and always_comb (transitions + outputs). This co-locates control and datapath — a readability win over SV’s split-block style.

fsm MulDiv
  reg acc_r: UInt<64> reset rst => 0 sync high;
  let done: Bool = (cycle_r == 31);
  state Idle
    seq on clk rising
      acc_r <= 0;
    end seq
    -> Multiply when req_valid;
  end state Idle
  ...
end fsm MulDiv

fifo — sync or dual-clock async (gray-code auto-generated); kind: fifo (default) | lifo

| fifo Name kind lifo; // optional, default fifo param DEPTH: const = 64; param WIDTH: type = UInt<32>; port clk: in Clock<D>; // or wr_clk+rd_clk for async port rst: in Reset<Sync>; port push_valid: in Bool; port push_ready: out Bool; port push_data: in WIDTH; port pop_valid: out Bool; port pop_ready: in Bool; port pop_data: out WIDTH; end fifo Name | Dual-clock: replace clk with port wr_clk: in Clock<WrD>; port rd_clk: in Clock<RdD>; Compiler adds gray-code CDC. kind lifo restricted to single-clock only. |
| --- | --- |

synchronizer — CDC synchronizer (ff / gray / handshake / reset / pulse)

synchronizer Name

kind ff; // ff (default) | gray | handshake | reset | pulse

param STAGES: const = 2; // 2 or 3 (default 2)

port src_clk: in Clock<SrcDomain>;

port dst_clk: in Clock<DstDomain>; // chain clocked on dst_clk

port rst: in Reset<Async>; // optional

port data_in: in Bool; // or UInt<N> for multi-bit

port data_out: out Bool;

end synchronizer Name

Strategies:

- kind ff; (default) — N-stage flip-flop shift chain on dst clock. Best for 1-bit signals.

- kind gray; — Binary-to-gray encode, FF chain, gray-to-binary decode. Safe for multi-bit counters/pointers.

- kind handshake; — Req/ack toggle protocol with synchronized control signals. Safe for arbitrary multi-bit data.

- kind reset; — Reset synchronizer: async assert (immediate), sync deassert through N-stage FF chain. Single-bit only (Bool). Used for synchronizing reset deassertion to a clock domain.

- kind pulse; — Pulse synchronizer: converts a single-cycle pulse into a level toggle in the source domain, syncs the toggle through the FF chain, then edge-detects in the destination domain to regenerate a single-cycle pulse. Single-bit only (Bool). Used for events, interrupts, triggers across clock domains.

Notes: two Clock ports must reference different domains (compile error otherwise). SV codegen emits strategy-specific logic; sim codegen generates C++ models for all 5 kinds.

Type checker warnings/errors: kind ff with multi-bit data (UInt<N> where N>1) warns — use kind gray or kind handshake instead. kind reset and kind pulse error if data is not Bool.

Compiler Warnings

| Warning | Cause | Fix |
| --- | --- | --- |
| todo! site | todo! used as placeholder | Replace with real logic |
| Redundant reset branch | if rst at top of seq body when regs have reset rst=>value | Remove the if rst branch — the declaration generates the outer guard automatically |

CDC detection covers both seq→seq and comb→seq crossings: a comb block reading a register from domain A whose output feeds a seq block in domain B is a compile error. CDC checking also extends across inst boundaries — the compiler traces clock port connections to map child domains to parent domains and flags any data connection that crosses domain boundaries without a synchronizer or async fifo.

ram — FPGA BRAM / ASIC SRAM

| ram Name param DEPTH: const = 1024; port clk: in Clock<D>; kind simple_dual; latency 1; store weights: Vec<SInt<8>, DEPTH>; biases: Vec<SInt<16>, DEPTH>; end store port rd en: in Bool; addr: in UInt<10>; data: out SInt<8>; end port rd port wr en: in Bool; addr: in UInt<10>; data: in SInt<8>; end port wr init: zero; end ram Name | kind: single¦simple_dual¦true_dual ¦ ¦ latency 0 = async (comb read) ¦ latency 1 = sync (1-cycle read) ¦ latency 2 = sync_out (2-cycle read) ¦ ¦ init: zero¦none¦file 'x.hex' ¦ ¦ store: multiple named logical ¦ address ranges (planned). ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ |
| --- | --- |

counter — wrap/saturate/gray/one_hot/johnson

| counter Name param WIDTH: const = 8; port clk: in Clock<D>; port rst: in Reset<Sync>; port en: in Bool; port count: out UInt<WIDTH>; port at_max: out Bool; kind wrap; direction: up; end counter Name | kind wrap¦saturate¦gray¦ one_hot¦johnson direction: up¦down¦up_down at_max / at_min output ports |
| --- | --- |

arbiter — N requesters, policy-driven grant with hook + latency

| arbiter Name ¦ Built-in policies: ¦ round_robin, priority, param N: const = 4; ¦ lru, weighted ¦ port clk: in Clock<D>; ¦ Custom policy: use function ¦ name as policy + hook decl: port rst: in Reset<Sync>; ¦ ¦ policy: MyGrantFn; ports[N] req ¦ hook grant_select( ¦ req_mask: UInt<N>, valid: in Bool; ready: out Bool; ¦ last_grant: UInt<N>, ¦ extra_port: UInt<8>) end ports req ¦ -> UInt<N> ¦ = MyGrantFn(req_mask, port grant_valid: out Bool; ¦ last_grant, extra_port); ¦ port grant_requester: out UInt<$clog2(N)>; ¦ Hook args bind to: ¦ ¦ hook params (internal sigs), policy: round_robin; ¦ user-declared ports/params ¦ latency 1; // default: comb grant ¦ latency 2 = +1 pipeline stage ¦ ¦ latency 3 = +2 pipeline stages end arbiter Name ¦ |  |
| --- | --- |

regfile — multi-port register file

| regfile Name param DEPTH: const = 32; param WIDTH: const = 32; port clk: in Clock<D>; port rst: in Reset<Sync>; port rd0 addr: in UInt<5>; data: out UInt<WIDTH>; end port rd0 port wr0 en: in Bool; addr: in UInt<5>; data: in UInt<WIDTH>; end port wr0 forward write_before_read: false; init [0] = 0; end regfile Name | Multiple read + write ports forward write_before_read: true enables bypass forwarding init [i] = v; sets reset values ¦ |
| --- | --- |

linklist — singly/doubly/circular linked list

| linklist Name param DEPTH: const = 256; param DATA_WIDTH: const = 32; port clk: in Clock<D>; port rst: in Reset<Sync>; variant: doubly; end linklist Name | variant: singly¦doubly¦ circular_singly¦circular_doubly Operations (via op port): insert_head, insert_tail, insert_after, delete_head, delete, next, prev (doubly), alloc, free, read_data, write_data Built-in free list + FSM controller 2-cycle latency per operation |
| --- | --- |

generate — compile-time ports and instances

| generate for i in 0..SIZE-1 ¦ Generates REAL named ports. ¦ port a[i]: in SInt<8>; ¦ Caller uses: a[0], a[3], etc. ¦ inst pe[i]: ProcElem ¦ Type-checked per index. ¦ clk <- clk; ¦ Boundary expression handles ¦ ¦ a_in <- a[i]; ¦ chain wiring: i==0 ? 0 : prev ¦ ¦ sum_in <- ¦ generate if: port does NOT ¦ ¦ i==0 ? 0 : pe[i-1].sum_out; ¦ exist when condition false. ¦ end inst pe[i] ¦ ¦ end generate for i ¦ ¦ generate if DEBUG_EN ¦ ¦ port dbg: out UInt<32>; ¦ ¦ end generate if ¦ ¦ // accessing dbg when DEBUG_EN=false ¦ ¦ // is a COMPILE ERROR ¦ |  |
| --- | --- |

bus — reusable port bundle with initiator/target perspectives

| bus AxiLite ¦ Signals from initiator’s perspective ¦ param ADDR_W: const = 32; ¦ initiator keeps directions ¦ aw_valid: out Bool; ¦ target flips all directions ¦ aw_ready: in Bool; ¦ Usage in module: ¦ aw_addr: out UInt<ADDR_W>; ¦ port axi: initiator AxiLite; ¦ ¦ end bus AxiLite ¦ port axi: target AxiLite; ¦ ¦ Access: axi.aw_valid (dot notation) ¦ ¦ SV: flattened axi_aw_valid, etc. |  |
| --- | --- |

template — user-defined interface contract

| template MyInterface param NUM_REQ: const; port clk: in Clock<D>; port rst: in Reset<Sync>; port grant_valid: out Bool; hook grant_select( req_mask: UInt<4>) -> UInt<4>; end template MyInterface | Compile-time only — no SV emitted Defines required params, ports, and hooks that implementing modules must provide. Missing any required item is a compile error. Modules opt in with: module Foo implements MyInterface Hooks in template: signature only Hooks in module: + binding = FnName(args); |
| --- | --- |

package — reusable type/function namespace

| package BusPkg enum BusOp Read, Write, Idle end enum BusOp domain FastClk freq_mhz: 500 end domain FastClk struct BusReq op: BusOp; addr: UInt<32>; data: UInt<32>; end struct BusReq function max(a: UInt<32>, b: UInt<32>) -> UInt<32> return a > b ? a : b; end function max end package BusPkg | Contains: enum, struct, function, param, domain declarations. No modules/pipelines/FSMs inside. domain inside package: share clock domains across files via use ClockPkg; File: PkgName.arch (one package per file, name must match). Consumer imports with: use BusPkg; Emits SV: package BusPkg; … endpackage import BusPkg::*; Resolved from same directory or multi-file command line. |
| --- | --- |

5. Logging

log(Level, “TAG”, “format %0d”, arg);

Levels: Always, Low, Medium, High, Full, Debug

Works in seq and comb blocks

Runtime control: +arch_verbosity=N (0=Always only … 5=Debug)

NBA semantics in seq: value printed is last cycle’s registered value

6. TLM Concurrency Modes (planned)

blocking ret: T directly — caller suspends until done — APB/MMIO

pipelined ret: Future<T> — issue many, await later — AXI in-order

out_of_order ret: Token<T,id: N> — any-order response by ID — Full AXI

burst ret: Future<Vec<T,L>>— one AR, N data beats — AXI INCR burst

await f // wait for one Future

await_all(f0,f1,f2) // wait for all

await_any(t0,t1) // first Token to complete (out_of_order only)

7. Simulation & Build

arch check F.arch // type-check only

arch build F.arch // emit SystemVerilog

arch build F.arch -o out.sv // combined output

arch build a.arch b.arch // multi-file: one .sv per input

arch sim F.arch –tb F_tb.cpp // compile + run C++ testbench

arch sim F.arch –tb F_tb.cpp –outdir build/

arch sim F.arch –tb F_tb.cpp –check-uninit // warn on reads of uninitialized reset-none regs

arch sim F.arch –tb F_tb.cpp –cdc-random // randomize synchronizer latency; cdc_skip_pct (0-100, default 25) controllable from testbench

arch sim F.arch // generate models only (no testbench)

arch formal F.arch // emit SMT-LIB2 (planned)

arch sim C++ testbench interface (Verilator-compatible)

// Ports map to public fields: dut->clk, dut->rst, dut->data_in

// Wide ports (UInt<N> where N>64) use VlWide<WORDS>:

// dut->key.data() returns uint32_t* (word 0 = LSB, word N-1 = MSB)

// Standard cycle loop:

auto tick = [&]() { dut->clk=0; dut->eval(); dut->clk=1; dut->eval(); };

// Multi-clock modules: compiler auto-generates tick() from domain freq_mhz.

// tick() toggles each clock at the correct frequency ratio and calls eval().

dut.tick(); // auto-toggles fast_clk (200MHz) and slow_clk (50MHz) at 4:1 ratio

// Manual control also works — each seq block fires only on its own clock’s rising edge.

// Compile: g++ -std=c++17 build/verilated.cpp build/V*.cpp tb.cpp -Ibuild -o sim

8. AI Prompting Patterns

1. CONSTRUCT-FIRST (most reliable)

’Generate an Arch fifo named InstrQueue, depth 64, element type

InstrPacket, single clock SysDomain. Add cover push_when_full.’

2. todo! SCAFFOLDING

’Generate the skeleton for a 5-stage RISC-V pipeline.

Use todo! for all stage bodies.’

3. PASTE COMPILER ERRORS

‘Fix this Arch error: [paste arch check output]’

Errors are self-sufficient — no spec lookup needed.

4. ONE CONSTRUCT PER PROMPT

structs → functions → primitives → pipeline → top module → testbench

Compile and verify each before moving to the next.

5. ABSTRACTION PROGRESSION

Start –tlm-lt. Add rtl_accurate only after function verified.

9. ARCH MCP Server

Tools available when running under the ARCH MCP server:

get_construct_syntax(construct) — returns syntax template + reserved keywords for any construct

write_and_check(path, content) — write .arch file + type-check in one call

arch_build_and_lint(files, top_module) — build SV + Verilator lint in one call

Recommended AI workflow: fetch syntax first → write_and_check → arch_build_and_lint

Arch AI Reference Card · March 2026 · v0.24.0 · arch check is your first line of defence