ARCH

Hardware Description Language

Language Specification  ·  v0.1  ·  March 2026

A purpose-built micro-architecture HDL — clean RTL semantics, ownership-aware types, and first-class pipelines, FSMs, FIFOs, and arbiters.  Designed to be generated correctly by AI without prior training.

1.  Design Philosophy

Arch is a hardware description language built from first principles for micro-architecture work.  It carries no host-language baggage — no JVM, no Python runtime, no Scala implicits.  Every construct maps directly to a concrete hardware structure.  The compiler emits deterministic, readable SystemVerilog compatible with any standard EDA tool.

| No Baggage | No JVM, no Python runtime, no Scala implicits. Every keyword maps directly to a hardware structure. Nothing is a library pattern hiding behind operator overloading. |
| --- | --- |
| Strong Types | Bit widths, clock domains, port directions, and signal ownership are tracked statically. Every mismatch is a compile-time error — never a simulation-time surprise. |
| Micro-Arch First | Pipeline, FSM, FIFO, Arbiter, and RegFile are first-class language keywords with compiler-verified semantics — not user-defined patterns bolted onto raw RTL. |
| AI-Generatable | Uniform schema, named block endings, English keywords, no braces, and a todo! escape hatch let any LLM produce valid, correct Arch code from a natural-language description — without fine-tuning. |
| Predictable RTL | One Arch construct always produces the same SystemVerilog structure. Designers can audit every output line. Synthesis tools see clean, idiomatic RTL. |

1.1  The AI-Generatability Contract

Arch makes a hard design commitment: a large language model that has read this specification and nothing else must be able to generate structurally correct, type-safe Arch from a plain-English hardware description.  Every syntactic choice serves this goal:

- Uniform schema: every construct uses the identical param / port / body / verification layout.  Learn it once; apply it everywhere.

- Named block endings: every block closes with end keyword name.  The AI never loses context in deeply nested structures.

- No braces: the keyword+name header and end keyword name are the sole delimiters.  There is no redundant { to emit or forget.

- Intention-first syntax: the construct kind always comes first — pipeline Decode, fsm TrafficLight.  What it is precedes all detail.

- One assignment, one meaning: comb blocks use = (combinational); reg blocks use <= (registered).  No context-dependent semantics.

- todo! escape hatch: any expression or block body may be replaced with todo! to produce a compilable, type-checked skeleton.

- Required explicitness: widths, domains, directions, and policies are always written out.  There is no implicit default the AI must remember.

2.  Lexical Conventions

2.1  Naming Rules  (compiler-enforced)

| Category | Convention | Examples |
| --- | --- | --- |
| Modules, interfaces, structs, enums | PascalCase | FetchUnit, AluOp, DecodedInstr |
| Signals, registers, ports, locals | snake_case | pc_next, mem_addr, req_valid |
| Parameters and constants | UPPER_SNAKE | XLEN, CACHE_DEPTH, NUM_PORTS |
| Clock ports | typed Clock<Domain> | clk: in Clock<SysDomain> |
| Reset ports | typed Reset<Sync¦Async, High¦Low> | rst: in Reset<Sync> (polarity defaults High; e.g. Reset<Sync, Low> for active-low) |

2.2  Literals

literals.arch // All literals require explicit type ascription — no implicit widths let a: UInt<8> = 255; let b: UInt<8> = 0xFF; let c: UInt<8> = 0b1111_1111; // underscores allowed anywhere // Sized literals: width'format'value — compiler checks against ascribed type let d: UInt<4> = 4'b1010; let e: UInt<16> = 16'hDEAD; let f: SInt<12> = 12'sd-42; let g: Bool = true; let h: Bool = false;

2.3  Block Structure — The Core Grammar Rule

Every compound construct in Arch opens with a keyword-and-name header on its own line and closes with a matching end keyword name.  No braces are used anywhere.  This rule applies uniformly to every construct in the language without exception.

block_structure.arch // The universal block grammar: // // keyword Name // ... body ... // end keyword Name // // Examples across every construct type: module Alu // body end module Alu pipeline Decode stage Fetch // nested body end stage Fetch end pipeline Decode fifo TxBuffer // body end fifo TxBuffer fsm TrafficLight state Red // body end state Red end fsm TrafficLight

⚑  No braces, no ambiguity.  The opening keyword+name and closing end keyword name are always a matched pair.  An AI generating code cannot accidentally close the wrong block.

2.4  The todo! Escape Hatch

Any expression or block body may be replaced with todo! to produce a compilable skeleton.  The compiler emits a warning for every todo! site.  Simulation aborts with a clear diagnostic if a todo! site is reached at runtime.

todo_escape.arch module Multiplier param WIDTH: const = 32; port clk: in Clock<SysDomain>; port a: in UInt<WIDTH>; port b: in UInt<WIDTH>; port result: out UInt<WIDTH>; comb result = todo!; // WARN: todo! at Multiplier::result end comb end module Multiplier

3.  Type System

The Arch type system enforces four independent safety dimensions simultaneously.  A signal that satisfies all four is guaranteed correct-by-construction — no simulation required to catch structural errors.

3.1  Primitive Types

| Type | Hardware Width | Notes |
| --- | --- | --- |
| Bit | 1 bit | Raw logic bit. Not Bool — use only for bit-level manipulation. |
| UInt<N> | N bits | Unsigned integer. N must be a const expression (literal or param). |
| SInt<N> | N bits | Two's-complement signed integer. |
| Bool | 1 bit | Logical boolean. Not implicitly castable to/from UInt or Bit. |
| Clock<D> | 1 bit | Carries clock-domain tag D. Cannot appear in arithmetic. |
| Reset<Sync> | 1 bit | Synchronous reset — deasserted on the clock edge. |
| Reset<Async> | 1 bit | Asynchronous reset — deasserted immediately. |
| Vec<T,N> | N × ¦T¦ | Fixed-size array of any hardware type T. |
| struct S | Σ fields | Named aggregate. Width = sum of field widths (packed). |
| enum E | ⌈log₂n⌉ bits | Discriminated union. Compiler picks minimum encoding width. |
| Token | 0 bits | Pure handshake carrier. Holds valid/ready signals; zero data width. |

3.2  Bit-Width Safety

Every assignment, port connection, and arithmetic result is width-checked at compile time.  There is no implicit truncation, zero-extension, or sign-extension anywhere in the language.

width_safety.arch let a: UInt<8> = 0xFF; let b: UInt<16> = 0x1234; // ✗ COMPILE ERROR — cannot assign UInt<16> to UInt<8> let bad: UInt<8> = b; // ✓ Explicit operations — intent is always visible let lo: UInt<8> = b.trunc<8>(); let ext: UInt<16> = a.zext<16>(); let sx: SInt<16> = (a as SInt<8>).sext<16>(); // Arithmetic: result widths conservatively inferred; must match ascription let sum: UInt<9> = a + a; // UInt<8> + UInt<8> → UInt<9> let wide: UInt<24> = a.zext<24>() * b.zext<24>();

⚑  Width inference follows IEEE 1800-2012 §11.6.  Arch promotes all mismatches to hard errors — never warnings.

3.3  Struct and Enum Types

struct_enum.arch /// A decoded RISC-V instruction — packed struct, total 57 bits struct DecodedInstr opcode: UInt<7>, rd: UInt<5>, rs1: UInt<5>, rs2: UInt<5>, funct3: UInt<3>, imm: SInt<32>, end struct DecodedInstr /// ALU operation selector — compiler assigns 4-bit encoding enum AluOp Add, Sub, And, Or, Xor, Sll, Srl, Sra, Slt, Sltu, end enum AluOp // Struct literal let instr: DecodedInstr = DecodedInstr opcode: 7'b0110011, rd: 5'd1, rs1: 5'd2, rs2: 5'd3, funct3: 3'b000, imm: 0, end DecodedInstr // Field access let dst: UInt<5> = instr.rd; // Enum match — must be exhaustive let result: UInt<32> = match op AluOp::Add => a + b, AluOp::Sub => a - b, AluOp::And => a & b, _ => todo!, end match

3.4  Signal Ownership  (Single-Driver Rule)

Every signal in Arch has exactly one driver — the block (comb or reg) that assigns it.  The compiler enforces this statically.  Multiple drivers, floating signals, and unintentional latches are all compile-time errors.

ownership.arch module Example port clk: in Clock<SysDomain>; port a: in UInt<8>; port b: in UInt<8>; port y: out UInt<9>; comb y = a.zext<9>() + b.zext<9>(); end comb // ✗ COMPILE ERROR — y is already driven by the comb block above // comb // y = 0; // end comb end module Example

⚑  The single-driver rule eliminates the most common class of RTL bugs: multiply-driven nets and unintentional latches.  It also makes AI-generated code safe — the compiler rejects any double-assignment.

4.  Modules

A module is the fundamental unit of design in Arch.  Every module follows the same four-section schema — params, ports, body, optional verification.  This regularity is intentional: an AI encountering any Arch construct can immediately orient itself using the same mental model.

4.1  Declaration Schema

module_schema.arch // The universal Arch schema — identical across every first-class construct: // // keyword Name // param NAME: const = value; // section 1: compile-time parameters // param NAME: type = SomeType; // // port name: dir Type; // section 2: interface ports // // <construct-specific body> // section 3: logic / structure // // assert / cover / assume ...; // section 4: verification (optional) // // end keyword Name module Adder param WIDTH: const = 8; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port a: in UInt<WIDTH>; port b: in UInt<WIDTH>; port sum: out UInt<WIDTH+1>; comb sum = a.zext<WIDTH+1>() + b.zext<WIDTH+1>(); end comb end module Adder

4.2  Combinational vs Registered Blocks

Arch has exactly two assignment forms.  Mixing operators between them is a compile error.

| Block | Keyword | Operator | Hardware Meaning |
| --- | --- | --- | --- |
| Combinational | comb ... end comb | = | Continuous assignment; no flip-flop inferred. |
| Registered | reg ... end reg | <= | Clocked assignment; flip-flop inferred on the active clock edge. |

counter.arch module Counter param WIDTH: const = 8; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port en: in Bool; port count: out UInt<WIDTH>; reg count_r: UInt<WIDTH> init 0; reg on clk rising, rst high if rst count_r <= 0; end if else if en count_r <= count_r + 1; end if end else end reg comb count = count_r; end comb end module Counter

⚑  on clk rising, rst high is the only place a clock and reset are named. The compiler propagates domain membership automatically through all downstream logic in the module.

4.2.1 Conditional Statements: if / elsif / else

Arch uses elsif (one word) for chained conditionals, not else if (two words). In a brace-free language, else if is ambiguous — does else start a new body block, or does else if chain? The elsif keyword resolves this unambiguously:

if condition_a

r <= val_a;

elsif condition_b

r <= val_b;

elsif condition_c

r <= val_c;

else

r <= val_default;

end if

The same syntax applies in both seq and comb blocks. A chain begins with if, continues with zero or more elsif branches, optionally ends with else, and is always closed by a single end if. The compiler emits standard SystemVerilog if / else if / else from this syntax.

4.2.1a For Loops in comb and seq Blocks

A for loop iterates over an inclusive integer range inside a comb or seq block. The syntax is:

for VAR in START..END

// body — VAR is an integer variable usable as an index

end for

The range START..END is inclusive (both endpoints are visited). The compiler emits a SystemVerilog for loop:

SystemVerilog

for (int VAR = START; VAR <= END; VAR++) begin

// body

end

Example in a comb block:

comb

for i in 0..7

out[i] = data[7 - i];

end for

end comb

Example in a seq block:

seq on clk rising

for i in 0..3

shift_r[i] <= shift_r[i + 1];

end for

shift_r[3] <= data_in;

end seq

⚑ This `for` construct is a runtime loop emitted as a SV `for` statement. It differs from `generate for`, which is a compile-time unrolling that creates distinct ports and instances.

4.2.1b For Loops with Value Lists

A for loop may also iterate over an explicit value list enclosed in braces. The syntax is:

for VAR in {val1, val2, val3}

// body — VAR takes each value in turn

end for

The compiler unrolls the loop at compile time, emitting one copy of the body per value. This is useful when the iteration values are non-contiguous or follow no simple range pattern.

Example in a comb block:

comb

for i in {0, 3, 7, 15}

mask[i] = true;

end for

end comb

The compiler emits one assignment per value:

SystemVerilog

mask[0] = 1'b1;

mask[3] = 1'b1;

mask[7] = 1'b1;

mask[15] = 1'b1;

⚑ Value-list `for` is compile-time unrolled (like `generate for`), not a runtime SV `for` loop. Use range-based `for i in 0..N` for runtime loops.

4.2.1c The `inside` Set Membership Operator

The inside operator tests whether an expression matches any value or falls within any range in a set. It returns Bool.

expr inside {val1, val2, lo..hi}

Individual values and inclusive ranges (lo..hi) may be freely mixed inside the braces. The compiler emits the SystemVerilog inside operator directly:

SystemVerilog

expr inside {val1, val2, [lo:hi]}

Example:

let is_special: Bool = opcode inside {3, 7, 16..31};

Emits:

SystemVerilog

assign is_special = opcode inside {3, 7, [16:31]};

The inside expression can be used anywhere a Bool expression is valid — in if conditions, ternary operands, comb assignments, transition when guards, etc.

4.2.1d The `unique` Modifier for `if` and `match`

The unique keyword may be prepended to any if or match statement to assert to the synthesis tool that all conditions are mutually exclusive. The compiler passes the unique qualifier directly to SystemVerilog, enabling parallel mux inference instead of priority encoding.

unique if sel == 0

y = a;

else

y = b;

end if

unique match opcode

0 => result <= a;

1 => result <= b;

_ => result <= 0;

end match

The emitted SystemVerilog uses unique if (...) and unique case (...) respectively. Use unique when you know the conditions cannot overlap and want the synthesis tool to optimize accordingly. Omit it when conditions may overlap and priority resolution is required.

4.2.2 Bit Concatenation and Replication

Arch uses standard SystemVerilog syntax for bit concatenation and replication:

let word: UInt<16> = {high_byte, low_byte}; // concatenation (MSB first)

let sign_ext: UInt<8> = {8{sign_bit}}; // replication

let sext32: UInt<32> = {{24{sign_bit}}, byte_val}; // replication inside concat

{a, b, c} concatenates operands MSB-first, producing a UInt whose width is the sum of all operand widths. {N{expr}} replicates expr N times. Replication may be nested inside concatenation.

4.2.3 Signal Declarations: let, wire, and reg

Arch has three kinds of module-scope signal declarations. Each has a distinct syntax, assignment location, and SV equivalent.

| Construct | Syntax | Assigned in | SV equivalent |
| --- | --- | --- | --- |
| let | let x: T = expr; | declaration (fixed combinational expr) | logic [W-1:0] x; assign x = expr; |
| wire | wire x: T; | comb` block (`=`) | logic [W-1:0] x;` (driven in `assign`/`always_comb`) |
| reg | reg x: T [init V] [reset R=>V]; | seq` block (`<=`) | logic [W-1:0] x = V;` (driven in `always_ff`) |

`let` declares a combinational binding fixed to a single expression at the declaration site. It cannot appear on the left-hand side of a comb or seq block.

`wire` declares an explicitly-typed combinational net with no initializer. It must be driven exactly once inside a comb block using =. This is the right choice when the combinational expression is too complex for a single inline let, or when the wire must be conditionally driven using if/elsif/else inside the comb block. The type checker enforces that only wire declarations and output ports are valid targets for comb assignment — assigning to a reg inside a comb block is a compile error.

`reg` declares a flip-flop. It must be assigned inside a seq block using <=. Reset polarity and mode are declared per register. The syntax is reg x: T [init VALUE] [reset SIGNAL=>VALUE]; where init is optional (sets only the SV declaration initializer logic x = VALUE;) and reset SIGNAL=>VALUE specifies both the reset signal and the value to load on reset. Use reset none for registers that should not be reset. A reg default: declaration sets the default init and reset for all subsequent registers in scope: reg default: [init VALUE] reset SIGNAL=>VALUE;.

`port reg` declares an output port that is also a register, eliminating the common reg r + comb out = r; end comb boilerplate. The syntax is port reg name: out T [init V] [reset R=>V];. It can only be used on output ports (in direction is a compile error). The port is assigned with <= inside a seq block, just like a regular reg. If reg default: is in scope, it inherits the default init and reset. In generated SV, the port is declared as output logic [W-1:0] name and driven directly in the always_ff block.

module Mux2

port sel: in Bool;

port a: in UInt<8>;

port b: in UInt<8>;

port y: out UInt<8>;

wire result: UInt<8>; // declared here, driven in comb below

comb

if sel

result = a;

else

result = b;

end if

y = result;

end comb

end module Mux2

⚑ The type checker enforces: `reg` cannot be a `comb` target (error: "cannot assign to register `x` in a comb block; use `<=` inside a seq block"). Only `wire` declarations and output ports are valid comb targets.

`multicycle` reg annotation (planned) — reg result: UInt<32> multicycle 3 reset rst => 0; declares that the combinational path feeding this register has a multi-cycle timing budget. Unlike pipe_reg (which inserts N physical flip-flop stages), a multicycle register remains a single flop — no extra area or power. The compiler emits an SDC constraint (set_multicycle_path N -to result) and can statically verify that consumers only sample the value at the correct rate. This is useful for slow-settling operations (multipliers, dividers, complex ALU) where the path does not affect end-to-end throughput.

The Counter example from §4.2 can be simplified using port reg:

module Counter

param WIDTH: const = 8;

port clk: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port en: in Bool;

port reg count: out UInt<WIDTH> reset rst => 0;

seq on clk rising

if en

count <= (count + 1).trunc<WIDTH>();

end if

end seq

end module Counter

⚑ `port reg` eliminates the intermediate `reg count_r` and the `comb count = count_r; end comb` wiring block. The port is directly assigned in the `seq` block. This is the preferred style when the output port is a straightforward registered value.

`multicycle` reg annotation (planned) — reg result: UInt<32> multicycle 3 reset rst => 0; declares that the combinational path feeding this register has a multi-cycle timing budget. Unlike pipe_reg (which inserts N physical flip-flop stages), a multicycle register remains a single flop — no extra area or power. The compiler auto-detects all input signals feeding the register by walking the assignment expression tree. Three modes of enforcement: (1) Simulation (–check-uninit): hidden valid tracking with input change detection and latency counter; reads before the counter expires return poison/X. (2) Synthesis: SDC constraint generation (set_multicycle_path N -to result). (3) Formal: optional assert property to verify the multicycle timing assumption holds.

4.2.4 Width-Qualified Parameters

An untyped param NAME: const = value emits parameter int NAME = value in SystemVerilog (32-bit signed). When a parameter is compared or assigned against a specific-width signal, this width mismatch produces Verilator WIDTHEXPAND warnings. Use the bracket form to declare the parameter's bit width explicitly:

param H_ACTIVE[9:0]: const = 640; // emits: parameter [9:0] H_ACTIVE = 640

param H_FRONT[9:0]: const = 16; // emits: parameter [9:0] H_FRONT = 16

The compiler validates that the default value fits within the declared range. Comparisons with UInt<10> signals are now width-matched and Verilator-clean.

4.2.1 Conditional Statements: if / elsif / else

Arch uses elsif (one word) for chained conditionals, not else if (two words). In a brace-free language, else if is ambiguous — does else start a new body block, or does else if chain? The elsif keyword resolves this unambiguously:

if condition_a

r <= val_a;

elsif condition_b

r <= val_b;

elsif condition_c

r <= val_c;

else

r <= val_default;

end if

The same syntax applies in both seq and comb blocks. A chain begins with if, continues with zero or more elsif branches, optionally ends with else, and is always closed by a single end if. The compiler emits standard SystemVerilog if / else if / else from this syntax.

4.2.1a For Loops in comb and seq Blocks

A for loop iterates over an inclusive integer range inside a comb or seq block. The syntax is:

for VAR in START..END

// body — VAR is an integer variable usable as an index

end for

The range START..END is inclusive (both endpoints are visited). The compiler emits a SystemVerilog for loop:

SystemVerilog

for (int VAR = START; VAR <= END; VAR++) begin

// body

end

Example in a comb block:

comb

for i in 0..7

out[i] = data[7 - i];

end for

end comb

Example in a seq block:

seq on clk rising

for i in 0..3

shift_r[i] <= shift_r[i + 1];

end for

shift_r[3] <= data_in;

end seq

⚑ This `for` construct is a runtime loop emitted as a SV `for` statement. It differs from `generate for`, which is a compile-time unrolling that creates distinct ports and instances.

4.2.1b For Loops with Value Lists

A for loop may also iterate over an explicit value list enclosed in braces. The syntax is:

for VAR in {val1, val2, val3}

// body — VAR takes each value in turn

end for

The compiler unrolls the loop at compile time, emitting one copy of the body per value. This is useful when the iteration values are non-contiguous or follow no simple range pattern.

Example in a comb block:

comb

for i in {0, 3, 7, 15}

mask[i] = true;

end for

end comb

The compiler emits one assignment per value:

SystemVerilog

mask[0] = 1'b1;

mask[3] = 1'b1;

mask[7] = 1'b1;

mask[15] = 1'b1;

⚑ Value-list `for` is compile-time unrolled (like `generate for`), not a runtime SV `for` loop. Use range-based `for i in 0..N` for runtime loops.

4.2.1c The `inside` Set Membership Operator

The inside operator tests whether an expression matches any value or falls within any range in a set. It returns Bool.

expr inside {val1, val2, lo..hi}

Individual values and inclusive ranges (lo..hi) may be freely mixed inside the braces. The compiler emits the SystemVerilog inside operator directly:

SystemVerilog

expr inside {val1, val2, [lo:hi]}

Example:

let is_special: Bool = opcode inside {3, 7, 16..31};

Emits:

SystemVerilog

assign is_special = opcode inside {3, 7, [16:31]};

The inside expression can be used anywhere a Bool expression is valid — in if conditions, ternary operands, comb assignments, transition when guards, etc.

4.2.1d The `unique` Modifier for `if` and `match`

The unique keyword may be prepended to any if or match statement to assert to the synthesis tool that all conditions are mutually exclusive. The compiler passes the unique qualifier directly to SystemVerilog, enabling parallel mux inference instead of priority encoding.

unique if sel == 0

y = a;

else

y = b;

end if

unique match opcode

0 => result <= a;

1 => result <= b;

_ => result <= 0;

end match

The emitted SystemVerilog uses unique if (...) and unique case (...) respectively. Use unique when you know the conditions cannot overlap and want the synthesis tool to optimize accordingly. Omit it when conditions may overlap and priority resolution is required.

4.2.2 Bit Concatenation and Replication

Arch uses standard SystemVerilog syntax for bit concatenation and replication:

let word: UInt<16> = {high_byte, low_byte}; // concatenation (MSB first)

let sign_ext: UInt<8> = {8{sign_bit}}; // replication

let sext32: UInt<32> = {{24{sign_bit}}, byte_val}; // replication inside concat

{a, b, c} concatenates operands MSB-first, producing a UInt whose width is the sum of all operand widths. {N{expr}} replicates expr N times. Replication may be nested inside concatenation.

4.2.3 Signal Declarations: let, wire, and reg

Arch has three kinds of module-scope signal declarations. Each has a distinct syntax, assignment location, and SV equivalent.

| Construct | Syntax | Assigned in | SV equivalent |
| --- | --- | --- | --- |
| let` (declare) | let x: T = expr; | declaration — type required | logic [W-1:0] x; assign x = expr; |
| let` (assign) | let x = expr; | declaration — x must already exist as output port or wire | assign x = expr;` (no new declaration) |
| wire | wire x: T; | comb` block (`=`) | logic [W-1:0] x;` (driven in `assign`/`always_comb`) |
| reg | reg x: T [init V] [reset R=>V]; | seq` block (`<=`) | logic [W-1:0] x = V;` (driven in `always_ff`) |

`let` has two forms:

- let x: T = expr; — declares a new combinational wire x of type T, fixed to expr. Emits logic [W-1:0] x; assign x = expr; in SV. Type annotation is required when declaring.

- let x = expr; — assigns to an already-declared output port or wire named x. No new signal is created; the type is taken from the existing declaration. Emits assign x = expr;. This replaces the old one-liner comb x = expr; form. Errors if x is not in scope, is an input port, or is a reg (use seq for those).

`wire` declares an explicitly-typed combinational net with no initializer. It must be driven by a let x = expr; assignment or inside a comb ... end comb block. Use wire when the value is conditionally assigned (if/elsif/else) — let x: T = expr; only supports a single fixed expression. The type checker enforces that only wire declarations and output ports are valid comb targets — assigning to a reg in comb is a compile error.

`reg` declares a flip-flop. It must be assigned inside a seq block using <=. Reset polarity and mode are declared per register. The syntax is reg x: T [init VALUE] [reset SIGNAL=>VALUE]; where init is optional (sets only the SV declaration initializer logic x = VALUE;) and reset SIGNAL=>VALUE specifies both the reset signal and the value to load on reset. Use reset none for registers that should not be reset. A reg default: declaration sets the default init and reset for all subsequent registers in scope: reg default: [init VALUE] reset SIGNAL=>VALUE;.

`port reg` declares an output port that is also a register, eliminating the common reg r + comb out = r; end comb boilerplate. The syntax is port reg name: out T [init V] [reset R=>V];. It can only be used on output ports (in direction is a compile error). The port is assigned with <= inside a seq block, just like a regular reg. If reg default: is in scope, it inherits the default init and reset. In generated SV, the port is declared as output logic [W-1:0] name and driven directly in the always_ff block.

module Mux2

port sel: in Bool;

port a: in UInt<8>;

port b: in UInt<8>;

port y: out UInt<8>;

wire result: UInt<8>; // declared here, driven in comb below

comb

if sel

result = a;

else

result = b;

end if

y = result;

end comb

end module Mux2

⚑ The type checker enforces: `reg` cannot be a `comb` target (error: "cannot assign to register `x` in a comb block; use `<=` inside a seq block"). Only `wire` declarations and output ports are valid comb targets.

`multicycle` reg annotation (planned) — reg result: UInt<32> multicycle 3 reset rst => 0; declares that the combinational path feeding this register has a multi-cycle timing budget. Unlike pipe_reg (which inserts N physical flip-flop stages), a multicycle register remains a single flop — no extra area or power. The compiler emits an SDC constraint (set_multicycle_path N -to result) and can statically verify that consumers only sample the value at the correct rate. This is useful for slow-settling operations (multipliers, dividers, complex ALU) where the path does not affect end-to-end throughput.

The Counter example from §4.2 can be simplified using port reg:

module Counter

param WIDTH: const = 8;

port clk: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port en: in Bool;

port reg count: out UInt<WIDTH> reset rst => 0;

seq on clk rising

if en

count <= (count + 1).trunc<WIDTH>();

end if

end seq

end module Counter

⚑ `port reg` eliminates the intermediate `reg count_r` and the `comb count = count_r; end comb` wiring block. The port is directly assigned in the `seq` block. This is the preferred style when the output port is a straightforward registered value.

`multicycle` reg annotation (planned) — reg result: UInt<32> multicycle 3 reset rst => 0; declares that the combinational path feeding this register has a multi-cycle timing budget. Unlike pipe_reg (which inserts N physical flip-flop stages), a multicycle register remains a single flop — no extra area or power. The compiler auto-detects all input signals feeding the register by walking the assignment expression tree. Three modes of enforcement: (1) Simulation (–check-uninit): hidden valid tracking with input change detection and latency counter; reads before the counter expires return poison/X. (2) Synthesis: SDC constraint generation (set_multicycle_path N -to result). (3) Formal: optional assert property to verify the multicycle timing assumption holds.

4.2.4 Width-Qualified Parameters

An untyped param NAME: const = value emits parameter int NAME = value in SystemVerilog (32-bit signed). When a parameter is compared or assigned against a specific-width signal, this width mismatch produces Verilator WIDTHEXPAND warnings. Use the bracket form to declare the parameter's bit width explicitly:

param H_ACTIVE[9:0]: const = 640; // emits: parameter [9:0] H_ACTIVE = 640

param H_FRONT[9:0]: const = 16; // emits: parameter [9:0] H_FRONT = 16

The compiler validates that the default value fits within the declared range. Comparisons with UInt<10> signals are now width-matched and Verilator-clean.

4.3  Module Instantiation

instantiation.arch module Top port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port a: in UInt<8>; port b: in UInt<8>; port out: out UInt<9>; inst add: Adder param WIDTH = 8; connect clk <- clk; connect rst <- rst; connect a <- a; connect b <- b; connect sum -> out; end inst add end module Top

◈  connect port <- signal drives an input.  connect port -> signal reads an output.  The arrow direction always shows which way data flows — a property that makes the design readable to both humans and AI without any context lookup.

5.  Clock Domains and CDC Safety

Every Clock signal in Arch carries a domain tag as part of its type.  The compiler tracks which domain every signal belongs to.  Crossing domain boundaries without an explicit crossing block is a compile-time error — never a simulation-time surprise.

5.1  Declaring Domains

domains.arch // Domains are declared at the top level — before any module that uses them domain SysDomain freq_mhz: 200, end domain SysDomain domain UsbDomain freq_mhz: 48, end domain UsbDomain domain PcieDomain freq_mhz: 250, end domain PcieDomain

5.2  Clock Domain Crossing

cdc.arch module UsbBridge port sys_clk: in Clock<SysDomain>; port usb_clk: in Clock<UsbDomain>; port rst: in Reset<Async>; port sys_data: in UInt<32>; port usb_data: out UInt<32>; // ✗ COMPILE ERROR — signal crosses domains without a declared crossing // comb // usb_data = sys_data; // end comb // ✓ Explicit CDC — compiler inserts a verified synchroniser crossing sys_to_usb from: SysDomain, to: UsbDomain, sync: two_flop, // policy: two_flop ¦ gray_code ¦ handshake data: sys_data -> usb_data, end crossing sys_to_usb end module UsbBridge

⚑  The compiler generates a verified synchroniser for each crossing declaration.  Engineers choose the policy; correctness of the CDC structure is guaranteed by the language, not by convention or code review.

5.3 Clock Output Ports

Clock<Domain> may appear as an output port direction in any module, enabling clock passthrough, gating, and division:

// Passthrough

module ClkPassthrough

port clk_in: in Clock<SysDomain>;

port clk_out: out Clock<SysDomain>;

comb clk_out = clk_in;

end module ClkPassthrough

// Inline gate (AND with enable)

module ClkGate

port clk_in: in Clock<SysDomain>;

port enable: in Bool;

port clk_out: out Clock<SysDomain>;

comb clk_out = clk_in & enable;

end module ClkGate

// Divide-by-2

module ClkDiv2

port clk_in: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port clk_out: out Clock<SysDomain>;

reg toggle: Bool reset rst=>false;

default seq on clk_in rising;

seq toggle <= ~toggle; end seq

comb clk_out = toggle;

end module ClkDiv2

A Clock<> output port emits output logic in SV and may be driven by a comb assignment. For dedicated clock gating with integrated latch, use the first-class clkgate construct instead.

5.3 Clock Output Ports

Clock<Domain> may appear as an output port direction in any module, enabling clock passthrough, gating, and division:

// Passthrough

module ClkPassthrough

port clk_in: in Clock<SysDomain>;

port clk_out: out Clock<SysDomain>;

comb clk_out = clk_in;

end module ClkPassthrough

// Inline gate (AND with enable)

module ClkGate

port clk_in: in Clock<SysDomain>;

port enable: in Bool;

port clk_out: out Clock<SysDomain>;

comb clk_out = clk_in & enable;

end module ClkGate

// Divide-by-2

module ClkDiv2

port clk_in: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port clk_out: out Clock<SysDomain>;

reg toggle: Bool reset rst=>false;

default seq on clk_in rising;

seq toggle <= ~toggle; end seq

comb clk_out = toggle;

end module ClkDiv2

A Clock<> output port emits output logic in SV and may be driven by a comb assignment. For dedicated clock gating with integrated latch, use the first-class clkgate construct instead.

6.  First-Class Construct: pipeline

A pipeline is a first-class Arch construct — not a pattern you build from registers and muxes.  The compiler understands stall, flush, and forward semantics natively and generates all hazard control logic automatically from declarative annotations.

6.1  Declaration

pipeline.arch pipeline Decode param XLEN: const = 32; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port in_pc: in UInt<XLEN>; port out_pc: out UInt<XLEN>; port out_instr: out DecodedInstr; stage Fetch reg pc: UInt<XLEN> init 0; reg on clk rising, rst high if rst pc <= 0; end if else pc <= in_pc; end else end reg end stage Fetch stage Decode let raw: UInt<32> = Fetch.pc; comb out_pc = Fetch.pc; out_instr = decode(raw); end comb end stage Decode stall when Decode.out_instr.rd == 0; flush Fetch when branch_mispred; forward Decode.rs1_val from Execute.alu_result when Execute.rd == Decode.rs1 and Execute.rd != 0; end pipeline Decode

◈  stall, flush, and forward are declarative.  The compiler generates all enable signals, bubble insertion logic, and bypass muxes.  The designer describes intent; the compiler generates mechanism.

7.  First-Class Construct: fsm

An fsm block declares a finite state machine with named states and exhaustive coverage enforced by the compiler.  Missing transitions, undriven outputs in any state, and unreachable states are all compile-time errors.

7.1  Declaration

fsm.arch fsm TrafficLight param TIMER_W: const = 8; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port timer: in UInt<TIMER_W>; port red: out Bool; port yellow: out Bool; port green: out Bool; state Red, Yellow, Green; default state Red; state Red comb red = true; yellow = false; green = false; end comb transition to Green when timer == 0; end state Red state Green comb red = false; yellow = false; green = true; end comb transition to Yellow when timer == 0; end state Green state Yellow comb red = false; yellow = true; green = false; end comb transition to Red when timer == 0; end state Yellow end fsm TrafficLight

⚑  The compiler verifies: every state has at least one outgoing transition; all output ports are driven in every state; no two transitions from the same state can be simultaneously enabled.

7.2 FSM Default Block

The default ... end default block provides default assignments that are emitted before the state case statement. It may contain comb ... end comb and/or seq on clk rising ... end seq sub-blocks:

default

comb

out_a = false;

out_b = 0;

end comb

seq on clk rising

data_reg <= 0;

end seq

end default

Semantics:

- Default comb assignments are emitted at the top of the output always_comb block, before case (state_r). States that override a signal simply reassign it inside their case branch.

- Default seq assignments are emitted at the top of the always_ff else branch, before the per-state case. States with seq blocks override as needed.

- Output ports not covered by either a default block or per-state comb assignments will be X (undriven) in the generated Verilog — this matches real hardware behavior.

Generated SystemVerilog:

SystemVerilog

always_comb begin

out_a = 1'b0; // ← from default block

out_b = 0; // ← from default block

case (state_r)

FOO: begin

out_a = 1'b1; // state-level override

end

default: ; // other states inherit the defaults

endcase

end

7.2 FSM Default Block

The default ... end default block provides default assignments that are emitted before the state case statement. It may contain comb ... end comb and/or seq on clk rising ... end seq sub-blocks:

default

comb

out_a = false;

out_b = 0;

end comb

seq on clk rising

data_reg <= 0;

end seq

end default

Semantics:

- Default comb assignments are emitted at the top of the output always_comb block, before case (state_r). States that override a signal simply reassign it inside their case branch.

- Default seq assignments are emitted at the top of the always_ff else branch, before the per-state case. States with seq blocks override as needed.

- Output ports not covered by either a default block or per-state comb assignments will be X (undriven) in the generated Verilog — this matches real hardware behavior.

Generated SystemVerilog:

SystemVerilog

always_comb begin

out_a = 1'b0; // ← from default block

out_b = 0; // ← from default block

case (state_r)

FOO: begin

out_a = 1'b1; // state-level override

end

default: ; // other states inherit the defaults

endcase

end

8.  First-Class Construct: fifo

A fifo is a first-class construct with compile-time-verified flow control.  The designer specifies depth, width, and domain.  The compiler generates the full implementation — counters, full/empty flags, and gray-code pointer CDC for dual-clock FIFOs.

8.1  Single-Clock FIFO

fifo_single.arch fifo TxQueue param DEPTH: const = 16; param WIDTH: type = UInt<32>; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port push_valid: in Bool; port push_ready: out Bool; port push_data: in WIDTH; port pop_valid: out Bool; port pop_ready: in Bool; port pop_data: out WIDTH; port full: out Bool; port empty: out Bool; end fifo TxQueue

8.2  Dual-Clock (Async) FIFO

fifo_async.arch fifo AsyncBridge param DEPTH: const = 32; param WIDTH: type = UInt<8>; port wr_clk: in Clock<FastDomain>; port rd_clk: in Clock<SlowDomain>; port rst: in Reset<Async>; port push_valid: in Bool; port push_ready: out Bool; port push_data: in WIDTH; port pop_valid: out Bool; port pop_ready: in Bool; port pop_data: out WIDTH; end fifo AsyncBridge

◈  When two different Clock domains are detected on wr_clk and rd_clk, the compiler automatically selects gray-code pointer synchronisation.  This can be overridden with an explicit sync: policy annotation inside the fifo body.

8.2a LIFO (Stack)

lifo_stack.arch

fifo LifoStack

kind lifo;

param DEPTH: const = 16;

param TYPE: type = UInt<8>;

port clk: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port push_valid: in Bool;

port push_ready: out Bool;

port push_data: in TYPE;

port pop_valid: out Bool;

port pop_ready: in Bool;

port pop_data: out TYPE;

port full: out Bool;

port empty: out Bool;

end fifo LifoStack

◈ The LIFO uses a single stack pointer sp. Push writes at mem[sp] and increments; pop decrements and reads mem[sp-1]. Simultaneous push+pop replaces the top of stack without changing the pointer. The same push/pop handshake protocol (valid/ready) is used for both FIFO and LIFO.

8.3 First-Class Construct: synchronizer

A synchronizer is a first-class construct for clock domain crossing (CDC) of individual signals. While a dual-clock fifo handles bulk data transfer between domains, a synchronizer handles single signals — control bits, status flags, counters, and event pulses. The designer declares the synchronization strategy via kind and the compiler generates the correct CDC logic, including all intermediate flip-flop stages, encoding/decoding, and protocol handshaking.

The kind keyword selects the synchronization strategy (same syntax as ram). If omitted, the default is ff. The param STAGES controls the flip-flop chain depth (default 2).

8.3.1 Synchronizer Kinds

| Kind | Signal Type | Description |
| --- | --- | --- |
| ff` (default) | Bool` or 1-bit | N-stage flip-flop shift chain clocked on `dst_clk`. Best for single-bit level signals (e.g. status flags, enables). |
| gray | UInt<N>` (multi-bit) | Binary-to-gray encode in source domain, N-stage FF chain, gray-to-binary decode in destination domain. Safe for multi-bit counters and pointers where only one bit changes per cycle. |
| handshake | Any type | Req/ack toggle protocol with synchronized control signals. Safe for arbitrary multi-bit data that changes infrequently. Higher latency than `gray` but works for any data pattern. |
| reset | Bool` only | Reset synchronizer: asynchronous assert (immediate propagation), synchronous deassert through N-stage FF chain. Used for synchronizing reset deassertion to a clock domain. Compile error if data type is not `Bool`. |
| pulse | Bool` only | Pulse synchronizer: converts a single-cycle pulse in the source domain into a level toggle, syncs the toggle through the N-stage FF chain, then edge-detects in the destination domain to regenerate a single-cycle pulse. Used for events, interrupts, and triggers across clock domains. Compile error if data type is not `Bool`. |

8.3.2 Declaration

sync_ff.arch — 1-bit FF synchronizer (default kind)

synchronizer StatusSync

param STAGES: const = 2;

port src_clk: in Clock<SrcDomain>;

port dst_clk: in Clock<DstDomain>;

port rst: in Reset<Async>;

port data_in: in Bool;

port data_out: out Bool;

end synchronizer StatusSync

sync_gray.arch — multi-bit gray-code synchronizer

synchronizer PtrSync

kind gray;

param STAGES: const = 2;

port src_clk: in Clock<WrDomain>;

port dst_clk: in Clock<RdDomain>;

port rst: in Reset<Async>;

port data_in: in UInt<5>;

port data_out: out UInt<5>;

end synchronizer PtrSync

sync_handshake.arch — multi-bit handshake synchronizer

synchronizer ConfigSync

kind handshake;

param STAGES: const = 2;

port src_clk: in Clock<CpuDomain>;

port dst_clk: in Clock<AccelDomain>;

port rst: in Reset<Sync>;

port data_in: in UInt<32>;

port data_out: out UInt<32>;

end synchronizer ConfigSync

sync_reset.arch — reset synchronizer

synchronizer RstSync

kind reset;

param STAGES: const = 3;

port src_clk: in Clock<PllDomain>;

port dst_clk: in Clock<CoreDomain>;

port rst: in Reset<Async>;

port data_in: in Bool;

port data_out: out Bool;

end synchronizer RstSync

sync_pulse.arch — pulse synchronizer for events

synchronizer EventSync

kind pulse;

param STAGES: const = 2;

port src_clk: in Clock<SrcDomain>;

port dst_clk: in Clock<DstDomain>;

port rst: in Reset<Sync>;

port data_in: in Bool;

port data_out: out Bool;

end synchronizer EventSync

8.3.3 Compile-Time Checks

The compiler enforces the following rules for synchronizer constructs:

- The two Clock<Domain> ports must reference different domains. Same-domain clocks are a compile error — use a direct assignment instead.

- kind reset and kind pulse require the data port type to be Bool. Multi-bit data is a compile error.

- kind gray requires UInt<N> data. The compiler warns if the source signal can change by more than one LSB per cycle (e.g. arbitrary data rather than a counter).

- STAGES must be at least 2. Values of 2 or 3 are typical; the compiler warns on values above 4 (diminishing returns, increased latency).

8.3.4 Generated Hardware

| Kind | SystemVerilog Output |
| --- | --- |
| ff | Chain of `STAGES` flip-flops clocked on `dst_clk`: `always_ff @(posedge dst_clk)` with shift register `sync[0] <= data_in; sync[1] <= sync[0]; ... |
| gray | Source domain: binary-to-gray encoder. Destination domain: `STAGES`-FF chain + gray-to-binary decoder. |
| handshake | Req toggle in source domain, req synchronized to destination domain via FF chain, ack toggle back to source domain via FF chain, data latched on ack. |
| reset | always_ff @(posedge dst_clk or posedge data_in)`: async set on assertion, synchronous shift chain for deassertion. |
| pulse | Source domain: toggle register flipped on input pulse. Destination domain: `STAGES`-FF chain on toggle + XOR edge detector to regenerate single-cycle pulse. |

◈ The synchronizer construct and the dual-clock fifo are the two legal CDC crossing mechanisms in Arch. Any other cross-domain signal access is a compile error, with the error message directing the user to use either a synchronizer or an async fifo.

◈ `–cdc-random` simulation flag. When arch sim is invoked with –cdc-random, each synchronizer's FF-chain shift is probabilistically skipped on any given clock edge, adding +1 cycle of latency. This verifies that designs do not depend on exact synchronizer propagation delay. The probability is controlled by the cdc_skip_pct public member (0–100, default 25) on each generated C++ model, allowing testbenches to tune randomization intensity at runtime (e.g. dut.cdc_skip_pct = 50; for aggressive stress testing). Internally uses a 32-bit LFSR for deterministic pseudo-random sequencing.

◈ CDC checking extends across inst boundaries. When a parent module instantiates a child, the compiler traces clock port connections to map child domains to parent domains, then verifies that all data connections respect clock domain boundaries. If a signal from DomainA is connected to a port that operates in DomainB inside the child, the compiler reports a CDC violation — the same error and guidance as for intra-module crossings.

8.2a LIFO (Stack)

lifo_stack.arch

fifo LifoStack

kind lifo;

param DEPTH: const = 16;

param TYPE: type = UInt<8>;

port clk: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port push_valid: in Bool;

port push_ready: out Bool;

port push_data: in TYPE;

port pop_valid: out Bool;

port pop_ready: in Bool;

port pop_data: out TYPE;

port full: out Bool;

port empty: out Bool;

end fifo LifoStack

◈ The LIFO uses a single stack pointer sp. Push writes at mem[sp] and increments; pop decrements and reads mem[sp-1]. Simultaneous push+pop replaces the top of stack without changing the pointer. The same push/pop handshake protocol (valid/ready) is used for both FIFO and LIFO.

8.3 First-Class Construct: synchronizer

A synchronizer is a first-class construct for clock domain crossing (CDC) of individual signals. While a dual-clock fifo handles bulk data transfer between domains, a synchronizer handles single signals — control bits, status flags, counters, and event pulses. The designer declares the synchronization strategy via kind and the compiler generates the correct CDC logic, including all intermediate flip-flop stages, encoding/decoding, and protocol handshaking.

The kind keyword selects the synchronization strategy (same syntax as ram). If omitted, the default is ff. The param STAGES controls the flip-flop chain depth (default 2).

8.3.1 Synchronizer Kinds

| Kind | Signal Type | Description |
| --- | --- | --- |
| ff` (default) | Bool` or 1-bit | N-stage flip-flop shift chain clocked on `dst_clk`. Best for single-bit level signals (e.g. status flags, enables). |
| gray | UInt<N>` (multi-bit) | Binary-to-gray encode in source domain, N-stage FF chain, gray-to-binary decode in destination domain. Safe for multi-bit counters and pointers where only one bit changes per cycle. |
| handshake | Any type | Req/ack toggle protocol with synchronized control signals. Safe for arbitrary multi-bit data that changes infrequently. Higher latency than `gray` but works for any data pattern. |
| reset | Bool` only | Reset synchronizer: asynchronous assert (immediate propagation), synchronous deassert through N-stage FF chain. Used for synchronizing reset deassertion to a clock domain. Compile error if data type is not `Bool`. |
| pulse | Bool` only | Pulse synchronizer: converts a single-cycle pulse in the source domain into a level toggle, syncs the toggle through the N-stage FF chain, then edge-detects in the destination domain to regenerate a single-cycle pulse. Used for events, interrupts, and triggers across clock domains. Compile error if data type is not `Bool`. |

8.3.2 Declaration

sync_ff.arch — 1-bit FF synchronizer (default kind)

synchronizer StatusSync

param STAGES: const = 2;

port src_clk: in Clock<SrcDomain>;

port dst_clk: in Clock<DstDomain>;

port rst: in Reset<Async>;

port data_in: in Bool;

port data_out: out Bool;

end synchronizer StatusSync

sync_gray.arch — multi-bit gray-code synchronizer

synchronizer PtrSync

kind gray;

param STAGES: const = 2;

port src_clk: in Clock<WrDomain>;

port dst_clk: in Clock<RdDomain>;

port rst: in Reset<Async>;

port data_in: in UInt<5>;

port data_out: out UInt<5>;

end synchronizer PtrSync

sync_handshake.arch — multi-bit handshake synchronizer

synchronizer ConfigSync

kind handshake;

param STAGES: const = 2;

port src_clk: in Clock<CpuDomain>;

port dst_clk: in Clock<AccelDomain>;

port rst: in Reset<Sync>;

port data_in: in UInt<32>;

port data_out: out UInt<32>;

end synchronizer ConfigSync

sync_reset.arch — reset synchronizer

synchronizer RstSync

kind reset;

param STAGES: const = 3;

port src_clk: in Clock<PllDomain>;

port dst_clk: in Clock<CoreDomain>;

port rst: in Reset<Async>;

port data_in: in Bool;

port data_out: out Bool;

end synchronizer RstSync

sync_pulse.arch — pulse synchronizer for events

synchronizer EventSync

kind pulse;

param STAGES: const = 2;

port src_clk: in Clock<SrcDomain>;

port dst_clk: in Clock<DstDomain>;

port rst: in Reset<Sync>;

port data_in: in Bool;

port data_out: out Bool;

end synchronizer EventSync

8.3.3 Compile-Time Checks

The compiler enforces the following rules for synchronizer constructs:

- The two Clock<Domain> ports must reference different domains. Same-domain clocks are a compile error — use a direct assignment instead.

- kind reset and kind pulse require the data port type to be Bool. Multi-bit data is a compile error.

- kind gray requires UInt<N> data. The compiler warns if the source signal can change by more than one LSB per cycle (e.g. arbitrary data rather than a counter).

- STAGES must be at least 2. Values of 2 or 3 are typical; the compiler warns on values above 4 (diminishing returns, increased latency).

8.3.4 Generated Hardware

| Kind | SystemVerilog Output |
| --- | --- |
| ff | Chain of `STAGES` flip-flops clocked on `dst_clk`: `always_ff @(posedge dst_clk)` with shift register `sync[0] <= data_in; sync[1] <= sync[0]; ... |
| gray | Source domain: binary-to-gray encoder. Destination domain: `STAGES`-FF chain + gray-to-binary decoder. |
| handshake | Req toggle in source domain, req synchronized to destination domain via FF chain, ack toggle back to source domain via FF chain, data latched on ack. |
| reset | always_ff @(posedge dst_clk or posedge data_in)`: async set on assertion, synchronous shift chain for deassertion. |
| pulse | Source domain: toggle register flipped on input pulse. Destination domain: `STAGES`-FF chain on toggle + XOR edge detector to regenerate single-cycle pulse. |

◈ The synchronizer construct and the dual-clock fifo are the two legal CDC crossing mechanisms in Arch. Any other cross-domain signal access is a compile error, with the error message directing the user to use either a synchronizer or an async fifo.

◈ `–cdc-random` simulation flag. When arch sim is invoked with –cdc-random, each synchronizer's FF-chain shift is probabilistically skipped on any given clock edge, adding +1 cycle of latency. This verifies that designs do not depend on exact synchronizer propagation delay. The probability is controlled by the cdc_skip_pct public member (0–100, default 25) on each generated C++ model, allowing testbenches to tune randomization intensity at runtime (e.g. dut.cdc_skip_pct = 50; for aggressive stress testing). Internally uses a 32-bit LFSR for deterministic pseudo-random sequencing.

◈ CDC checking extends across inst boundaries. When a parent module instantiates a child, the compiler traces clock port connections to map child domains to parent domains, then verifies that all data connections respect clock domain boundaries. If a signal from DomainA is connected to a port that operates in DomainB inside the child, the compiler reports a CDC violation — the same error and guidance as for intra-module crossings.

9.  First-Class Construct: arbiter

An arbiter manages N requesters competing for M shared resources.  The designer declares port counts and an arbitration policy.  The compiler generates all grant logic, stall propagation, and fairness guarantees.

9.1  Declaration

arbiter_decl.arch arbiter BusArbiter param NUM_REQ: const = 4; param NUM_RSRC: const = 1; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; ports[NUM_REQ] request valid: in Bool; ready: out Bool; end ports ports[NUM_RSRC] grant valid: out Bool; requester: out UInt<$clog2(NUM_REQ)>; end ports policy round_robin; end arbiter BusArbiter

9.2  Arbitration Policies

| Policy | Fairness | Latency | Best For |
| --- | --- | --- | --- |
| round_robin | Strong — every requester served in ≤ N cycles | O(N) | General-purpose bus arbitration |
| priority | None — highest-index wins unconditionally | O(1) | Interrupt controllers, emergency paths |
| weighted<W> | Proportional to weight vector W | O(N) | QoS-aware arbitration, bandwidth shaping |
| lru | Approximate — least-recently-used first | O(N) | Cache miss handling, memory controllers |
| custom fn | User-defined — compiler checks liveness | O(?) | Specialised protocols |

10.  First-Class Construct: regfile

A regfile declares a structured register file with typed entries, multiple read ports, and multiple write ports.  The compiler generates the read mux, write enable logic, and optionally write-before-read forwarding.

10.1  Declaration

regfile.arch regfile IntRegs param XLEN: const = 32; param NREGS: const = 32; param NREAD: const = 2; param NWRITE: const = 1; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; ports[NREAD] read addr: in UInt<$clog2(NREGS)>; data: out UInt<XLEN>; end ports ports[NWRITE] write en: in Bool; addr: in UInt<$clog2(NREGS)>; data: in UInt<XLEN>; end ports init [0] = 0; forward write_before_read: true; end regfile IntRegs

◈  forward write_before_read: true generates bypass muxes so a write and read to the same address in the same cycle returns the new value.  Setting it to false generates simpler hardware and surfaces the hazard to the pipeline.

11.  First-Class Construct: ram

A ram is a first-class construct that maps to a physical memory — FPGA BRAM, distributed RAM, or ASIC SRAM macro.  Unlike regfile (which always uses flip-flops), ram targets technology memory primitives.  The designer declares the port topology, read timing, and a set of logical variables that live inside the RAM's address space.  The compiler assigns address ranges, computes total depth and word width, and translates all logical accesses to physical addresses — no manual address arithmetic required.

11.1  Port Topologies

| Topology | Keyword | Simultaneous Access | Typical Mapping |
| --- | --- | --- | --- |
| Single port | kind single | Read or write, never both | Small LUT RAM, single-port SRAM macro |
| Simple dual port | kind simple_dual | Read on one port, write on the other | Standard FPGA BRAM (most common mode) |
| True dual port | kind true_dual | Any combination on both ports simultaneously | True dual-port BRAM, high-bandwidth caches |

11.2  Read Timing Modes

| Mode | Latency | Behaviour | Use When |
| --- | --- | --- | --- |
| read: async | 0 cycles | Output is combinationally derived from address | Register files, small look-up tables |
| read: sync | 1 cycle | Address and enable registered; data available next cycle | Standard BRAM, most cache SRAMs |
| read: sync_out | 2 cycles | Both input and output registered for max frequency | Timing-critical paths, deep pipelines |

11.3  Single-Port RAM

ram_single.arch // Single-port ROM/RAM — one port shared for reads and writes ram InstrRom param DEPTH: const = 4096; port clk: in Clock<SysDomain>; kind single; read: sync; write: first; // write_first ¦ read_first ¦ no_change // One logical variable filling the whole address space store instructions: Vec<UInt<32>, DEPTH>; end store port access en: in Bool; wen: in Bool; addr: in UInt<$clog2(DEPTH)>; wdata: in UInt<32>; rdata: out UInt<32>; end port access init: file "instr_rom.hex"; // load from hex file at simulation/power-on end ram InstrRom

11.4  Simple Dual-Port RAM

ram_simple_dual.arch // Simple dual-port — dedicated read port and write port ram DataCache param DEPTH: const = 1024; param LINE: type = CacheLine; // compiler computes word width from struct port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind simple_dual; read: sync; store lines: Vec<LINE, DEPTH>; end store port read_port en: in Bool; addr: in UInt<$clog2(DEPTH)>; data: out LINE; end port read_port port write_port en: in Bool; addr: in UInt<$clog2(DEPTH)>; data: in LINE; mask: in Vec<Bool, $bytes(LINE)>; // optional byte-enable mask end port write_port end ram DataCache

11.5  True Dual-Port RAM

ram_true_dual.arch // True dual-port — both ports can read or write independently ram SharedScratchpad param DEPTH: const = 512; param WIDTH: type = UInt<64>; port clk_a: in Clock<CoreADomain>; port clk_b: in Clock<CoreBDomain>; // may be same or different domain kind true_dual; read: sync; store data: Vec<WIDTH, DEPTH>; end store // Port A — full read/write port a en: in Bool; wen: in Bool; addr: in UInt<$clog2(DEPTH)>; wdata: in WIDTH; rdata: out WIDTH; end port a // Port B — full read/write port b en: in Bool; wen: in Bool; addr: in UInt<$clog2(DEPTH)>; wdata: in WIDTH; rdata: out WIDTH; end port b // Collision policy when both ports access the same address simultaneously collision: port_a_wins; // port_a_wins ¦ port_b_wins ¦ undefined end ram SharedScratchpad

11.6  Multi-Variable Mapping  (compiler-managed address layout)

A ram's store block can declare multiple logical variables of different types and depths.  The compiler assigns non-overlapping address ranges automatically and translates all logical accesses to physical addresses.  The designer never computes base offsets manually.

ram_multi_var.arch // One physical RAM holding integer registers, FP registers, and CSRs ram UnifiedRegRam port clk: in Clock<SysDomain>; kind simple_dual; read: async; // register-file mode — combinational read // Compiler lays these out contiguously and picks the widest type as word width. // Address map (auto-assigned): // int_regs → 0x000 .. 0x01F (32 entries) // fp_regs → 0x020 .. 0x03F (32 entries) // csr_regs → 0x040 .. 0x43F (1024 entries) // Total depth: 1088, word width: 64 bits store int_regs: Vec<UInt<64>, 32>; fp_regs: Vec<UInt<64>, 32>; csr_regs: Vec<UInt<64>, 1024>; end store port read_port en: in Bool; addr: in UInt<$clog2(1088)>; data: out UInt<64>; end port read_port port write_port en: in Bool; addr: in UInt<$clog2(1088)>; data: in UInt<64>; end port write_port end ram UnifiedRegRam // ── Accessing logical variables by name ───────────────────────────── // The compiler translates logical names to physical addresses automatically. // No manual offset arithmetic ever appears in Arch source. inst regram: UnifiedRegRam connect clk <- clk; // Access int_regs[rs1] — compiler emits physical address rs1 + base(int_regs) connect read_port.en <- true; connect read_port.addr <- int_regs[rs1_addr]; connect read_port.data -> rs1_val; // Access csr_regs[csr_idx] — compiler emits physical address csr_idx + base(csr_regs) connect write_port.en <- csr_wen; connect write_port.addr <- csr_regs[csr_idx]; connect write_port.data <- csr_wdata; end inst regram

⚑  The logical variable syntax  int_regs[index]  in a connect address expression is resolved entirely at compile time to physical_base + index.  The generated SystemVerilog contains only numeric address expressions — no overhead, no indirection.

11.7  Struct Types as RAM Words

Any Arch struct can be used as the word type of a ram.  The compiler packs the struct fields into the physical word width and generates the appropriate bit-slice assignments in the output SystemVerilog.

ram_struct.arch struct CacheLine valid: Bool, dirty: Bool, tag: UInt<20>, data: Vec<UInt<32>, 16>, // 16-word cache line end struct CacheLine // Compiler computes: word width = 1+1+20+(32×16) = 534 bits ram L1Cache param SETS: const = 256; port clk: in Clock<SysDomain>; kind simple_dual; read: sync; store lines: Vec<CacheLine, SETS>; end store port read_port en: in Bool; addr: in UInt<8>; data: out CacheLine; // returned as a fully decoded struct end port read_port port write_port en: in Bool; addr: in UInt<8>; data: in CacheLine; end port write_port end ram L1Cache // Field access on the read result — compiler emits the correct bit slice let line: CacheLine = l1_rdata; let hit: Bool = line.valid and (line.tag == req_tag);

◈  When a struct is the word type, the compiler automatically generates the packing/unpacking bit slices in the emitted SystemVerilog.  Designers work entirely with named fields — no manual [hi:lo] indexing into packed words.

11.8  RAM Initialization

| Annotation | Behaviour | Supported In |
| --- | --- | --- |
| init: zero | All words set to zero at reset | Simulation and FPGA (BRAM init) |
| init: none | Words undefined until written | ASIC default; simulation X-propagation enabled |
| init: file "path.hex" | Load from Intel HEX file at elaboration | Simulation and FPGA bitstream |
| init: file "path.bin" | Load from raw binary file | Simulation only |
| init: value expr | All words set to the given const expression | Simulation and FPGA |

11.9  RAM vs regfile — When to Use Which

|  | regfile | ram |
| --- | --- | --- |
| Implementation | Flip-flops (always) | Technology memory primitive (BRAM / SRAM macro) |
| Read latency | Zero (async only) | Zero (async), one cycle (sync), two cycles (sync_out) |
| Depth | Small (≤ 64 typically) | Any — limited by target technology |
| Variable mapping | Single flat array | Multiple typed logical variables, compiler assigns layout |
| Struct word type | No | Yes — compiler packs/unpacks fields automatically |
| Byte enable mask | No | Yes — mask port on write_port |
| FPGA primitive | LUT / distributed RAM | BRAM18 / BRAM36 / URAM |
| ASIC target | Std-cell array | SRAM macro (requires technology library) |

12.  First-Class Construct: linklist

A linklist is a first-class Arch construct that maps a hardware-native linked list to a physical node pool RAM plus a compiler-generated controller IP.  In hardware, pointers do not exist — a linklist uses slot indices into a fixed-depth node pool as its internal references.  The designer declares the pool depth, payload type, list topology, and a latency budget per operation.  The compiler generates the node pool RAM, free-list FIFO, head/tail registers, and a controller FSM that meets the declared latency constraints.

12.1  Physical Implementation Model

Every linklist declaration compiles to the following hardware components, generated and wired automatically:

| Component | Implementation | Purpose |
| --- | --- | --- |
| Node pool RAM | simple_dual RAM, depth × (¦data¦ + ¦ptr¦ × links) | Stores payload and next/prev pointer per slot |
| Free list | fifo, depth entries of UInt<$clog2(depth)> | Tracks available slot indices for alloc/free |
| Head register | reg, UInt<$clog2(depth)> | Points to the first node |
| Tail register | reg, UInt<$clog2(depth)> | Points to the last node (if tail tracking enabled) |
| Length counter | reg, UInt<$clog2(depth+1)> | Optional; tracks current list length |
| Controller FSM | fsm, compiler-generated | Sequences all multi-cycle operations |

⚑  The slot index width is always $clog2(depth) — computed automatically.  Designers never declare or manipulate raw pointer widths.

12.2  List Topology

| kind | Links per node | delete cost | Best For |
| --- | --- | --- | --- |
| singly | next only | O(N) — must traverse to find predecessor | Queues, stacks, simple ordered lists |
| doubly | next + prev | O(1) — predecessor known directly | LRU lists, priority queues, reorderable lists |
| circular_singly | next (tail→head) | O(N) | Token ring, round-robin scheduling |
| circular_doubly | next + prev (circular) | O(1) | High-performance scheduling, deques |

12.3  Operation Latency

Every operation on a linklist has a latency declaration — the number of clock cycles from request assertion to result valid.  The compiler uses this budget to choose an implementation: lower latency requires simpler (potentially slower-clocking) logic; higher latency allows pipelining for higher Fmax.

| Operation | Minimum Latency | Typical Latency | Notes |
| --- | --- | --- | --- |
| alloc | 1 cycle | 1 cycle | Pop from free-list FIFO — always O(1) |
| free | 1 cycle | 1 cycle | Push to free-list FIFO — always O(1) |
| insert_head | 1 cycle | 2 cycles | Write node pool, update head register |
| insert_tail | 1 cycle | 2 cycles | Write node pool, update tail register |
| insert_after(h) | 1 cycle | 2 cycles | Write node, patch next/prev pointers |
| delete_head | 1 cycle | 2 cycles | Read next, update head, free slot |
| delete(h) | 1 cycle (doubly) | 2–3 cycles | singly: O(N) traverse; doubly: O(1) |
| next(h) | 1 cycle | 1 cycle | One RAM read for next pointer |
| prev(h) | 1 cycle | 1 cycle | doubly only; one RAM read for prev pointer |
| read_data(h) | 1 cycle | 1–2 cycles | Read payload at slot h |
| write_data(h) | 1 cycle | 1–2 cycles | Write payload at slot h |
| search(pred) | N cycles | N cycles | Worst case: full traversal of N nodes |
| length | 0 cycles | 0 cycles | Combinational from length counter register |

12.4  Declaration — Singly-Linked Queue

linklist_singly.arch // A hardware task queue — singly linked, 64 nodes, 3-cycle insert/delete linklist TaskQueue param DEPTH: const = 64; param DATA: type = TaskDescriptor; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind singly; track tail: true; // maintain a tail pointer for O(1) insert_tail track length: true; // maintain a length counter // ── Operation port bundles ──────────────────────────────────── // Each op has a valid/ready handshake and a declared latency. // The compiler generates the controller FSM to meet each budget. op alloc latency: 1; port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; // the new slot index end op alloc op free latency: 1; port req_valid: in Bool; port req_ready: out Bool; port req_handle: in UInt<$clog2(DEPTH)>; end op free op insert_tail latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_data: in DATA; port resp_valid: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; end op insert_tail op delete_head latency: 2; port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_data: out DATA; end op delete_head op read_data latency: 1; port req_valid: in Bool; port req_handle: in UInt<$clog2(DEPTH)>; port resp_valid: out Bool; port resp_data: out DATA; end op read_data // Combinational status — no latency port empty: out Bool; port full: out Bool; port length: out UInt<$clog2(DEPTH+1)>; end linklist TaskQueue

12.5  Declaration — Doubly-Linked LRU List

linklist_doubly.arch // Doubly-linked list for an LRU cache eviction policy. // O(1) delete anywhere — no traversal needed because prev pointer is available. linklist LruList param DEPTH: const = 256; param DATA: type = UInt<$clog2(DEPTH)>; // payload = cache set index port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind doubly; track tail: true; track length: false; // not needed for LRU op insert_head latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_data: in DATA; port resp_valid: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; end op insert_head // delete(h): O(1) because prev pointer known — doubly-linked op delete latency: 3; // read prev+next, write two pointer patches port req_valid: in Bool; port req_ready: out Bool; port req_handle: in UInt<$clog2(DEPTH)>; port resp_valid: out Bool; end op delete // Evict: read and remove the tail node (LRU victim) op evict_tail latency: 2; port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_data: out DATA; end op evict_tail port empty: out Bool; port full: out Bool; end linklist LruList

12.6  Multi-List Shared Node Pool

Multiple linklists can share a single physical node pool RAM — directly analogous to the ram construct's multi-variable store block.  The compiler manages a unified free list across all lists declared in the pool, computes the total depth, and assigns non-overlapping slot ranges.  This is the hardware equivalent of a shared heap.

linklist_pool.arch // Shared node pool for two independent lists. // Physical RAM: depth = 64 + 32 = 96 nodes. // Unified free list: 96-entry FIFO of 7-bit slot indices. // Compiler assigns: // high_pri → slots 0 .. 63 // low_pri → slots 64 .. 95 linklist_pool SchedPool port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // The store block declares all lists that share this pool. // DATA type must be identical across all lists in a pool. store high_pri: linklist<TaskDescriptor, depth 64, kind singly>; low_pri: linklist<TaskDescriptor, depth 32, kind singly>; end store // Each list gets its own operation port set, prefixed by its name op high_pri.insert_tail latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_data: in TaskDescriptor; port resp_valid: out Bool; port resp_handle: out UInt<7>; end op high_pri.insert_tail op high_pri.delete_head latency: 2; port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_data: out TaskDescriptor; end op high_pri.delete_head op low_pri.insert_tail latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_data: in TaskDescriptor; port resp_valid: out Bool; port resp_handle: out UInt<7>; end op low_pri.insert_tail op low_pri.delete_head latency: 2; port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_data: out TaskDescriptor; end op low_pri.delete_head port high_pri.empty: out Bool; port high_pri.full: out Bool; port low_pri.empty: out Bool; port low_pri.full: out Bool; end linklist_pool SchedPool

◈  The shared pool's free list is unified — an alloc from either list draws from the same physical slot space.  The compiler prevents a slot assigned to high_pri from ever being accessed via low_pri addresses.  Overflow of the shared pool is a compile-time warning if the total declared depth exceeds the physical RAM.

12.7  Latency Contract and Pipelining

The latency declaration on each op is a binding contract.  The compiler selects an implementation that guarantees results arrive within the declared number of cycles.  For operations that naturally require multiple RAM accesses, the compiler generates a pipelined controller sub-FSM.

latency_contract.arch // Example: insert_after with latency 3 on a doubly-linked list. // The compiler generates a 3-stage controller pipeline: // cycle 1: read current node's next pointer // cycle 2: write new node (data, next=old_next, prev=current) // cycle 3: patch old_next's prev pointer to new node // // The designer only declares the interface and budget: op insert_after latency: 3; pipelined: true; // allow multiple outstanding requests port req_valid: in Bool; port req_ready: out Bool; port req_handle: in UInt<$clog2(DEPTH)>; // insert after this node port req_data: in DATA; port resp_valid: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; // handle of new node end op insert_after

⚑  pipelined: true tells the compiler the controller may accept a new request before the previous one completes.  The compiler generates hazard detection for conflicting accesses to the same slot.  pipelined: false (default) generates a simpler blocking controller.

12.8  Search with Predicate

The search operation traverses the list and returns the first node whose payload satisfies a declared predicate.  Worst-case latency is DEPTH cycles.  The predicate is a pure combinational expression over the payload type.

linklist_search.arch op search latency: DEPTH; // worst case — full traversal pipelined: false; // one search at a time port req_valid: in Bool; port req_ready: out Bool; // Inline predicate: compiler generates comparator hardware port req_key: in UInt<16>; // search key predicate: payload.task_id == req_key; port resp_valid: out Bool; port resp_found: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; port resp_data: out DATA; end op search

◈  The predicate expression is combinationally evaluated against the payload at each visited node.  It may reference any field of the DATA struct and the req_* ports.  The compiler verifies the predicate is pure — no side effects, no register access.

12.9  Generated Hardware Summary

For a linklist of kind doubly, depth D, and payload type T, the compiler generates the following hardware automatically:

| Generated Component | Size / Type | Notes |
| --- | --- | --- |
| Data RAM | simple_dual, D × ¦T¦ bits | Payload storage |
| Next pointer RAM | simple_dual, D × $clog2(D) bits | Forward links |
| Prev pointer RAM | simple_dual, D × $clog2(D) bits | Backward links (doubly only) |
| Free list FIFO | depth D, width $clog2(D) | Available slot tracking; pre-loaded 0..D-1 at reset |
| Head register | reg, $clog2(D) bits | Points to first node |
| Tail register | reg, $clog2(D) bits | Points to last node (if track tail: true) |
| Length counter | reg, $clog2(D+1) bits | Current occupancy (if track length: true) |
| Controller FSM | 1 per declared op | Meets each op's declared latency contract |
| Search engine | Linear scan FSM | Generated only if op search declared |

⚑  All generated components are named after the linklist instance in the output SystemVerilog, making the generated code fully auditable.  For example: SchedPool_high_pri_data_ram, SchedPool_free_list, SchedPool_ctrl_insert_tail_fsm.

12.10  linklist vs fifo vs ram — When to Use Which

|  | fifo | ram | linklist |
| --- | --- | --- | --- |
| Order | FIFO only | Random access | Any — insert/delete anywhere |
| Access pattern | Head/tail only | Any address | By handle (slot index) |
| Insertion | Tail only | Any address | Head, tail, or after any node |
| Deletion | Head only | Any address | Head, tail, or any node (doubly: O(1)) |
| Physical impl | Counters + RAM | BRAM / SRAM | RAM + free list + controller FSM |
| Latency | 1 cycle | 1–2 cycles | 1–N cycles per op (declared) |
| Dynamic allocation | No | No | Yes — alloc/free per node |
| Reordering | No | N/A | Yes — move nodes without copying payload |

13.  First-Class Construct: cam

A cam (Content-Addressable Memory) is a first-class construct that searches by content rather than by address.  Instead of 'return the value at address X,' a CAM answers 'is key X present, and if so at which index?'  The designer declares the CAM kind, depth, key and value types, match policy, and a lookup latency budget.  The compiler generates the comparator array, priority encoder, replacement logic, and pipeline registers to meet the declared constraints.

13.1  CAM Kinds

| kind | Match Semantics | Don't-Care Bits | Typical Applications |
| --- | --- | --- | --- |
| binary | Exact match only — every key bit must match | No | TLBs, MAC address tables, exact-match classifiers |
| ternary | Each key bit may be 0, 1, or X (wildcard) | Yes | Longest-prefix-match routing, firewall ACLs, packet classifiers |
| associative | Exact match; lookup returns value directly | No | Cache tag arrays, register rename tables, forwarding tables |

13.2  Lookup Latency and Implementation

The latency parameter is the number of clock cycles from lookup request assertion to resp_valid.  The compiler uses this budget to select a comparator implementation:

| Latency Budget | Comparator Strategy | Area | Fmax | Best For |
| --- | --- | --- | --- | --- |
| 1 cycle | Fully parallel — all entries compared simultaneously in one clock | High (O(depth × ¦key¦)) | Moderate | Small CAMs (≤ 64 entries), critical-path TLBs |
| 2 cycles | Two-stage pipelined comparator tree — half in each stage | Medium | High | Medium CAMs (64–256 entries) |
| N cycles | N-stage pipeline — compiler partitions comparator tree evenly | Lower | Highest | Large TCAMs (256–4096 entries) |
| banked | Entries divided into B banks; one bank checked per cycle (rotating) | Lowest | High | Very large CAMs where not all entries need checking every cycle |

⚑  The compiler always meets the declared latency exactly.  If the requested latency is unachievable on the target technology given the key width and depth, it emits a compile-time error with the minimum achievable latency.

13.3  Binary CAM — Exact Match

cam_binary.arch // TLB: 64-entry binary CAM mapping virtual page numbers to physical addresses cam Tlb param DEPTH: const = 64; param VPN_W: const = 27; // virtual page number width (Sv39) param PPN_W: const = 44; // physical page number width port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind binary; match: first_match; // first_match ¦ all_matches key_type: UInt<VPN_W>; value_type: TlbEntry; // struct: PPN, flags, ASID, ... // Replacement policy when CAM is full and a new entry is inserted replace: lru; // lru ¦ fifo ¦ random ¦ none op lookup latency: 1; port req_valid: in Bool; port req_key: in UInt<VPN_W>; port resp_valid: out Bool; port resp_hit: out Bool; port resp_index: out UInt<$clog2(DEPTH)>; port resp_value: out TlbEntry; end op lookup op insert latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<VPN_W>; port req_value: in TlbEntry; port resp_valid: out Bool; port resp_index: out UInt<$clog2(DEPTH)>; // slot used end op insert op invalidate latency: 1; port req_valid: in Bool; port req_index: in UInt<$clog2(DEPTH)>; end op invalidate op invalidate_all latency: 1; port req_valid: in Bool; port resp_valid: out Bool; end op invalidate_all port full: out Bool; port empty: out Bool; end cam Tlb

13.4  Ternary CAM — Wildcard Match

A TCAM stores a key and a mask per entry.  During lookup, a key bit matches if the stored mask bit is X (don't-care) or if the stored key bit equals the lookup key bit.  Entries are evaluated in priority order; the first matching entry wins unless all_matches is specified.

cam_ternary.arch // IPv4 longest-prefix-match routing table cam RoutingTable param DEPTH: const = 1024; param PREFIX_W: const = 32; // IPv4 address width port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind ternary; match: first_match; // priority-ordered; first hit wins (longest prefix) // TCAM key: the prefix bits to match // TCAM mask: X on don't-care bits (host portion of the prefix) key_type: UInt<PREFIX_W>; mask_type: UInt<PREFIX_W>; // 1 = must match, 0 = don't-care value_type: RouteEntry; // struct: next_hop, port, metric, ... // Higher priority index = higher priority in first_match resolution priority: index_ascending; // index_ascending ¦ index_descending ¦ explicit replace: none; // routing table managed explicitly by control plane op lookup latency: 2; // 1024 entries — 2-stage pipelined comparator tree port req_valid: in Bool; port req_key: in UInt<PREFIX_W>; port resp_valid: out Bool; port resp_hit: out Bool; port resp_index: out UInt<$clog2(DEPTH)>; port resp_value: out RouteEntry; end op lookup // TCAM insert: provide key, mask, value, and priority slot op insert latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<PREFIX_W>; port req_mask: in UInt<PREFIX_W>; port req_value: in RouteEntry; port req_index: in UInt<$clog2(DEPTH)>; // explicit priority slot port resp_valid: out Bool; end op insert op delete latency: 1; port req_valid: in Bool; port req_index: in UInt<$clog2(DEPTH)>; end op delete // TCAM-specific: read back the stored key and mask at a given index op read_entry latency: 1; port req_valid: in Bool; port req_index: in UInt<$clog2(DEPTH)>; port resp_valid: out Bool; port resp_key: out UInt<PREFIX_W>; port resp_mask: out UInt<PREFIX_W>; port resp_value: out RouteEntry; end op read_entry port occupancy: out UInt<$clog2(DEPTH+1)>; end cam RoutingTable

13.5  Associative CAM — Direct Value Return

An associative CAM combines lookup and value retrieval in a single operation.  It is the most common form in processor micro-architecture.  The lookup key and its associated value are stored together; a hit returns the value directly without a separate RAM read.

cam_associative.arch // Register rename table: maps architectural register numbers to physical registers cam RenameTable param DEPTH: const = 128; // number of in-flight physical registers param ARCH_W: const = 5; // architectural register number width (32 regs) param PHYS_W: const = 7; // physical register number width (128 regs) port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind associative; match: first_match; key_type: UInt<ARCH_W>; value_type: UInt<PHYS_W>; replace: none; // rename table managed explicitly by rename stage // Two simultaneous lookups for rs1 and rs2 each cycle op lookup latency: 1; ports[2] req valid: in Bool; key: in UInt<ARCH_W>; end ports ports[2] resp valid: out Bool; hit: out Bool; value: out UInt<PHYS_W>; end ports end op lookup op insert latency: 1; port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<ARCH_W>; port req_value: in UInt<PHYS_W>; end op insert op delete_by_key latency: 1; port req_valid: in Bool; port req_key: in UInt<ARCH_W>; end op delete_by_key op delete_by_index latency: 1; port req_valid: in Bool; port req_index: in UInt<$clog2(DEPTH)>; end op delete_by_index end cam RenameTable

◈  Multi-port lookup (ports[2]) is a first-class feature of the cam op block — the same ports[] syntax used in arbiter and regfile.  The compiler replicates the comparator array once per lookup port.  For a 2-port lookup with 128 entries and a 5-bit key, this is 2 × 128 comparators, all evaluated in parallel within one cycle.

13.6  Multi-Table Shared Physical CAM

Multiple logical CAM tables can share one physical CAM array — the same multi-variable mapping pattern used in ram and linklist_pool.  The compiler assigns non-overlapping index ranges to each logical table and generates per-table valid-bit arrays so lookups never cross table boundaries.

cam_bank.arch // Shared TCAM for instruction TLB and data TLB. // Physical CAM: 128 entries total. // Compiler assigns: // itlb → indices 0 .. 63 // dtlb → indices 64 .. 127 cam_bank TlbBank port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind binary; match: first_match; // Both tables share the same key and value types. // The compiler generates a single comparator array of depth 128. key_type: UInt<27>; // virtual page number value_type: TlbEntry; store itlb: cam<depth 64, replace lru>; dtlb: cam<depth 64, replace lru>; end store op itlb.lookup latency: 1; port req_valid: in Bool; port req_key: in UInt<27>; port resp_valid: out Bool; port resp_hit: out Bool; port resp_value: out TlbEntry; end op itlb.lookup op itlb.insert latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<27>; port req_value: in TlbEntry; port resp_valid: out Bool; end op itlb.insert op dtlb.lookup latency: 1; port req_valid: in Bool; port req_key: in UInt<27>; port resp_valid: out Bool; port resp_hit: out Bool; port resp_value: out TlbEntry; end op dtlb.lookup op dtlb.insert latency: 2; port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<27>; port req_value: in TlbEntry; port resp_valid: out Bool; end op dtlb.insert op itlb.invalidate_all latency: 1; port req_valid: in Bool; end op itlb.invalidate_all op dtlb.invalidate_all latency: 1; port req_valid: in Bool; end op dtlb.invalidate_all end cam_bank TlbBank

⚑  A lookup on itlb only evaluates entries in the itlb index range (0–63).  The compiler generates a range-masked hit-vector so dtlb entries never produce a false hit on an itlb lookup, and vice versa.  This masking is zero-overhead — it is absorbed into the valid-bit array of the comparator.

13.7  Match Policies

| Policy | Returns | Generated Hardware | Typical Use |
| --- | --- | --- | --- |
| first_match | Index and value of lowest-priority matching entry | Priority encoder over hit vector | TLBs, routing (longest prefix = highest priority) |
| all_matches | A hit_vec: Vec<Bool, DEPTH> — one bit per entry | Hit vector register; no priority encoder | Firewall logging, multicast replication |
| priority | Index and value of highest-priority matching entry (explicit ordering) | Priority encoder, compiler generates order from entry metadata | ACL rules with explicit priority levels |

13.8  Replacement Policies

| Policy | Mechanism | Generated Hardware | Best For |
| --- | --- | --- | --- |
| lru | Evict least-recently-used entry on insert when full | LRU tracking list (uses linklist<doubly> internally) | TLBs, cache tag arrays |
| fifo | Evict oldest-inserted entry | Age counter per entry | Simple classifiers, approximate LRU |
| random | Evict a pseudo-random entry | LFSR | Area-constrained designs, L1 caches |
| plru | Pseudo-LRU tree approximation | Binary tree of LRU bits | Large TLBs where true LRU is too costly |
| none | No auto-eviction; insert fails when full | Valid-bit check only | Software-managed tables (routing, ACLs) |

◈  When replace: lru is selected, the compiler automatically instantiates a linklist<doubly> construct internally to manage the LRU ordering — this is a clean example of first-class constructs composing with each other.

13.9  Generated Hardware Summary

For a cam of kind binary, depth D, key width K, value width V, and latency L, the compiler generates:

| Generated Component | Size | Notes |
| --- | --- | --- |
| Key storage RAM | D × K bits | One key per entry; simple_dual for concurrent lookup+insert |
| Value storage RAM | D × V bits | Associated value; separate from key array for timing |
| Valid bit array | D × 1 bit | Marks occupied entries; part of every comparator gate |
| Mask array (TCAM only) | D × K bits | Per-bit don't-care mask; absent for binary/associative |
| Comparator array | D × K gates per lookup port | All entries compared in parallel within latency budget |
| Pipeline registers | (L-1) stages | Cut into comparator tree to meet latency; zero if L=1 |
| Priority encoder | $clog2(D) bits | Reduces hit vector to index; one per match policy output |
| LRU linklist (if replace:lru) | doubly-linked, depth D | Compiler-instantiated; tracks access order for eviction |
| Replacement controller FSM | 1 FSM | Selects eviction candidate and writes new entry on insert |

13.10  cam vs ram vs linklist — Choosing the Right Construct

|  | ram | linklist | cam |
| --- | --- | --- | --- |
| Access model | By address | By handle (slot index) | By content (key match) |
| Search | O(N) — must scan externally | O(N) — traverse with predicate | O(1) — parallel comparators |
| Key storage | No — caller manages | No | Yes — stored alongside value |
| Dynamic alloc | No | Yes — alloc/free | Yes — insert/evict |
| Don't-care matching | No | No | Yes — TCAM only |
| Multi-port lookup | Via multiple port decls | No | Yes — ports[N] on op lookup |
| LRU tracking | Manual | Native — kind doubly | Compiler-generated (replace: lru) |
| FPGA primitive | BRAM / URAM | BRAM + LUT logic | LUT array (no dedicated primitive) |
| Area scaling | O(depth) | O(depth) | O(depth × key_width) per port |

14.  First-Class Construct: crossbar

A crossbar is a fully-connected N×M switch fabric.  Any of the N input ports can be routed to any of the M output ports simultaneously, subject to the constraint that each output can accept at most one input per cycle.  This is fundamentally different from an arbiter — an arbiter selects among N requesters for one resource; a crossbar routes N streams to M destinations concurrently.

The compiler generates the per-output arbiters, the routing mux array, the input-side backpressure logic, and the optional virtual-channel buffers.  The designer declares topology, data type, routing function, and arbitration policy.

14.1  Declaration

crossbar.arch // 4-input × 4-output crossbar for a 2D mesh NoC router crossbar NoC4x4 param N: const = 4; // number of input ports param M: const = 4; // number of output ports param FLIT_W: const = 64; // data width per port port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // Routing: input port declares destination as a tag in the data stream // The crossbar extracts the tag and steers the flit accordingly routing: by_tag; tag_field: UInt<$clog2(M)>; // high bits of each flit carry destination tag_offset: FLIT_W - $clog2(M); // bit position of tag within flit // Per-output arbitration policy when multiple inputs target the same output policy: round_robin; // Optional: virtual channels per input port to avoid head-of-line blocking virtual_channels: 2; ports[N] in_port valid: in Bool; ready: out Bool; data: in UInt<FLIT_W>; end ports ports[M] out_port valid: out Bool; ready: in Bool; data: out UInt<FLIT_W>; end ports end crossbar NoC4x4

14.2  Routing Modes

| mode | How Destination is Determined | Generated Hardware |
| --- | --- | --- |
| by_tag | Destination index encoded in a bit field of each flit | Tag extractor + mux per output port |
| by_table | External routing table (cam) maps source+tag → output port | Tag extractor + cam lookup + mux |
| by_port | Explicit per-input destination signal alongside the data | Dedicated dest port per input + mux |
| broadcast | One input drives all outputs simultaneously | Fanout + per-output backpressure AND reduction |

14.2.1 Compiler Warnings

The compiler emits warnings (non-fatal, printed before "OK: no errors") for the following:

- `todo!` sites — every todo! expression or block body emits a warning. Simulation aborts if one is reached at runtime.

- Redundant reset branch — a seq block whose top-level if tests a reset signal that also appears in a reset signal=>value declaration. The if branch is dead code because the declaration already generates an outer reset guard:

reg q: UInt<8> reset rst => 0;

seq on clk rising

if rst // WARNING: redundant — dead inside the outer reset guard

q <= 0;

else

q <= d;

end if

end seq

Correct form:

seq on clk rising

q <= d; // reset guard is generated from the declaration

end seq

⚑ The output guarantee means synthesis tools receive RTL that passes lint clean with no attributes, no translate_off pragmas, and no synthesis workarounds required.

14.2.1 Compiler Warnings

The compiler emits warnings (non-fatal, printed before "OK: no errors") for the following:

- `todo!` sites — every todo! expression or block body emits a warning. Simulation aborts if one is reached at runtime.

- Redundant reset branch — a seq block whose top-level if tests a reset signal that also appears in a reset signal=>value declaration. The if branch is dead code because the declaration already generates an outer reset guard:

reg q: UInt<8> reset rst => 0;

seq on clk rising

if rst // WARNING: redundant — dead inside the outer reset guard

q <= 0;

else

q <= d;

end if

end seq

Correct form:

seq on clk rising

q <= d; // reset guard is generated from the declaration

end seq

⚑ The output guarantee means synthesis tools receive RTL that passes lint clean with no attributes, no translate_off pragmas, and no synthesis workarounds required.

14.3  Structural Variants

crossbar_variant.arch // Non-square: 8 inputs, 2 outputs — concentrator topology crossbar MemConcentrator param N: const = 8; param M: const = 2; param W: const = 512; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; routing: by_port; policy: weighted<8'b11110000>; // first 4 inputs weighted higher ports[N] in_port valid: in Bool; ready: out Bool; data: in UInt<W>; dest: in UInt<$clog2(M)>; // explicit destination (by_port mode) end ports ports[M] out_port valid: out Bool; ready: in Bool; data: out UInt<W>; end ports end crossbar MemConcentrator

◈  The compiler generates one arbiter instance (using the declared policy) per output port, plus an N-input mux per output.  For N=8, M=2, this is 2 arbiters and 2 mux arrays — all correctly wired and backpressure-connected.

15.  First-Class Construct: scoreboard

A scoreboard tracks in-flight operations and their data dependencies.  It decides whether a new operation can be issued (all source operands are ready) or must be stalled (a previous operation will write a needed operand).  It is the central hazard-management unit of any pipelined or out-of-order design.

The designer declares the number of tracked entries, the number of simultaneous issue and writeback ports, and which hazard classes to detect.  The compiler generates the dependency matrix, the ready-bit array, the issue gate logic, and the stall propagation signals.

15.1  Hazard Classes

| Hazard | Full Name | Condition | Default Action |
| --- | --- | --- | --- |
| RAW | Read-After-Write | Consumer issued before producer writes back | Stall consumer until writeback |
| WAW | Write-After-Write | Two writers to the same destination in flight | Stall second writer |
| WAR | Write-After-Read | Writer issued before prior reader completes | Stall writer (only matters in OOO) |
| Structural | Resource conflict | Two ops need the same functional unit | Stall later op |

15.2  Declaration

scoreboard.arch // Scoreboard for a 4-wide issue, 2-wide writeback in-order pipeline scoreboard IssueBoard param ENTRIES: const = 32; // max in-flight operations param REG_W: const = 5; // register address width (32 arch regs) param ISSUE_W: const = 4; // simultaneous issue ports param WB_W: const = 2; // simultaneous writeback ports port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; hazards: [raw, waw]; // detect RAW and WAW; WAR not needed for in-order // Issue ports — one per simultaneously-issued operation ports[ISSUE_W] issue valid: in Bool; ready: out Bool; // scoreboard grants or stalls rs1_addr: in UInt<REG_W>; rs2_addr: in UInt<REG_W>; rd_addr: in UInt<REG_W>; rd_valid: in Bool; // does this op write a destination register? token: out UInt<$clog2(ENTRIES)>; // scoreboard entry handle end ports // Writeback ports — mark an entry complete and release its destination ports[WB_W] writeback valid: in Bool; token: in UInt<$clog2(ENTRIES)>; end ports // Flush: invalidate all entries (branch mispredict, exception) port flush_valid: in Bool; // Status port full: out Bool; port empty: out Bool; end scoreboard IssueBoard

15.3  Scoreboard with Structural Hazards

scoreboard_ooo.arch // OOO scoreboard detecting all hazard types plus functional unit conflicts scoreboard OooBoard param ENTRIES: const = 64; param REG_W: const = 7; // 128 physical registers (post-rename) param ISSUE_W: const = 4; param WB_W: const = 4; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; hazards: [raw, waw, war, structural]; // Functional unit availability — structural hazard inputs port alu_free: in Bool; port mul_free: in Bool; port lsu_free: in Bool; port fpu_free: in Bool; ports[ISSUE_W] issue valid: in Bool; ready: out Bool; rs1_addr: in UInt<REG_W>; rs2_addr: in UInt<REG_W>; rs3_addr: in UInt<REG_W>; // FMA third source rd_addr: in UInt<REG_W>; rd_valid: in Bool; fu_type: in FuType; // which functional unit needed latency: in UInt<4>; // declared execution latency token: out UInt<$clog2(ENTRIES)>; end ports ports[WB_W] writeback valid: in Bool; token: in UInt<$clog2(ENTRIES)>; end ports port flush_valid: in Bool; port full: out Bool; end scoreboard OooBoard

⚑  When latency: is provided per issue, the scoreboard automatically generates a countdown timer per entry and self-completes the entry when the timer expires — eliminating the need for explicit writeback on fixed-latency operations.

16.  First-Class Construct: reorder_buf

A reorder buffer (ROB) is a circular buffer that allows operations to complete out-of-order while retiring their results strictly in-order.  Entries are allocated at the tail in program order on issue, can be marked complete in any order as operations finish, and are committed from the head in order.  The ROB is the commit stage of every out-of-order processor.

The Arch reorder_buf construct captures this circular, decoupled head/tail structure which cannot be expressed as a plain fifo.  The designer declares the number of entries, the result payload type, the number of allocate and complete ports, and the commit policy.  The compiler generates the circular index logic, the status array, the head/tail advancement logic, and the in-order commit gating.

16.1  Declaration

rob.arch // ROB for a 4-wide issue, 2-wide commit OOO processor reorder_buf Rob param DEPTH: const = 128; // max in-flight instructions param ALLOC_W: const = 4; // simultaneous allocation ports (= issue width) param COMMIT_W: const = 2; // simultaneous commit ports port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // Payload stored per ROB entry — whatever the commit stage needs entry_type: RobEntry; // Allocation: reserve ROB slots for newly-issued instructions ports[ALLOC_W] alloc valid: in Bool; ready: out Bool; data: in RobEntry; token: out UInt<$clog2(DEPTH)>; // ROB index — carried through pipeline end ports // Complete: mark a slot done and write back its result // May arrive in any order ports[ALLOC_W] complete valid: in Bool; token: in UInt<$clog2(DEPTH)>; result: in RobEntry; // updated entry (result, exception flags, etc.) end ports // Commit: head entries that are complete retire in-order ports[COMMIT_W] commit valid: out Bool; data: out RobEntry; token: out UInt<$clog2(DEPTH)>; ack: in Bool; // downstream (regfile, store buffer) accepts commit end ports // Flush: squash all entries from a given token to the tail // Used on branch mispredict or exception port flush_valid: in Bool; port flush_token: in UInt<$clog2(DEPTH)>; // Status port full: out Bool; port empty: out Bool; port occupancy: out UInt<$clog2(DEPTH+1)>; end reorder_buf Rob

16.2  Commit Policies

| Policy | Behaviour | Use When |
| --- | --- | --- |
| in_order | Commit head entry only when it is marked complete; stall if head not done | Most OOO processors — precise exception model required |
| speculative | Commit head even if not complete; mark as speculative; confirm or squash later | Hardware transactional memory, speculative stores |
| exception_stop | Stop committing at first entry with exception flag set; flush tail | Precise exception delivery — default for RISC-V, x86 |

16.3  ROB Entry Type

rob_entry.arch // Typical ROB entry for an integer OOO pipeline struct RobEntry pc: UInt<64>, // program counter of the instruction rd_addr: UInt<7>, // physical destination register rd_valid: Bool, // does this inst write a register? result: UInt<64>, // computed result value complete: Bool, // has execution finished? exception: Bool, // did this inst raise an exception? exc_cause: UInt<6>, // exception code store_ptr: UInt<5>, // index into store buffer, if applicable is_store: Bool, is_branch: Bool, mispredict: Bool, // branch was mispredicted target_pc: UInt<64>, // correct PC after branch resolution end struct RobEntry // The reorder_buf compiler only stores and retrieves this struct. // It never inspects individual fields — complete and exception are // the only fields the ROB controller itself reads.

◈  The ROB controller reads only the complete and exception fields internally.  All other fields are stored opaquely and returned verbatim at commit time.  This keeps the reorder_buf construct general — it works for any pipeline by changing the entry_type.

16.4  Connecting scoreboard and reorder_buf Together

ooo_frontend.arch // Typical OOO front-end: scoreboard gates issue; ROB orders commit module OooFrontEnd port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // ... other ports ... inst sb: OooBoard // scoreboard from §15 connect clk <- clk; connect rst <- rst; // issue port 0 connected to decode stage output connect issue[0].valid <- dec_valid; connect issue[0].rs1_addr <- dec_rs1; connect issue[0].rs2_addr <- dec_rs2; connect issue[0].rd_addr <- dec_rd; connect issue[0].rd_valid <- dec_rd_en; connect issue[0].ready -> can_issue; connect issue[0].token -> sb_token; // writeback from execution units connect writeback[0].valid <- ex0_done; connect writeback[0].token <- ex0_token; connect flush_valid <- flush; end inst sb inst rob: Rob connect clk <- clk; connect rst <- rst; // allocate one ROB entry per issued instruction connect alloc[0].valid <- can_issue; connect alloc[0].data <- dec_rob_entry; connect alloc[0].token -> rob_token; // complete from execution units connect complete[0].valid <- ex0_done; connect complete[0].token <- ex0_rob_token; connect complete[0].result <- ex0_result; // commit to architectural state connect commit[0].valid -> commit_valid; connect commit[0].data -> commit_entry; connect commit[0].ack <- retire_ack; connect flush_valid <- flush; connect flush_token <- flush_rob_token; end inst rob end module OooFrontEnd

17.  First-Class Construct: counter

A counter is a first-class construct that captures the full variety of hardware counting patterns — saturating, wrapping, up/down, gray-code, one-hot, and Johnson.  Each mode has distinct synthesis implications that a raw reg block does not express.  The compiler generates the correct arithmetic, encoding, and overflow/underflow detection for each mode, and catches impossible parameter combinations at compile time.

17.1  Counter Modes

| mode | Behaviour at Limit | Encoding | Primary Use |
| --- | --- | --- | --- |
| wrap | Overflows back to 0 (or max) | Binary | FIFOs, address generators, time slices |
| saturate | Clamps at max (or 0) and holds | Binary | Rate limiters, credit counters, performance monitors |
| gray | Increments through Gray code sequence | Gray | CDC — only one bit changes per increment; safe across domains |
| one_hot | Single hot bit shifts through N positions | One-hot | State indicators, rotating selectors, low-power FSMs |
| johnson | Shift-register ring — 2N states from N bits | Johnson | Low-power clock dividers, phase generators |

17.2  Declaration

counter.arch // Saturating credit counter — clamps at 0 and MAX_CREDITS counter CreditCounter param MAX: const = 64; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; mode: saturate; direction: up_down; // up ¦ down ¦ up_down init: MAX; // reset value port inc: in Bool; port dec: in Bool; port value: out UInt<$clog2(MAX+1)>; port at_max: out Bool; // combinational: value == MAX port at_min: out Bool; // combinational: value == 0 end counter CreditCounter // Gray-code counter for safe clock-domain crossing counter GrayPtrCounter param DEPTH: const = 256; port clk: in Clock<WriteDomain>; port rst: in Reset<Async>; mode: gray; direction: up; init: 0; port inc: in Bool; port gray_value: out UInt<$clog2(DEPTH)>; // safe to sample across domain port bin_value: out UInt<$clog2(DEPTH)>; // binary equivalent, same domain only end counter GrayPtrCounter

17.3  One-Hot and Johnson Counters

counter_onehot.arch // One-hot counter: exactly one bit active, shifts each cycle counter RoundRobinSel param N: const = 8; // number of positions port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; mode: one_hot; direction: up; init: 0; // start at position 0 (bit 0 hot) port advance: in Bool; port sel: out Vec<Bool, N>; // one-hot selection vector port index: out UInt<$clog2(N)>; // binary equivalent end counter RoundRobinSel // Johnson counter: 2N states from N flip-flops, glitch-free transitions counter PhaseGen param N: const = 4; // generates 8 phases port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; mode: johnson; direction: up; init: 0; port advance: in Bool; port phase: out UInt<N>; // Johnson-encoded phase end counter PhaseGen

⚑  The compiler rejects impossible combinations at compile time — for example, mode: gray with direction: up_down is undefined (Gray codes only have a defined unidirectional sequence) and mode: one_hot with saturate semantics is self-contradictory.

18.  First-Class Construct: pqueue

A hardware priority queue (pqueue) maintains a dynamic set of elements and provides O(log N) insert and O(1) extract of the minimum or maximum element.  It is implemented as a pipelined binary heap in a RAM.  The construct is fundamentally different from fifo (which has no ordering), linklist (which is sequentially ordered by insertion), and arbiter (which selects among fixed simultaneous requesters).  Primary uses include hardware task schedulers, packet schedulers (WFQ, DRR, LSTF), and timer management units.

18.1  Declaration

pqueue.arch // Hardware min-heap task scheduler: 256 tasks, 32-bit priority key pqueue TaskScheduler param DEPTH: const = 256; param KEY_W: const = 32; // priority key width param DATA_W: type = TaskDescriptor; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; order: min_first; // min_first ¦ max_first // Insert: add an element with its priority key // Latency: O(log2 DEPTH) cycles — heap sift-up op insert latency: $clog2(DEPTH); // 8 cycles for depth 256 port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<KEY_W>; port req_data: in DATA_W; port resp_valid: out Bool; end op insert // Extract: remove and return the top element (min or max) // Latency: O(log2 DEPTH) cycles — heap sift-down after removal op extract latency: $clog2(DEPTH); port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_key: out UInt<KEY_W>; port resp_data: out DATA_W; end op extract // Peek: read the top element without removing it — O(1), combinational op peek latency: 0; port resp_key: out UInt<KEY_W>; port resp_data: out DATA_W; end op peek // Update: change the priority key of an element already in the heap // Requires a handle returned at insert time op update latency: $clog2(DEPTH); port req_valid: in Bool; port req_ready: out Bool; port req_handle: in UInt<$clog2(DEPTH)>; port req_new_key: in UInt<KEY_W>; port resp_valid: out Bool; end op update port full: out Bool; port empty: out Bool; port occupancy: out UInt<$clog2(DEPTH+1)>; end pqueue TaskScheduler

18.2  Pipelining

The latency of insert and extract is O(log₂ DEPTH) by the nature of heap sift operations.  The compiler generates a pipelined sift controller that can accept a new request every cycle even while a sift is in progress, provided the new request does not conflict with an in-flight sift on the same heap path.  The compiler generates the conflict detection logic automatically.

pqueue_pipelined.arch // Pipelined insert: new insert accepted every cycle if no path conflict op insert latency: $clog2(DEPTH); pipelined: true; // compiler generates sift-path conflict detection port req_valid: in Bool; port req_ready: out Bool; port req_key: in UInt<KEY_W>; port req_data: in DATA_W; port resp_valid: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; // heap slot for update/cancel end op insert

18.3  Composite Priority Keys

Real schedulers rarely use a single integer key.  Arch allows struct types as priority keys with a declared comparison function.

pqueue_composite.arch // Packet scheduler key: deadline + flow_id for tie-breaking struct SchedKey deadline: UInt<32>, flow_id: UInt<16>, end struct SchedKey pqueue PacketSched param DEPTH: const = 512; param KEY_W: type = SchedKey; param DATA_W: type = PacketDescriptor; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; order: min_first; // Comparison function: primary key = deadline (ascending), // tie-break = flow_id (ascending) compare: fn(a: SchedKey, b: SchedKey) -> Bool a.deadline < b.deadline or (a.deadline == b.deadline and a.flow_id < b.flow_id) end fn; op insert latency: $clog2(DEPTH); pipelined: true; port req_valid: in Bool; port req_ready: out Bool; port req_key: in SchedKey; port req_data: in PacketDescriptor; port resp_valid: out Bool; port resp_handle: out UInt<$clog2(DEPTH)>; end op insert op extract latency: $clog2(DEPTH); port req_valid: in Bool; port req_ready: out Bool; port resp_valid: out Bool; port resp_key: out SchedKey; port resp_data: out PacketDescriptor; end op extract op peek latency: 0; port resp_key: out SchedKey; port resp_data: out PacketDescriptor; end op peek port empty: out Bool; port full: out Bool; end pqueue PacketSched

◈  The compare function is a pure combinational expression over the key type.  The compiler uses it as the comparison gate at every level of the heap, generating the correct comparator hardware throughout the sift controller.  It may reference any field of the key struct and any arithmetic operator.

18.4  Generated Hardware

| Component | Implementation | Notes |
| --- | --- | --- |
| Heap storage RAM | simple_dual, DEPTH × (KEY_W + DATA_W) | Stores key+data per node; index 0 = root |
| Handle→index table | cam<associative> | Maps user handle to current heap position; updated on every sift |
| Sift-up controller | Pipeline FSM, log₂(DEPTH) stages | Executes insert and bubble-up |
| Sift-down controller | Pipeline FSM, log₂(DEPTH) stages | Executes extract and trickle-down |
| Comparator | Combinational, generated from compare fn | One instance per heap level |
| Conflict detector | Bitmask of active sift paths | Blocks pipelined requests that share a heap path |
| Occupancy counter | counter<wrap> | Tracks current size; drives full/empty |

19.  Compile-Time Generation: generate

The generate system allows any structural element of an Arch construct — ports, instances, connections, registers, and assertions — to be created by compile-time iteration or conditional evaluation.  This is a fundamental capability that SystemVerilog generate lacks: in SystemVerilog, the port list of a module is always a fixed declaration; generate can add internal logic but cannot add or remove ports.  In Arch, generate operates before elaboration, so generated ports are indistinguishable from hand-written ports from the perspective of the type system, safety checks, and callers.

19.1  generate for — Iteration

A generate for loop iterates a compile-time constant range and emits one copy of its body per iteration.  The loop variable is a compile-time integer constant within the body.  It may appear at the top level of any construct body to generate ports, instances, registers, connections, or assertions.

gen_for.arch // Syntax generate for VARNAME in START..END // inclusive range // body — VARNAME is a compile-time const within here end generate for VARNAME generate for VARNAME in START..END step STEP // body end generate for VARNAME // Example: generate 8 input ports of varying width module FanIn param N: const = 8; param BASE: const = 4; // port[i] has width BASE*(i+1) generate for i in 0..N-1 port in[i]: in UInt<BASE * (i + 1)>; port vld[i]: in Bool; end generate for i port out: out UInt<BASE * N>; port out_vld: out Bool; end module FanIn

◈  The range START..END is inclusive on both ends.  START..END step STEP increments by STEP each iteration.  All three expressions must be compile-time constants derivable from param declarations.

19.2  generate if — Conditional Ports and Logic

A generate if block is evaluated entirely at compile time.  If the condition is true, the body is emitted; if false, it is as if the block was never written.  This enables optional ports — a capability that does not exist in SystemVerilog.

gen_if.arch // Syntax generate if CONDITION // emitted only when CONDITION is true at compile time end generate if generate if CONDITION // true branch end generate if generate else // false branch end generate else // Example: optional debug port — only exists when DEBUG_EN param is true module Accelerator param DEBUG_EN: const = false; param DATA_W: const = 32; param DEPTH: const = 256; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port data_in: in UInt<DATA_W>; port data_out: out UInt<DATA_W>; // These ports only exist when DEBUG_EN is true. // When DEBUG_EN is false, connecting to them is a compile error. generate if DEBUG_EN port dbg_pc: out UInt<32>; port dbg_state: out UInt<8>; port dbg_valid: out Bool; end generate if // Optional second memory port for DEPTH > 512 — bandwidth required generate if DEPTH > 512 port mem_port_b en: in Bool; addr: in UInt<$clog2(DEPTH)>; data: out UInt<DATA_W>; end port mem_port_b end generate if end module Accelerator

⚑  A caller that instantiates Accelerator with DEBUG_EN = false and attempts to connect dbg_pc receives a compile-time error: port dbg_pc does not exist when DEBUG_EN = false.  The type system is fully aware of which ports exist for each parameter combination.

19.3  generate match — Structural Variant Selection

A generate match selects one of several structural bodies based on the value of a compile-time expression.  It is the generate-level equivalent of a type-level switch.  It is commonly used to select between different internal implementations based on a mode parameter.

gen_match.arch // Syntax generate match EXPRESSION case VALUE_A: // body for VALUE_A end case case VALUE_B: // body for VALUE_B end case default: // fallback body end default end generate match // Example: select internal FIFO implementation based on a mode parameter module DataPath param FIFO_MODE: const = 'standard'; // 'standard' ¦ 'async' ¦ 'showahead' param DEPTH: const = 64; param WIDTH: type = UInt<32>; port clk_wr: in Clock<WriteDomain>; port clk_rd: in Clock<ReadDomain>; port rst: in Reset<Async>; generate match FIFO_MODE case 'standard': // Single-clock FIFO — both clocks must be same domain fifo DataFifo param DEPTH = DEPTH; param WIDTH = WIDTH; port clk <- clk_wr; port push_valid <- push_vld; end fifo DataFifo end case case 'async': // Dual-clock FIFO — compiler generates gray-code CDC fifo DataFifo param DEPTH = DEPTH; param WIDTH = WIDTH; port wr_clk <- clk_wr; port rd_clk <- clk_rd; port push_valid <- push_vld; end fifo DataFifo end case case 'showahead': // FIFO with combinational pop — registered internally, async output fifo DataFifo param DEPTH = DEPTH; param WIDTH = WIDTH; port clk <- clk_wr; read: async; port push_valid <- push_vld; end fifo DataFifo end case end generate match end module DataPath

19.4  Generated Instance Arrays — With Inter-Instance Wiring

The primary structural use of generate for is instantiating N copies of a submodule and wiring them together.  Arch handles the common case of chain wiring — where instance i receives output from instance i-1 — with a boundary expression using the ?: operator.  The boundary condition is evaluated at compile time for i=0.

systolic_generate.arch // Systolic array: N processing elements wired in a chain // The sum_in of PE[0] is 0; for PE[i>0] it is PE[i-1].sum_out pipeline SystolicArray param SIZE: const = 4; // change to 8, 16, 32 — no other changes needed param ACC_W: const = 32; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port en: in Bool; // Generated ports — SIZE input activations, SIZE weight inputs, SIZE results // This is impossible in SystemVerilog — ports cannot be generated. generate for i in 0..SIZE-1 port a_in[i]: in SInt<8>; port b_in[i]: in SInt<8>; port result[i]: out SInt<ACC_W>; end generate for i stage Compute generate for i in 0..SIZE-1 inst pe[i]: SystolicPE param ACC_W = ACC_W; connect clk <- clk; connect rst <- rst; connect en <- en; connect a_in <- a_in[i]; connect b_in <- b_in[i]; // Boundary expression: PE[0] gets 0; PE[i] gets PE[i-1].sum_out connect sum_in <- i == 0 ? 0.sext<ACC_W>() : pe[i-1].sum_out; connect sum_out -> result[i]; end inst pe[i] end generate for i end stage Compute stall when en == false; // Generated assertions — one overflow check per PE generate for i in 0..SIZE-1 assert pe_valid[i]: pe[i].acc_reg >= -SInt<ACC_W>().min() and pe[i].acc_reg <= SInt<ACC_W>().max(); end generate for i end pipeline SystolicArray // Instantiation — the caller sees SIZE ports by name inst array: SystolicArray param SIZE = 8; // scale from 4×4 to 8×8 by changing one param param ACC_W = 32; connect clk <- clk; connect en <- compute_en; // Generated ports accessed by index generate for i in 0..7 connect a_in[i] <- act_row[i]; connect b_in[i] <- wgt_col[i]; connect result[i] -> output_row[i]; end generate for i end inst array

◈  The SIZE parameter change from 4 to 8 requires one edit.  The compiler regenerates all 8 port declarations, 8 PE instances with correct chain wiring, and 8 overflow assertions.  In SystemVerilog, the equivalent change requires rewriting 64 PE instantiations, 8 port declarations, and every inter-instance wire.

19.5  Generated Ports on interfaces

Generated ports are fully supported inside interface declarations.  This allows a parameterized interface whose port bundle varies with configuration — a common need in bus protocols and configurable datapaths.

gen_interface.arch // AXI-lite interface with a configurable number of ID bits interface AxiLite param ADDR_W: const = 32; param DATA_W: const = 32; param ID_W: const = 0; // 0 = no ID field (standard AXI-lite) port awvalid: out Bool; port awready: in Bool; port awaddr: out UInt<ADDR_W>; // ID field is optional — only present when ID_W > 0 generate if ID_W > 0 port awid: out UInt<ID_W>; end generate if port wvalid: out Bool; port wready: in Bool; port wdata: out UInt<DATA_W>; port wstrb: out UInt<DATA_W / 8>; port bvalid: in Bool; port bready: out Bool; port bresp: in UInt<2>; port arvalid: out Bool; port arready: in Bool; port araddr: out UInt<ADDR_W>; generate if ID_W > 0 port arid: out UInt<ID_W>; port rid: in UInt<ID_W>; end generate if port rvalid: in Bool; port rready: out Bool; port rdata: in UInt<DATA_W>; port rresp: in UInt<2>; end interface AxiLite // A module using this interface with ID_W=4 module DmaEngine port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port axi: out AxiLite<ADDR_W 32, DATA_W 32, ID_W 4>; // Compiler knows axi.awid and axi.arid exist — ID_W=4 > 0 end module DmaEngine // A module using standard AXI-lite (no ID) module SimplePeripheral port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port axi: in AxiLite<ADDR_W 32, DATA_W 32, ID_W 0>; // Compiler knows axi.awid does NOT exist — compile error if accessed end module SimplePeripheral

⚑  When ID_W = 0, any attempt to connect axi.awid is a compile-time error.  This is structurally impossible in SystemVerilog — optional fields in an interface must be worked around with unused signals or conditional compilation macros.

19.6  generate for over Type Lists

A generate for loop may iterate over a list of types in addition to integer ranges.  This allows generating ports or instances of different types within one loop — useful for heterogeneous datapaths.

gen_typelist.arch // Generate three accumulator banks of different widths module MultiPrecisionAcc param N: const = 4; // accumulators per precision port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // Type list — three precisions generate for T in [UInt<8>, UInt<16>, UInt<32>] generate for i in 0..N-1 port in_T_[i]: in T; port acc_T_[i]: out T; reg r_T_[i]: T init 0; reg r_T_[i] on clk rising, rst high if rst r_T_[i] <= 0; end if else r_T_[i] <= r_T_[i] + in_T_[i]; end else end reg comb acc_T_[i] = r_T_[i]; end comb end generate for i end generate for T end module MultiPrecisionAcc // Generates: in_UInt8_0..3, acc_UInt8_0..3, // in_UInt16_0..3, acc_UInt16_0..3, // in_UInt32_0..3, acc_UInt32_0..3

◈  The type name is mangled into the port name using underscore concatenation.  Generated port names are always deterministic and derivable from the parameter values — the compiler documents the name mapping in the output port list for callers.

19.7  Constraints and Compile-Time Errors

| Rule | Error Raised When |
| --- | --- |
| Range must be const | Loop bound contains a runtime expression — e.g., depends on a port value |
| No overlap between generated and manual ports | A hand-written port name matches a generated port name |
| Boundary expression must be type-safe | i-1 boundary in connect sum_in <- i==0 ? 0 : pe[i-1].sum_out has mismatched types |
| generate if condition must be const | Condition references a port signal rather than a param |
| Caller must satisfy all generated ports | Instantiating a module with generate for ports without connecting all generated ports is an error |
| Type list generate must produce uniform structure | Each type in the list must produce the same structural shape so the compiler can verify completeness |

19.8  generate vs ports[] — When to Use Which

|  | ports[N] | generate for ports |
| --- | --- | --- |
| Port types | All ports in the bundle are identical | Ports may differ by index — different types, widths, or names |
| Count source | Must be a single const param | Any compile-time expression, including inequalities |
| Conditional existence | No — all N ports always exist | Yes — generate if wraps the entire group |
| Inter-port arithmetic | Not applicable | Yes — port[i] can reference port[i-1] |
| Interface reuse | Works with ports[] bundles | Works with interface declarations |
| Best for | Uniform bus: N identical data channels | Heterogeneous: chain wiring, precision banks, optional ports |

19.9  SystemVerilog Comparison

| Feature | Arch generate | SystemVerilog generate |
| --- | --- | --- |
| Generate ports | Yes — generate for and generate if can add ports | No — port list is fixed at module declaration |
| Optional ports | Yes — generate if PARAM > 0 wrapping port decl | No — all ports always exist; unused signals are left unconnected |
| Conditional interface fields | Yes — inside interface declarations | No — interface port list is fixed |
| Named loop variable in body | Yes — i is a named const throughout the body | Yes — genvar i |
| Named block endings | end generate for i — unambiguous | end — no name; nesting requires counting ends |
| Inter-instance wiring | pe[i-1].sum_out with boundary expression | Separate genvar + generate if for boundary case |
| Type list iteration | Yes — generate for T in [UInt<8>, UInt<16>] | No — must use parameters and conditional casts |
| Assertion generation | Yes — assert inside generate for body | Yes — property/assert inside generate |
| Compile-time error quality | Error names the parameter and generated construct | Generic elaboration error with line number only |

20.  Native Execution and Compiled Simulation

Arch compiles directly to a native binary for simulation.  No external simulator — no VCS, ModelSim, Questa, or Icarus — is required.  The arch sim command produces a self-contained executable that models the design cycle-accurately, evaluates all assertions, accumulates coverage, and optionally writes waveform files.  This is possible because Arch's type system and structural rules eliminate the sources of complexity that make traditional HDL simulation hard to compile efficiently.

20.1  Why Arch Can Be Natively Compiled

Traditional Verilog simulators carry a heavyweight runtime because the language permits constructs that require dynamic resolution at simulation time.  Arch eliminates every one of them at the language level:

| Verilog/SV Runtime Complexity | Arch Equivalent | Enables |
| --- | --- | --- |
| Delta cycles — multiple zero-time evaluation passes until stability | No combinational loops (compile error) — evaluation order is a static DAG computed once | Single-pass evaluation per clock edge; no convergence loop |
| X/Z propagation — unknown and high-impedance states require 4-valued logic | No undriven signals (single-driver rule) — all signals have defined values at all times | 2-valued logic throughout; eliminates 4-state simulation kernel |
| Multiple drivers — wired-OR/AND resolution requires dynamic arbitration | Single-driver rule enforced at compile time — no multi-driver resolution at runtime | Every signal has exactly one update site; no resolution function needed |
| Non-deterministic always block ordering — simulator-defined evaluation order | Topological evaluation order computed at compile time from the dataflow DAG | Deterministic simulation; identical results on every run and every platform |
| Implicit latches — partial assignments create state-holding elements | No implicit latches (compile error) — every register is explicitly declared | Register set is fully enumerated; allocation is static |
| Dynamic clock generation — clocks can be any signal | All clocks are Clock<D> typed — domain membership verified at compile time | Clock schedule is fully static; multi-clock interleaving is pre-computed |

◈  These are not runtime workarounds — they are language-level invariants.  The compiler proves them once during type-checking.  The simulation binary never needs to check them again.  This is what makes the simulation kernel fast: it is executing a pre-verified, topologically ordered dataflow graph, not interpreting an arbitrary event-driven program.

20.2  Arch Intermediate Representation (FIR)

All three Arch output targets — native simulation binary, SystemVerilog for synthesis, and SMT-LIB for formal verification — are lowered from a common intermediate representation called FIR (Arch Intermediate Representation).  FIR is a typed, clock-domain-aware dataflow graph.

fir_conceptual.fir // FIR is an internal compiler format — shown here for conceptual clarity // It is not written by designers; it is generated by the compiler from Arch source. // FIR nodes: signals, registers, combinational expressions, clock edges fir_module SystolicPE // Signals — typed, domain-tagged sig a_in: SInt<8> domain SysDomain; sig b_in: SInt<8> domain SysDomain; sig sum_in: SInt<32> domain SysDomain; sig sum_out: SInt<32> domain SysDomain; // Register state — initial value + update expression + trigger reg acc_reg: SInt<32> init = 0; clock = SysDomain.clk rising; reset = SysDomain.rst high → 0; update = sum_in + sext<32>(a_in) * sext<32>(b_in); end reg // Combinational graph — topological evaluation order pre-assigned // Order 0: inputs (no dependencies) // Order 1: depends only on inputs // Order 2: depends on order-1 nodes // ... comb sum_out = acc_reg; // order 0 — from register // Assertions — evaluated after comb settle assert no_overflow expr = acc_reg >= SInt<32>.min and acc_reg <= SInt<32>.max; end assert end fir_module SystolicPE

From FIR, the three backends diverge:

| Backend | arch Command | AIR Lowering Strategy | Output |
| --- | --- | --- | --- |
| Native simulation | arch sim | FIR nodes → LLVM IR → native machine code (x86-64 / ARM64 / RISC-V) | Self-contained simulation binary |
| Synthesis | arch build | FIR nodes → SystemVerilog always_ff / always_comb / assign | Synthesisable SV for FPGA/ASIC tools |
| Formal | arch formal | FIR nodes → SMT-LIB2 transition relation | Constraint file for Z3 / Bitwuzla / JaspserGold |

20.3  The Simulation Execution Model

The native simulation binary implements a cycle-accurate two-phase execution loop.  The two phases per clock edge are identical to hardware behaviour: evaluate combinational logic, then commit register updates.  This ordering is statically determined from FIR at compile time — the binary contains no dynamic scheduling logic.

sim_kernel_pseudocode // Pseudocode of the generated simulation kernel — illustrative only // Phase 1: evaluate all combinational nodes in pre-computed topological order // The order is a compile-time constant array — no dynamic scheduling. fn eval_comb(state: &mut SimState) { // Each line is generated from one Arch comb expression. // The compiler unrolls the topological order into straight-line code. state.dot_product = vec_dot(state.q, state.k); // order 0 state.scaled = (state.dot_product * RECIP_SQRT) >> 8; // order 1 state.softmax_out = piecewise_exp(state.scaled); // order 2 // ... } // Phase 2: commit all register updates (non-blocking semantics) // All reg blocks evaluated against pre-phase-1 state, committed simultaneously. fn commit_regs(state: &mut SimState, next: &SimState) { state.acc_reg = next.acc_reg; state.pc = next.pc; // ... } // Main simulation loop fn simulate(cycles: u64) { let mut state = SimState::init(); // all registers at declared init values for cycle in 0..cycles { let next = compute_next_state(&state); // all reg update expressions eval_comb(&mut state); // combinational settle commit_regs(&mut state, &next); // register commit eval_assertions(&state, cycle); // assert/cover evaluation if waveform_enabled { record_wave(&state, cycle); } } }

⚑  The two-phase model is identical to how FPGA place-and-route tools model setup/hold timing.  Phase 1 (combinational settle) determines the data that arrives at register inputs.  Phase 2 (commit) captures that data on the clock edge.  This is not an approximation — it is the exact semantics of synchronous digital hardware.

20.4  Multi-Clock Scheduling

Designs with multiple clock domains are handled by a compile-time-generated multi-clock scheduler.  The scheduler interleaves clock edges from different domains at their declared frequencies and ensures crossing signals are sampled at the correct relative timing.

multiclock_schedule.arch // Domain declarations drive the simulation scheduler domain FastDomain { freq_mhz: 500 end domain FastDomain // period: 2ns domain SlowDomain { freq_mhz: 125 end domain SlowDomain // period: 8ns // The compiler generates a static event table: // t=0ns: FastDomain rising, SlowDomain rising // t=2ns: FastDomain rising // t=4ns: FastDomain rising // t=6ns: FastDomain rising // t=8ns: FastDomain rising, SlowDomain rising // t=10ns: FastDomain rising // ... (LCM = 8ns → 4 Fast edges per 1 Slow edge) // CDC signals declared with crossing blocks are sampled // at the correct phase — the compiler inserts the two-flop // synchroniser model into the event table automatically. // Runtime: the scheduler is a pre-sorted static array of (time, domain) pairs. // No dynamic priority queue — O(1) per step.

20.5  The testbench Construct

A testbench is a first-class Arch construct that describes simulation stimulus, checking, and control.  It is not synthesisable — the compiler rejects arch build on a design containing a testbench.  It compiles only for arch sim and arch formal targets.

testbench.arch testbench AttnUnitTb // Declare the design under test dut: AttentionUnit param D_K = 64; param SEQ_LEN = 2048; end dut // Simulation parameters run_for: 10000 cycles; timeout: 50000 cycles; // fail if simulation does not finish // Declare testbench signals sig clk: Clock<SysDomain>; sig rst: Reset<Sync>; sig in_valid: Bool; sig in_token: QKVToken; sig out_ready: Bool; // Clock generation — the testbench drives clocks clock clk period 2ns; // Reset sequence — runs at simulation start init rst = high; wait 10 cycles; rst = low; wait 2 cycles; end init // Stimulus task — a reusable named sequence task send_token(q: Vec<SInt<8>,64>, k: Vec<SInt<8>,64>, v: Vec<SInt<8>,64>) in_valid = true; in_token.q = q; in_token.k = k; in_token.v = v; wait until dut.in_ready == true; wait 1 cycle; in_valid = false; end task send_token // Check task — verify output task expect_score(expected: SInt<16>, tolerance: UInt<4>) wait until dut.out_valid == true; check dut.out_score within expected ± tolerance message "Score mismatch: got {dut.out_score}, expected {expected}±{tolerance}"; out_ready = true; wait 1 cycle; out_ready = false; end task expect_score // Test body — the main stimulus sequence sequence main // Test 1: zero input — score should be 0 send_token(zeros(), zeros(), zeros()); expect_score(expected: 0, tolerance: 1); // Test 2: identity input — score equals d_k send_token(identity_vec(), identity_vec(), ones()); expect_score(expected: 64, tolerance: 2); // Test 3: stress — random inputs for 1000 cycles repeat 1000 send_token(rand_vec(), rand_vec(), rand_vec()); wait until dut.out_valid == true; out_ready = true; wait 1 cycle; out_ready = false; end repeat // Test 4: backpressure — hold out_ready low for 5 cycles send_token(rand_vec(), rand_vec(), rand_vec()); out_ready = false; wait 5 cycles; out_ready = true; wait until dut.out_valid == true; wait 1 cycle; out_ready = false; report "All tests passed."; end sequence main // Waveform output — optional; adds ~20% simulation overhead waveform: "attn_tb.fst"; // FST format for GTKWave / Surfer waveform_depth: 2; // record signals 2 levels deep into dut end testbench AttnUnitTb

20.6  Native FFI — Linking with C, Rust, and Python

The simulation binary exposes a C-compatible ABI so testbenches written in any language can drive the simulation directly.  This enables hardware-software co-simulation without an external simulator bridge like DPI or VPI.

ffi.arch // The Arch compiler generates a C header for every module marked with export_sim: module AttentionUnit export_sim: true; // generate C ABI wrapper param D_K: const = 64; // ... rest of module ... end module AttentionUnit // Generated C header: attention_unit_sim.h // // typedef struct AttentionUnit_State AttentionUnit_State; // AttentionUnit_State* attention_unit_new(); // void attention_unit_free(AttentionUnit_State*); // void attention_unit_reset(AttentionUnit_State*); // void attention_unit_step(AttentionUnit_State*); // advance one cycle // void attention_unit_set_in_valid(AttentionUnit_State*, bool); // void attention_unit_set_in_token(AttentionUnit_State*, const int8_t* q, // const int8_t* k, const int8_t* v); // bool attention_unit_get_out_valid(const AttentionUnit_State*); // int16_t attention_unit_get_out_score(const AttentionUnit_State*); // Python binding (auto-generated via ctypes wrapper): // import arch_sim // dut = arch_sim.AttentionUnit(D_K=64) // dut.reset() // dut.in_valid = True // dut.in_token_q = [1]*64 // dut.step() // print(dut.out_score) // Rust binding (auto-generated via arch-rs crate): // let mut dut = AttentionUnit::new(D_K: 64); // dut.reset(); // dut.in_valid.set(true); // dut.step(); // println!("{}", dut.out_score.get());

◈  The Python binding enables direct integration with ML training frameworks.  A PyTorch or JAX training loop can call the Arch hardware simulation on every forward pass to verify numerical equivalence between software and hardware implementations — without leaving the Python environment.

20.7  Assertion and Coverage Runtime

All assert and cover properties declared in Arch source are active in the simulation binary with no additional instrumentation step.  Failed assertions terminate the simulation with a precise error report including the cycle number, signal values, and the source location of the failing assertion.

assert_coverage_output // At simulation runtime, each assert becomes a native conditional halt: // // cycle 4721: // ASSERTION FAILED: AttentionUnit.assert score_in_range // Location: attention_unit.arch:81 // Condition: Scale.scaled >= -32767 and Scale.scaled <= 32767 // Actual: Scale.scaled = -34201 // Signal dump: // DotProduct.acc = -274008 // Scale.RECIP_SQRT = 32 // Scale.scaled = -34201 ← FAIL // // arch sim outputs a waveform snapshot of the 100 cycles preceding the failure // automatically when an assertion fires — no manual waveform setup needed. // Coverage report — generated at end of simulation run: // // COVERAGE SUMMARY: AttentionUnit // ┌─────────────────────────┬────────┬────────┬───────────┐ // │ Property │ Kind │ Count │ Status │ // ├─────────────────────────┼────────┼────────┼───────────┤ // │ score_in_range │ assert │ 10000 │ PASS │ // │ softmax_nonzero │ assert │ 10000 │ PASS │ // │ pipeline_full │ cover │ 147 │ HIT ✓ │ // │ push_when_full │ cover │ 0 │ MISS ✗ │ ← not exercised // └─────────────────────────┴────────┴────────┴───────────┘ // Coverage: 75.0% (3/4 properties hit)

◈  ✓ A missed cover property is a warning, not an error.  It tells the designer their test suite does not exercise a declared important scenario — exactly the information needed to improve test quality.

20.8  Simulation Performance Model

Because Arch's execution model compiles to a straight-line native loop with no dynamic dispatch, its simulation throughput is competitive with Verilator — currently the fastest open-source HDL simulator.  The performance advantage over event-driven simulators grows with design size.

| Simulator | Execution Model | Relative Speed | Overhead Source |
| --- | --- | --- | --- |
| VCS / Questa (interpreted) | Event-driven, 4-state logic, dynamic scheduling | 1× (baseline) | Event queue, X/Z resolution, always-block scheduler |
| VCS / Questa (compiled) | Compiled C, but still 4-state and event-driven | 3–8× | 4-state kernel, residual event overhead |
| Verilator | Cycle-accurate compiled C++, 2-state | 10–50× | C++ ABI overhead, conservative eval ordering |
| Arch native sim | Cycle-accurate compiled LLVM IR, 2-state, pre-ordered | 15–60× | Near-zero: straight-line loop, static order, no GC |
| Arch native sim (SIMD) | Vectorised LLVM IR using AVX-512 / NEON | 50–200× | Only for designs with wide Vec<> operations — AI accelerators |

⚑  SIMD vectorisation is automatic for designs that operate on Vec<SInt<8>,N> or Vec<UInt<N>,M> types — exactly the types used in systolic arrays and attention units.  The compiler maps these to native SIMD intrinsics when targeting x86-64 with AVX-512 or ARM with NEON.

20.9  arch sim Command

arch_sim_commands # Compile and run a testbench arch sim AttnUnitTb.arch # Compile only (produce binary, do not run) arch sim --compile-only --output attn_tb.bin AttnUnitTb.arch # Run with waveform output arch sim --wave attn_tb.fst AttnUnitTb.arch # Run for a fixed number of cycles arch sim --cycles 100000 AttnUnitTb.arch # Run with code coverage of Arch source arch sim --coverage AttnUnitTb.arch arch coverage report --format html # Enable SIMD vectorisation (auto-detected; explicit flag forces it) arch sim --simd avx512 AttnUnitTb.arch # Produce a C header for FFI integration arch sim --export-c --output attn_unit_sim.h AttentionUnit.arch # Produce a Python binding arch sim --export-python --output arch_sim.py AttentionUnit.arch # Produce a Rust crate arch sim --export-rust --crate-name arch-attn AttentionUnit.arch # Interactive step-by-step simulation (REPL) arch sim --interactive AttnUnitTb.arch arch> step 10 # advance 10 cycles arch> print dut.Scale.scaled arch> break when dut.out_valid == true arch> continue arch> wave dump last 50 # dump 50-cycle waveform snapshot

20.10  Comparison with Existing Simulation Flows

| Capability | Arch Native Sim | Verilator | VCS / Questa | Icarus Verilog |
| --- | --- | --- | --- | --- |
| Requires external tool install | No — self-contained binary | Yes — Verilator | Yes — commercial licence | Yes — Icarus |
| Simulation language | Arch testbench (first-class) | C++ testbench wrapper | SystemVerilog/SV-UVM | Verilog/SV |
| Python integration | Native auto-generated binding | Via verilator-python | Via VPI (slow) | Via vpipy (slow) |
| Assertion evaluation | Native — no overhead | Native | Native | Interpreted |
| Waveform output | FST (optional, default off) | VCD/FST | VCD/FSDB | VCD |
| SIMD vectorisation | Automatic for Vec<> types | Manual with DPI | No | No |
| Interactive debugger | Built-in REPL | GDB on C++ | Simulator GUI | No |
| Formal verification target | Same source → arch formal | Separate sby flow | Jasper/VC Formal | No |
| Licence cost | Free (open compiler) | Free | $10k–$500k/year | Free |

21.  Parallel Simulation — Multi-Core Native Execution

Arch supports parallel cycle-accurate simulation across all available CPU cores.  Three levels of parallelism are exploited simultaneously, each enabled by structural invariants that Arch enforces at compile time:

- DAG-level parallelism: independent nodes in the same topological level execute concurrently within a single clock cycle.

- Module-level parallelism: independent module instances with no intra-cycle data dependency execute on separate threads.

- Domain-level parallelism: separate clock domains run on dedicated threads, synchronising only at CDC crossing points and at common clock edges.

These three levels are not in conflict — the compiler stacks them.  A design with 4 clock domains and 16 modules per domain can potentially exploit all three levels simultaneously, distributing work across dozens of cores.

21.1  Why Parallel Simulation is Hard for SystemVerilog — and Easy for Arch

| Problem | SystemVerilog | Arch |
| --- | --- | --- |
| Dependency graph | Dynamic — known only at runtime; any always block can read/write any signal | Static — full DAG pre-computed at compile time from single-driver rule |
| Write conflicts | Any signal can have multiple always block writers — requires locking | Single-driver rule: exactly one writer per signal — zero locking needed |
| Evaluation ordering | Non-deterministic across always blocks — simulator-defined | Topological order fixed at compile time — deterministic on all thread counts |
| Delta cycles | Each delta may create new events — feedback requires convergence detection | No combinational loops — DAG is acyclic — single pass per level |
| Clock domain boundaries | Implicit — clocks are just signals; no declared partition points | Explicit — every Clock<D> declaration is a partition candidate |
| CDC synchronisation points | Unknown until runtime — requires event queue | Statically known from domain freq_mhz and crossing block declarations |
| Determinism guarantee | Only with +define+SIM_DETERMINISTIC and restricted coding style | Structural guarantee — identical results regardless of thread count |

21.2  Level 1 — DAG-Level (Intra-Cycle) Parallelism

Within a single clock cycle, the combinational evaluation DAG is partitioned into levels.  All nodes at the same level have no dependencies on each other — they read only from nodes at lower levels or from register state.  Nodes at the same level can execute in parallel on separate cores with no synchronisation.

dag_levels_comment // Example: attention unit combinational cone // The compiler assigns each node a DAG level at compile time. // Nodes at the same level are fully independent. // Level 0 — reads only from registers or ports (no comb dependency) // pe[0].a_in, pe[1].a_in, pe[2].a_in, pe[3].a_in // pe[0].b_in, pe[1].b_in, pe[2].b_in, pe[3].b_in // q[0..63], k[0..63] (from RAM read, registered) // Level 1 — depends only on level-0 nodes // pe[0].product = pe[0].a_in * pe[0].b_in ← parallel with pe[1..3] // pe[1].product = pe[1].a_in * pe[1].b_in // pe[2].product = pe[2].a_in * pe[2].b_in // pe[3].product = pe[3].a_in * pe[3].b_in // dot_partial[0] = q[0] * k[0] ← parallel with above // dot_partial[1] = q[1] * k[1] // ... (64 independent multiplies at level 1) // Level 2 — depends on level-1 nodes // pe[0].sum_out = pe[0].sum_in + pe[0].product ← chain dependency // dot_acc = reduce_add(dot_partial[0..63]) ← tree reduction // Level 3 // pe[1].sum_out = pe[1].sum_in + pe[1].product (pe[1].sum_in = pe[0].sum_out) // scaled = dot_acc * RECIP_SQRT >> 8 // The generated parallel kernel dispatches each level to a thread pool. // Threads at level N+1 start only after all level-N threads complete. // This is a barrier per DAG level — typically 3–8 barriers per cycle.

◈  The number of DAG levels (and therefore the number of barriers per cycle) is printed by arch sim --stats.  Flat designs with wide parallel datapaths (like systolic arrays) typically have 4–6 levels.  Deeply chained designs may have more but each level has fewer nodes.

21.3  Level 2 — Module-Level Parallelism

Module instances that are independent within a cycle — no signal flows from one to the other within that cycle — are assigned to separate worker threads.  The compiler computes the inter-module dependency graph at elaboration time and partitions modules into independent groups.

module_parallel.arch // A top-level AI accelerator with independent processing lanes module AcceleratorTop param NUM_LANES: const = 8; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // 8 independent attention heads — no intra-cycle data flow between them // The compiler identifies these as independent and assigns each to a thread. generate for i in 0..NUM_LANES-1 inst head[i]: AttentionUnit param D_K = 64; param SEQ_LEN = 2048; connect clk <- clk; connect rst <- rst; connect in_valid <- head_in_valid[i]; connect in_token <- head_in_token[i]; connect out_ready <- head_out_ready[i]; connect out_score -> head_out_score[i]; end inst head[i] end generate for i // Aggregator — depends on all head outputs // Assigned to its own thread; starts after all head[i] threads complete inst agg: ScoreAggregator param N = NUM_LANES; generate for i in 0..NUM_LANES-1 connect score_in[i] <- head_out_score[i]; end generate for i end inst agg end module AcceleratorTop // Compiler partitions the module graph: // // Thread 0: head[0] ─┐ // Thread 1: head[1] ─┤ // Thread 2: head[2] ─┤ barrier → Thread 0: agg // Thread 3: head[3] ─┤ // ... ─┤ // Thread 7: head[7] ─┘ // // All 8 head instances run in parallel. // agg runs after all heads complete — data dependency enforced by barrier.

21.4  Level 3 — Domain-Level Parallelism

Different clock domains run on dedicated threads.  Each domain thread advances through time at its own pace.  Threads synchronise only when they reach a common time point (e.g. both have a rising edge at t=8ns) or when a CDC crossing is about to transfer data.  This is the coarsest and most efficient form of parallelism — domain threads may run thousands of cycles before needing to synchronise.

domain_parallel.arch // Multi-domain AI system: compute core + memory interface + PCIe link domain ComputeDomain { freq_mhz: 500 end domain ComputeDomain // 2ns period domain HbmDomain { freq_mhz: 300 end domain HbmDomain // 3.33ns period domain PcieDomain { freq_mhz: 250 end domain PcieDomain // 4ns period // LCM(2, 3.33, 4) ≈ 20ns — threads synchronise every 20ns of sim time. // ComputeDomain runs 10 cycles, HbmDomain runs 6 cycles, PcieDomain runs 5 cycles // before the next common synchronisation point. // Between synchronisation points, each domain thread runs independently: // // Core 0: ComputeDomain thread // t=0: cycle 1 // t=2: cycle 2 // t=4: cycle 3 ← CDC transfer to HbmDomain scheduled at t=4 // t=4: [post CDC data to shared buffer; HbmDomain thread reads it at t=6.67] // t=6: cycle 4 // ... // // Core 1: HbmDomain thread // t=0: cycle 1 // t=3.33: cycle 2 // t=6.67: cycle 3 ← reads CDC data posted by ComputeDomain at t=4 // ... // // Core 2: PcieDomain thread // t=0: cycle 1 // t=4: cycle 2 // t=8: cycle 3 // ... // CDC crossing — the declared crossing block defines the synchronisation protocol crossing compute_to_hbm from: ComputeDomain, to: HbmDomain, sync: two_flop, data: compute_req -> hbm_req, end crossing compute_to_hbm // The compiler generates a lock-free SPSC queue between the two domain threads. // ComputeDomain pushes; HbmDomain polls at its own clock rate. // No global lock needed — producer and consumer are on separate cores.

⚑  The synchronisation point schedule — when domain threads must rendezvous — is pre-computed from the domain frequency declarations at compile time.  It is a static sorted array, not a dynamic event queue.  No global simulation clock exists at runtime; each thread maintains its own local time and advances until the next rendezvous.

21.5  The Parallel Simulation Scheduler

The runtime parallel scheduler has three components, all generated at compile time with no dynamic allocation:

| Component | Implementation | Role |
| --- | --- | --- |
| Thread pool | Fixed-size OS thread pool, one thread per physical core (configurable) | Executes DAG-level and module-level work items |
| DAG barrier | Cache-line-padded atomic counter per DAG level | Enforces level ordering within a cycle; threads decrement on completion |
| Domain rendezvous | Static rendezvous table: (sim_time, domain_set) pairs | Defines when domain threads must synchronise; computed from LCM of all freq_mhz |
| CDC channel | Lock-free single-producer single-consumer ring buffer per crossing | Transfers data between domain threads without a global lock |
| Work queue | Per-thread work-stealing deque | Balances load across cores when module partitions are uneven |

21.6  Enabling Parallel Simulation

parallel_commands # Auto-detect core count and use all available cores arch sim --parallel AttnUnitTb.arch # Explicit core count arch sim --parallel --cores 16 AttnUnitTb.arch # Domain-level parallelism only (safest; recommended for first run) arch sim --parallel --strategy domain AttnUnitTb.arch # All three levels (maximum throughput) arch sim --parallel --strategy all AttnUnitTb.arch # Show parallelism analysis — partition report before running arch sim --parallel --report-partition AttnUnitTb.arch # Pin domain threads to specific cores (NUMA-aware) arch sim --parallel --pin ComputeDomain:0,HbmDomain:1,PcieDomain:2 AttnUnitTb.arch # Profile parallel efficiency (prints per-core utilisation and barrier wait times) arch sim --parallel --profile AttnUnitTb.arch # Output example: # Core 0 (ComputeDomain): 94.2% active, 5.8% waiting at barriers # Core 1 (HbmDomain): 87.1% active, 12.9% waiting at barriers # Core 2 (PcieDomain): 61.4% active, 38.6% waiting at barriers # Core 3–7 (module pool): 91.3% active avg # Parallel efficiency: 88.6% (ideal: 100%) # Bottleneck: PcieDomain — consider splitting into 2 subdomains

21.7  Partition Report

The --report-partition flag prints the full parallel decomposition before running, so the designer can understand and tune the partition before committing to a long simulation run.

partition_report $ arch sim --parallel --report-partition AcceleratorTop.arch PARALLEL SIMULATION PARTITION REPORT ====================================== Clock Domains (domain-level threads): ComputeDomain 500 MHz Core 0 HbmDomain 300 MHz Core 1 PcieDomain 250 MHz Core 2 Rendezvous period: 20 ns (10/6/5 cycles respectively) Module Partitions (module-level threads within ComputeDomain): Group A (parallel): head[0], head[1], head[2], head[3] head[4], head[5], head[6], head[7] → Assigned to Core 0 thread pool (8 workers) Group B (sequential after A): agg → Assigned to Core 0 after barrier DAG Levels (intra-cycle, within each module): AttentionUnit: 6 levels Level 0: 32 nodes (register reads, port inputs) Level 1: 64 nodes (element-wise multiply) ← SIMD-vectorised Level 2: 8 nodes (partial reduce) Level 3: 4 nodes (full reduce, scale) Level 4: 2 nodes (softmax, value-weight) Level 5: 1 node (output mux) CDC Channels: compute_to_hbm: SPSC ring, depth 4 Core 0 → Core 1 hbm_to_pcie: SPSC ring, depth 2 Core 1 → Core 2 Estimated Parallel Efficiency: Domain-level: ~88% (PcieDomain underloaded) Module-level: ~95% (8 balanced head instances) DAG-level: ~82% (6 levels → 5 barriers per cycle) Combined: ~70% on 11 cores (ideal speedup: 7.7×) Recommendation: PcieDomain has low utilisation. If PCIe logic can be partitioned, consider splitting into PcieRxDomain and PcieTxDomain to improve domain-level balance.

21.8  Determinism Guarantee

Parallel Arch simulation is guaranteed to produce bit-identical results to sequential simulation, regardless of the number of cores, OS thread scheduling, or hardware memory ordering.  This guarantee rests on three structural properties:

| Property | Guarantee | Mechanism |
| --- | --- | --- |
| No write conflicts | Two threads never write the same signal in the same cycle | Single-driver rule enforced at compile time — provably no shared write sites |
| Read-after-write order | A thread reading a signal always reads the value from the previous cycle, not from a concurrent write in the same cycle | Register commit barrier: all next-state values computed before any state update; committed atomically after all threads complete their comb evaluation |
| Evaluation order | The result of every combinational expression is identical regardless of which core evaluates it | DAG level barriers ensure level N+1 never starts before level N completes on all threads; no speculative reads across levels |
| CDC channel ordering | Data crosses domain boundaries in the correct cycle-relative order | SPSC ring buffers are sequentially consistent by construction; no reordering possible |

◈  ✓ Determinism is a theorem, not a best-effort property.  A simulation that passes on 1 core will produce the same assertion results on 64 cores.  This is not true of Verilog parallel simulation, where some tools require +define+NONDETERMINISM_WARNINGS to catch ordering bugs.

21.9  Performance Scaling

Achievable speedup depends on the design's inherent parallelism — the ratio of independent work to synchronisation overhead.  Designs well-suited to Arch parallel simulation share a common structure: multiple independent processing lanes (attention heads, systolic PE rows, network ports) within each clock domain.

| Design Type | Parallelism Profile | Expected Speedup (8 cores) | Expected Speedup (32 cores) |
| --- | --- | --- | --- |
| Single-domain, deeply chained pipeline | Low — chain creates DAG depth with few wide levels | 1.5–2× | 2–3× |
| Single-domain, wide parallel array (e.g. systolic) | Medium — many independent PEs at same DAG level | 3–5× | 5–8× |
| Multi-domain with balanced loads | High — domain threads run independently most of the time | 5–7× | 10–18× |
| Multi-domain + wide arrays per domain | Very high — all three parallelism levels active | 6–8× | 15–25× |
| AI accelerator (attention + systolic + HBM) | Very high — matches Arch's design target | 7–9× | 18–30× |

⚑  Speedup beyond the number of clock domains is achieved by module-level and DAG-level parallelism within each domain thread.  A 4-domain design on 32 cores can still achieve 20× speedup if each domain has sufficient internal parallelism.

21.10  Parallel Simulation vs External Tools

|  | Arch parallel sim | Verilator --threads | VCS --parallel | Cadence Incisive parallel |
| --- | --- | --- | --- | --- |
| Determinism guarantee | Structural — always identical | Best-effort — non-det warnings exist | Tool-specific flags required | Tool-specific flags required |
| Domain-aware partitioning | Automatic from Clock<D> decls | Manual pragma annotations | Manual partition file | Manual partition file |
| Lock-free CDC channels | Yes — SPSC ring per crossing | No — shared memory with locks | No | No |
| Partition report | arch sim --report-partition | No built-in report | Separate analysis step | Separate analysis step |
| Work stealing | Yes — per-thread deque | Yes | Yes | Yes |
| SIMD + parallel combined | Yes — stacked automatically | Manual — separate --simd flags | No | No |
| Speedup on AI designs | 18–30× (32 cores) | 5–12× (32 cores) | 8–15× (32 cores) | 8–15× (32 cores) |

22.  Transaction Level Modeling (TLM)

Arch supports Transaction Level Modeling (TLM) as a first-class abstraction layer above RTL.  At the TLM level, modules communicate by calling methods on typed interfaces rather than by driving individual signals cycle by cycle.  A memory read is membus.read(addr) → data — one call, one response — rather than a sequence of valid/ready handshake cycles.  TLM enables fast architectural simulation, software/hardware co-simulation, and performance modeling, all from the same Arch source as the RTL implementation.

22.1  Abstraction Levels in Arch

| Level | Communication Model | Time Model | Simulation Speed | Use When |
| --- | --- | --- | --- | --- |
| RTL | Signal-level: valid/ready handshakes, clock edges | Cycle-accurate | Baseline (1×) | Verification, timing closure, formal |
| Approximately-timed TLM (AT) | Method calls with declared timing annotations | Nanoseconds — one call may advance sim time by N ns | 10–50× | Performance modeling, latency analysis, power estimation |
| Loosely-timed TLM (LT) | Method calls, timing ignored for speed | Zero-time or coarse quantum | 100–1000× | SW/HW co-simulation, firmware development, architectural exploration |

22.2  Transaction Concurrency — Blocking, Pipelined, and Out-of-Order

The most important design decision for a TLM method interface is whether calls block the caller until the response arrives, or whether multiple calls can be in-flight simultaneously.  Arch makes this explicit with four method concurrency modes.  Getting this right is the difference between a TLM model that accurately represents AXI pipelining and one that degenerates into a serial bus that no real hardware resembles.

22.2.1  blocking — Serial, Caller Suspends

blocking_method.arch // The caller suspends until the response is received. // Next call cannot issue until this one completes. // Correct for: APB, AHB single-transfer, simple memory-mapped registers. methods blocking method read(addr: UInt<32>) -> UInt<64> timing: 4 cycles; end method read end methods // In a sequence body — call 2 cannot start until call 1 returns. // Total time: 4 + 4 = 8 cycles. Serial. sequence read_two let d0: UInt<64> = mem.read(0x1000); // suspends 4 cycles let d1: UInt<64> = mem.read(0x2000); // suspends 4 more cycles end sequence read_two

22.2.2  pipelined — Multiple Outstanding, In-Order Responses

pipelined_method.arch // The caller gets a Future<T> immediately and can issue more calls. // Responses arrive in the same order as requests. // Correct for: AXI without out-of-order IDs, read-after-read pipelining. methods pipelined method read(addr: UInt<32>) -> Future<UInt<64>> timing: 4 cycles; // latency from issue to response max_outstanding: 8; // AXI AR channel depth end method read end methods // In a sequence body — all three calls issue immediately (back-to-back), // then the caller awaits each response in order. // Total time: 4 cycles (all overlap). Pipelined. sequence read_three_pipelined let f0: Future<UInt<64>> = mem.read(0x1000); // issues immediately let f1: Future<UInt<64>> = mem.read(0x2000); // issues immediately let f2: Future<UInt<64>> = mem.read(0x3000); // issues immediately // Await responses in order let d0: UInt<64> = await f0; // waits up to 4 cycles let d1: UInt<64> = await f1; // already done if pipelined correctly let d2: UInt<64> = await f2; end sequence read_three_pipelined // If more than max_outstanding calls are issued before any response is received, // the (max_outstanding+1)th call blocks until one slot frees — modelling // the AXI AR channel handshake stall.

◈  Future<T> is a first-class Arch TLM type.  It is not a general-purpose async primitive — it only appears in TLM method return positions and sequence await expressions.  The compiler tracks Future lifetimes and emits an error if a Future is awaited after the simulation window in which it was issued.

22.2.3  out_of_order — Multiple Outstanding, Any-Order Responses

ooo_method.arch // The caller gets a Token<T, ID> immediately. // Responses may arrive in any order; matched to the original call by ID. // Correct for: full AXI with multiple IDs, out-of-order memory systems. methods out_of_order method read( addr: UInt<32> ) -> Token<UInt<64>, id_width: 4> // 4-bit AXI ID field timing: 4..20 cycles; // range: best-case to worst-case latency max_outstanding: 16; end method read end methods // In a sequence body — issue multiple reads, await in any order. // Responses may return in a different order than requests. sequence read_ooo let t0: Token<UInt<64>, 4> = mem.read(0x1000); // ID assigned by interface let t1: Token<UInt<64>, 4> = mem.read(0x2000); let t2: Token<UInt<64>, 4> = mem.read(0x3000); // await_any — return whichever token completes first match await_any(t0, t1, t2) t0 => let d0: UInt<64> = t0.value; // t0 finished first t1 => let d1: UInt<64> = t1.value; t2 => let d2: UInt<64> = t2.value; end match // await_all — wait for all outstanding tokens let (d0, d1, d2) = await_all(t0, t1, t2); end sequence read_ooo // Token<T, ID> carries the assigned AXI ID automatically. // The transactor extracts the ID field from the AXI R channel response // and routes it back to the correct Token — no designer involvement.

22.2.4  burst — One Call, N Responses

burst_method.arch // A burst method models AXI INCR bursts natively. // One call issues one AR transaction; N data beats return as a Vec. // This is more accurate than N separate pipelined reads — it models // the real AXI burst mechanism: one address, N sequential data beats. methods burst method read_burst( addr: UInt<32>, length: UInt<8> // AXI ARLEN: 0=1 beat, 255=256 beats ) -> Future<Vec<UInt<64>, length>> timing: 4 + length cycles; // address latency + one beat per cycle max_outstanding: 4; // max simultaneous burst transactions end method read_burst burst method write_burst( addr: UInt<32>, data: Vec<UInt<64>, length>, strobe: Vec<UInt<8>, length> ) -> Future<WriteResp> timing: 1 + length cycles; max_outstanding: 4; end method write_burst end methods // Burst call: one AR transaction, 16 data beats returned as Vec<UInt<64>,16> sequence dma_burst_read let f: Future<Vec<UInt<64>, 16>> = mem.read_burst(0x1000, length: 15); // Issue more work here while burst is in-flight... let beats: Vec<UInt<64>, 16> = await f; // beats[0] is the first 64-bit beat, beats[15] is the last end sequence dma_burst_read // Compare: 16 separate pipelined reads vs 1 burst read // Pipelined: 16 AR channel transactions — bandwidth-inefficient, not accurate // Burst: 1 AR channel transaction — matches real AXI behaviour

⚑  The length parameter in a burst method declaration is a special form — it is an implicit UInt<8> input parameter that also parameterises the return type Vec<T, length>.  The compiler infers the Vec length from the call site.  This is the only case in Arch where a method parameter influences the return type.

22.3  Concurrency Mode Comparison

| Mode | Return Type | Caller Blocks? | Multiple Outstanding? | Response Order | Models |
| --- | --- | --- | --- | --- | --- |
| blocking | T directly | Yes — until response | No — one at a time | N/A | APB, AHB, simple MMIO |
| pipelined | Future<T> | No — issues immediately | Yes — up to max_outstanding | In-order | AXI (no OOO), AMBA AHB burst |
| out_of_order | Token<T, ID> | No — issues immediately | Yes — up to max_outstanding | Any order | Full AXI with multiple IDs, DDR |
| burst | Future<Vec<T, length>> | No — issues immediately | Yes — up to max_outstanding | In-order beats | AXI INCR burst, HBM row access |

22.4  Full AXI4 Interface Declaration

Using all four concurrency modes, the AXI4 read and write channels can be modeled with full fidelity — including ID-based out-of-order completion, byte strobes, burst types, and response codes.

axi4_interface.arch interface Axi4 param ADDR_W: const = 32; param DATA_W: const = 64; param ID_W: const = 4; // ── RTL signals (present at RTL level) ──────────────────────── // AW channel port awvalid: out Bool; port awready: in Bool; port awaddr: out UInt<ADDR_W>; port awid: out UInt<ID_W>; port awlen: out UInt<8>; port awsize: out UInt<3>; port awburst: out UInt<2>; // W channel port wvalid: out Bool; port wready: in Bool; port wdata: out UInt<DATA_W>; port wstrb: out UInt<DATA_W/8>; port wlast: out Bool; // B channel port bvalid: in Bool; port bready: out Bool; port bresp: in UInt<2>; port bid: in UInt<ID_W>; // AR channel port arvalid: out Bool; port arready: in Bool; port araddr: out UInt<ADDR_W>; port arid: out UInt<ID_W>; port arlen: out UInt<8>; port arsize: out UInt<3>; port arburst: out UInt<2>; // R channel port rvalid: in Bool; port rready: out Bool; port rdata: in UInt<DATA_W>; port rresp: in UInt<2>; port rid: in UInt<ID_W>; port rlast: in Bool; // ── TLM methods (present at TLM level) ──────────────────────── methods // Single read — pipelined, in-order, up to 16 outstanding pipelined method read( addr: UInt<ADDR_W> ) -> Future<UInt<DATA_W>> timing: 4 cycles; max_outstanding: 16; end method read // Burst read — one AR transaction, up to 256 data beats burst method read_burst( addr: UInt<ADDR_W>, length: UInt<8> ) -> Future<Vec<UInt<DATA_W>, length>> timing: 4 + length cycles; max_outstanding: 4; end method read_burst // Out-of-order read — ID-tagged, responses may reorder out_of_order method read_ooo( addr: UInt<ADDR_W> ) -> Token<UInt<DATA_W>, id_width: ID_W> timing: 4..20 cycles; max_outstanding: 16; end method read_ooo // Single write — pipelined, in-order pipelined method write( addr: UInt<ADDR_W>, data: UInt<DATA_W>, strobe: UInt<DATA_W/8> ) -> Future<AxiResp> timing: 2 cycles; max_outstanding: 16; end method write // Burst write — one AW transaction, N data beats burst method write_burst( addr: UInt<ADDR_W>, data: Vec<UInt<DATA_W>, length>, strobe: Vec<UInt<DATA_W/8>, length>, length: UInt<8> ) -> Future<AxiResp> timing: 2 + length cycles; max_outstanding: 4; end method write_burst end methods end interface Axi4 struct AxiResp ok: Bool, resp: UInt<2>, // OKAY=0 EXOKAY=1 SLVERR=2 DECERR=3 id: UInt<4>, end struct AxiResp

22.5  Using the AXI4 Interface — Initiator Examples

dma_axi4_initiator.arch // DMA engine using all three read modes module DmaEngine socket axi: initiator Axi4<ADDR_W 32, DATA_W 64, ID_W 4>; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // ── Sequential read — blocking, one at a time ────────────────── sequence config_read // Read 4 config registers — serial, total latency: 16 cycles let ctrl: UInt<64> = await axi.read(0xF000_0000); let stat: UInt<64> = await axi.read(0xF000_0008); let base: UInt<64> = await axi.read(0xF000_0010); let limit: UInt<64> = await axi.read(0xF000_0018); // Note: using await directly on a pipelined method forces serial behaviour. // For maximum throughput, issue all futures first (see below). end sequence config_read // ── Pipelined read — all 4 issued immediately, overlap ───────── sequence config_read_fast // All 4 issue in the same cycle — total latency: ~4 cycles not 16 let f_ctrl = axi.read(0xF000_0000); let f_stat = axi.read(0xF000_0008); let f_base = axi.read(0xF000_0010); let f_limit = axi.read(0xF000_0018); let (ctrl, stat, base, limit) = await_all(f_ctrl, f_stat, f_base, f_limit); end sequence config_read_fast // ── Burst read — 64-word cache line fill ─────────────────────── sequence cache_line_fill(addr: UInt<32>) // One AR transaction, 64 data beats returned as Vec let f = axi.read_burst(addr, length: 63); // Issue next transaction or do other work here... let line: Vec<UInt<64>, 64> = await f; // Process line... end sequence cache_line_fill // ── Out-of-order — two reads, process whichever returns first ── sequence speculative_prefetch(addr_a: UInt<32>, addr_b: UInt<32>) let t_a = axi.read_ooo(addr_a); let t_b = axi.read_ooo(addr_b); match await_any(t_a, t_b) t_a => process_a(t_a.value); process_b(await t_b); t_b => process_b(t_b.value); process_a(await t_a); end match end sequence speculative_prefetch end module DmaEngine

22.6  TLM Interface Declaration — Method Calls

A Arch interface gains a methods block alongside its port declarations.  The interface serves both roles simultaneously: RTL ports are driven when operating at the RTL abstraction level; TLM methods are called when operating at the TLM level.  The arch sim flag selects which.

interface_tlm.arch // Annotated interface anatomy interface MemBusTlm param ADDR_W: const = 32; param DATA_W: const = 64; // RTL signals ─ synthesisable, used at RTL level port clk: in Clock<SysDomain>; port valid: out Bool; port ready: in Bool; port addr: out UInt<ADDR_W>; port rdata: in UInt<DATA_W>; port wdata: out UInt<DATA_W>; port wen: out Bool; // TLM methods ─ simulation-only, used at TLM level methods blocking method read( addr: UInt<ADDR_W> ) -> UInt<DATA_W> timing: 10 ns; end method read pipelined method read_pipe( addr: UInt<ADDR_W> ) -> Future<UInt<DATA_W>> timing: 10 ns; max_outstanding: 8; end method read_pipe non_blocking method write( addr: UInt<ADDR_W>, data: UInt<DATA_W>, strobe: UInt<DATA_W/8> ) -> Future<WriteResp> timing: 10 ns; max_outstanding: 8; end method write end methods end interface MemBusTlm

22.7  Timing Models — LT and AT

timing_models.arch // Loosely-Timed (LT) — zero timing, maximum speed // arch sim --tlm-lt // All method calls complete in zero simulation time. // Use for: firmware bring-up, API correctness, SW development. // Approximately-Timed (AT) — nanosecond timing // arch sim --tlm-at // Each method call advances sim time by its declared timing value. // For pipelined/OOO methods, the LATENCY elapses from issue to await. // For blocking methods, the caller is suspended for the duration. // Use for: performance analysis, latency budgeting, throughput measurement. // Quantum — LT with periodic coarse time advance // arch sim --tlm-quantum 100ns // Sim time advances in 100ns steps regardless of method calls. // Balances speed and temporal accuracy for mixed SW/HW workloads. // Per-method override — selectively restore timing in LT mode methods blocking method dram_read(addr: UInt<32>) -> UInt<64> timing: 40 ns; timing_in_lt: true; // honour this latency even in LT mode end method dram_read // useful for DRAM latency-sensitive tests end methods

22.8  Socket Binding and Automatic Transactor

binding.arch // One-to-one binding bind dma.axi -> dram.axi; // N-to-1: compiler inserts round-robin TLM arbiter generate for i in 0..NUM_CORES-1 bind core[i].axi -> dram.axi arbiter: round_robin; end bind end generate for i // 1-to-N: compiler inserts address decoder bind cpu.axi -> dram.axi when addr in 0x0000_0000..0x0FFF_FFFF; bind cpu.axi -> uart.axi when addr in 0x1000_0000..0x1000_FFFF; // RTL ↔ TLM transactor: RTL DMA → TLM DRAM bind dma.mem_rtl_port -> dram.axi transactor: protocol Axi4Lite; end bind // Built-in transactor protocols: AXI4, AXI4Lite, AXI4Stream, // AHB, APB, TileLink, Wishbone

22.9  TLM Testbench and Python Co-Simulation

tlm_testbench.arch testbench SocTb dut: SocFabric end dut run_for: 1 ms; socket tb_master: initiator Axi4<ADDR_W 32, DATA_W 64, ID_W 4>; bind tb_master -> dut.cpu.axi; task init_memory(base: UInt<32>, data: Vec<UInt<64>, 256>) // Issue all 256 writes as pipelined futures — all in-flight simultaneously let futures: Vec<Future<AxiResp>, 256>; repeat 256 with idx futures[idx] = tb_master.write(base + idx*8, data[idx], 8'hFF); end repeat await_all(futures); // wait for all write responses end task init_memory sequence main init_memory(base: 0x0000_0000, data: boot_rom_image); // Burst read back — one AXI transaction, 256 beats let f = tb_master.read_burst(0x0000_0000, length: 255); let readback: Vec<UInt<64>, 256> = await f; repeat 256 with i check readback[i] == boot_rom_image[i] message "Mismatch at word {i}"; end repeat report "Memory verified."; end sequence main end testbench SocTb // ── Python co-simulation (LT mode) ─────────────────────────────────── // import arch_sim // dut = arch_sim.AcceleratorTop(tlm_mode='lt') // dut.reset() // # Pipelined write: 1024 writes issued immediately, futures collected // futures = [dut.axi.write(WEIGHT_BASE + i*8, int(w)) for i,w in enumerate(weights)] // dut.await_all(futures) // # Burst read: one call, 64 words returned // output = dut.axi.read_burst(OUTPUT_BASE, length=63) // assert list(output) == torch_reference.tolist()

22.10  TLM vs RTL — When to Use Which

| Use Case | TLM Level | RTL | Reason |
| --- | --- | --- | --- |
| Firmware development, driver bring-up | LT — 1000× faster | Not needed | SW runs before HW is complete |
| Memory subsystem performance modeling | AT — timing annotation | Optional | Latency budgeting needs ns accuracy |
| AXI burst bandwidth measurement | AT with burst method | Optional | Burst method models AR/R channels accurately |
| Out-of-order reorder buffer verification | OOO method + AT | Also useful | Token<ID> models AXI ID routing accurately |
| Formal verification of protocol correctness | RTL | Required | Formal tools operate on signals not methods |
| Gate-level timing verification | RTL | Required | Post-synthesis netlist is always RTL |
| ML framework co-simulation (PyTorch/JAX) | LT | Optional | Python loops; cycle accuracy irrelevant |
| NoC throughput analysis | AT with OOO methods | Optional | Throughput and latency need timing |

22.11  Comparison with SystemC TLM-2.0

| Feature | Arch TLM | SystemC TLM-2.0 |
| --- | --- | --- |
| Concurrency modes | blocking, pipelined, out_of_order, burst — explicit per method | blocking transport only; non-blocking transport manual phase machine |
| Pipelined calls | Future<T> return — issue all, await later | Manual — must use nb_transport + phase machine + event notifications |
| Out-of-order calls | Token<T,ID> return — await_any / await_all | Manual — custom ID tracking + payload extension |
| Burst method | Native: one call → Vec<T,N> response | Not native — must model as N separate transactions or use custom extension |
| Method call syntax | axi.read(addr) — typed, checked at compile time | b_transport(trans, delay) — generic payload, cast at runtime |
| Phase machine | Compiler-generated from concurrency mode + timing | Hand-coded: BEGIN_REQ/END_REQ/BEGIN_RESP/END_RESP per method |
| await_any / await_all | Built-in — compiler generates wait logic | Manual — sc_event and wait() combinations |
| Automatic transactor | bind with transactor: Axi4 — compiler generates | Manual adapter: hundreds of lines per protocol |
| Type safety | Width, ID, length all compile-time checked | Runtime — wrong payload size is a simulation assertion |

23.  Cycle-Accurate TLM — RTL-Backed Method Implementations

A TLM method implementation can be backed by actual RTL signal-level logic instead of a nanosecond timing annotation.  When it is, calling the method drives real signals and waits for real clock edges — making the method call cycle-accurate.  The timing is no longer declared; it emerges from the actual RTL behavior.  The initiator code is identical at all three abstraction levels.

23.1  The Three Implementation Tiers

| Tier | Keyword | Timing Source | Speed | Accuracy | Use When |
| --- | --- | --- | --- | --- | --- |
| Loosely-timed | timing: loosely_timed | Zero — instant | ~1000× | Functional only | SW bring-up, API testing |
| Approximately-timed | timing: N ns | Declared annotation | ~50× | Approximate latency | Performance modeling, latency budget |
| RTL-backed | implement ... rtl | Actual clock cycles | 1× | Cycle-accurate | Full verification, timing closure, power |

All three tiers share the same initiator call syntax.  arch sim selects the tier at simulation time, or individual modules can be selectively refined to RTL while others remain at TLM.

23.2  RTL-Backed Method — Syntax

An implement block with an rtl body drives actual interface signals and uses wait until expressions to suspend on real clock events.  The method completes when the rtl body reaches a return statement or falls through.  The number of cycles consumed is determined by how many clock edges the body passes through — not by any declared annotation.

rtl_backed_impl.arch // AXI4 memory model — three tiers for the same read method // ── Tier 1: Loosely-timed ───────────────────────────────────────── implement axi.read(addr) -> Future<UInt<64>> timing: loosely_timed; return Storage.data[addr[27:3]]; // instant — no cycle cost end implement axi.read // ── Tier 2: Approximately-timed ─────────────────────────────────── implement axi.read(addr) -> Future<UInt<64>> timing: 40 ns; // declared annotation — approximate return Storage.data[addr[27:3]]; end implement axi.read // ── Tier 3: RTL-backed — cycle-accurate ─────────────────────────── // Drives actual AR/R channel signals. Timing emerges from clock edges. implement axi.read(addr) -> Future<UInt<64>> rtl // Drive AR channel — assert address and valid arvalid = true; araddr = addr; arid = alloc_id(); // assign next available AXI ID arlen = 0; // single beat arsize = 3; // 8 bytes arбurst = 2'b01; // INCR // Wait for AR handshake — actual cycles consumed here wait until arready == true; wait 1 cycle; arvalid = false; // Wait for R channel response matching our AXI ID wait until rvalid == true and rid == arid; let data: UInt<64> = rdata; rready = true; wait 1 cycle; rready = false; free_id(arid); return data; end rtl end implement axi.read

◈  The wait until and wait N cycle expressions inside an rtl body consume real simulation cycles.  The method's actual latency is the number of cycles between the method call and the return — determined entirely by the signal behavior of the target it is connected to, not by any annotation.

23.3  Pipelined and Out-of-Order RTL-Backed Methods

RTL-backed implementations of pipelined and out_of_order methods model the actual signal-level pipeline naturally.  The AR channel handshake and the R channel response are separated — the method issues the AR channel in the first phase and registers a pending response, then the response phase fires when the R channel data arrives.

rtl_backed_pipeline.arch // RTL-backed pipelined read — AXI AR and R channels are separated implement axi.read_pipe(addr) -> Future<UInt<64>> rtl // Phase 1: issue request on AR channel // Returns a Future immediately after AR handshake. // The Future resolves when R channel data arrives. arvalid = true; araddr = addr; arid = alloc_id(); wait until arready == true; // AR accepted — may be 0 or more cycles wait 1 cycle; arvalid = false; // Future is now pending — caller gets control back here // while we wait for R channel in the background // Phase 2: wait for R channel (runs concurrently with caller) wait until rvalid == true and rid == arid; let data: UInt<64> = rdata; rready = true; wait 1 cycle; rready = false; free_id(arid); return data; // resolves the Future — caller's await unblocks end rtl end implement axi.read_pipe // The initiator's pipelined sequence is unchanged — same code as before: sequence read_four_pipelined // All four ARs issue back-to-back — real AR channel handshakes let f0 = axi.read_pipe(0x1000); let f1 = axi.read_pipe(0x2000); let f2 = axi.read_pipe(0x3000); let f3 = axi.read_pipe(0x4000); // Await all R channel responses — actual cycle count depends on // the target's RTL implementation, not on any declared annotation let (d0, d1, d2, d3) = await_all(f0, f1, f2, f3); end sequence read_four_pipelined

⚑  The two-phase structure of the rtl body — issue phase (before first yield point) and response phase (after) — directly models the split AR/R channels of AXI.  The compiler recognises this pattern and generates the correct coroutine split: the Future is created after the AR handshake; the response phase runs as a background coroutine until the R channel fires.

23.4  Gradual Refinement — Mixed TLM and RTL in One System

The most powerful use of RTL-backed methods is gradual refinement.  A full system starts at LT TLM for maximum simulation speed.  Modules are refined to RTL-backed implementations one at a time.  The initiator code is identical at every stage — only the target implementation changes.  Modules at different refinement levels coexist in the same simulation; the automatic transactor at each socket boundary handles the level mismatch.

refinement_stages.arch // Refinement ladder — same system at three stages // ── Stage 1: Full LT system — all methods loosely-timed ─────────── // Speed: ~1000× RTL. Use for: firmware bring-up, API testing. module Stage1System inst cpu: CpuTlmModel end inst cpu // LT: instant method calls inst dram: DramLtModel end inst dram // LT: zero-cycle reads inst noc: NocLtModel end inst noc // LT: zero-latency routing bind cpu.mem -> noc.upstream; bind noc.port -> dram.mem; end module Stage1System // ── Stage 2: DRAM refined to AT — memory latency modeled ────────── // Speed: ~50× RTL. Use for: memory subsystem performance analysis. // CPU and NoC still LT. Only DRAM timing annotation added. module Stage2System inst cpu: CpuTlmModel end inst cpu // LT — unchanged inst dram: DramAtModel end inst dram // AT: 40ns read latency inst noc: NocLtModel end inst noc // LT — unchanged bind cpu.mem -> noc.upstream; bind noc.port -> dram.mem; end module Stage2System // ── Stage 3: DRAM refined to RTL-backed — cycle-accurate memory ─── // Speed: ~10× RTL (DRAM is RTL; CPU+NoC still TLM). // Use for: memory controller verification, power estimation. module Stage3System inst cpu: CpuTlmModel end inst cpu // TLM — still fast inst dram: DramRtlModel end inst dram // RTL-backed — cycle accurate inst noc: NocLtModel end inst noc // TLM — still fast bind cpu.mem -> noc.upstream; // Transactor auto-generated: TLM socket → RTL-backed target bind noc.port -> dram.mem transactor: protocol Axi4; end module Stage3System // ── Stage 4: Full RTL — all modules cycle-accurate ──────────────── // Speed: 1× RTL (parallel simulation on 32 cores: ~20× wall-clock). // Use for: final verification, formal, gate-level simulation. module Stage4System inst cpu: CpuRtlModule end inst cpu // Full RTL inst dram: DramRtlModel end inst dram // Full RTL inst noc: NocRtlModule end inst noc // Full RTL // All sockets now directly RTL — no transactors needed bind cpu.mem -> noc.upstream; bind noc.port -> dram.mem; end module Stage4System

23.5  Selective Refinement via arch sim --refine

arch_refine_commands # Run at full LT speed — all methods loosely-timed arch sim --tlm-lt SocTb.arch # Refine DRAM to AT timing — everything else stays LT arch sim --tlm-lt --refine DramModel:at SocTb.arch # Refine DRAM to RTL-backed — everything else stays LT arch sim --tlm-lt --refine DramModel:rtl SocTb.arch # Refine DRAM and NoC to RTL; CPU stays LT arch sim --tlm-lt --refine DramModel:rtl --refine NocRouter:rtl SocTb.arch # Full RTL simulation with parallel execution arch sim --rtl --parallel --cores 32 SocTb.arch # Refinement equivalence check: # Verify that the RTL-backed method produces the same responses # as the LT model for a given stimulus. Catches RTL bugs early. arch sim --check-refinement DramModel SocTb.arch # Runs both LT and RTL-backed implementations in lockstep. # Reports any cycle where the responses differ. # Output: # DramModel refinement check: 10000 transactions verified # PASS — RTL and LT responses identical on all transactions

23.6  Timing Emergence vs Timing Declaration

The distinction between declared and emergent timing is central to understanding RTL-backed methods.

| Property | AT: timing: 40 ns | RTL-backed: implement ... rtl |
| --- | --- | --- |
| Timing source | Declared by designer — fixed annotation | Emerges from actual signal behavior — varies per transaction |
| Accuracy | Approximate — same latency for all transactions | Cycle-accurate — row hits differ from row misses; bank conflicts modeled |
| Backpressure | Not modeled — max_outstanding limits concurrency | Fully modeled — arready stalls consume real cycles |
| Contention | Not modeled — all transactions get declared latency | Fully modeled — competing requests delay each other realistically |
| Power correlation | Cannot drive power estimation — no signal activity | Signal transitions drive switching activity for power estimation |
| Debugging | No waveform — method is a black box | Full waveform — every AR/R/AW/W/B signal visible in GTKWave |
| Speed | ~50× RTL simulation speed | 1× RTL (parallel on 32 cores: ~20× wall-clock) |

◈  A common pattern: develop firmware at LT (1000× speed), run performance analysis at AT (50× speed) to verify latency budgets, then run final verification at RTL-backed (1× speed) with full waveform capture.  All three use the same testbench and the same initiator code.

23.7  Refinement Equivalence Checking

The arch sim --check-refinement command runs the LT and RTL-backed implementations in lockstep and verifies that they produce the same functional responses.  This is not formal equivalence — it is transaction-level equivalence checking: for the same sequence of method calls with the same arguments, do both implementations return the same values?

equivalence_check.arch // Equivalence is checked at the method boundary — not at the signal level. // The RTL implementation may have different internal timing and signal behavior // as long as every method call returns the same value as the LT model. // What the checker verifies: // LT: data = Storage.data[addr] → returns 0x0102030405060708 // RTL: AR channel → wait arready → R channel → returns 0x0102030405060708 // PASS: both return the same value // What the checker does NOT verify: // Cycle count (RTL may take 4 cycles, LT takes 0) // Internal signal transitions (RTL drives arvalid etc., LT does not) // Side effects inside the target (both must have the same memory state) // Equivalence failure example: // LT: read(0x1008) returns 0x0000_0000_0000_0000 (zero-init) // RTL: read(0x1008) returns 0xXXXX_XXXX_XXXX_XXXX (uninitialised RAM) // FAIL — RTL implementation has different reset state than LT model // Fix: add init: zero to the RTL RAM declaration

23.8  Complete Refinement Example — DRAM Controller

dram_three_tiers.arch // Single module with all three tiers declared. // arch sim selects which implement block to use. module DramController param SIZE_MB: const = 256; param ROW_HIT_NS: const = 10; param ROW_MISS_NS: const = 40; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; socket axi: target Axi4<ADDR_W 32, DATA_W 64, ID_W 4>; // Internal storage — shared across all tiers ram Storage param DEPTH: const = SIZE_MB * 131072; port clk: in Clock<SysDomain>; kind simple_dual; read: sync; store data: Vec<UInt<64>, DEPTH>; end store port read_port { en: in Bool; addr: in UInt<24>; data: out UInt<64>; end port read_port port write_port { en: in Bool; addr: in UInt<24>; data: in UInt<64>; end port write_port init: zero; end ram Storage // Track open DRAM row per bank (4 banks) reg open_row: Vec<UInt<14>, 4> init Vec::splat(14'hFFFF); // ── LT implementation ───────────────────────────────────────── implement axi.read(addr) -> Future<UInt<64>> tier: loosely_timed; return Storage.data[addr[26:3]]; end implement axi.read // ── AT implementation — row hit/miss modeled ────────────────── implement axi.read(addr) -> Future<UInt<64>> tier: approximately_timed; let bank: UInt<2> = addr[4:3]; let row: UInt<14> = addr[26:13]; let latency: UInt<8> = open_row[bank] == row ? ROW_HIT_NS : ROW_MISS_NS; open_row[bank] = row; wait latency ns; return Storage.data[addr[26:3]]; end implement axi.read // ── RTL-backed implementation — cycle accurate ───────────────── implement axi.read(addr) -> Future<UInt<64>> tier: rtl; rtl arvalid = true; araddr = addr; arid = alloc_id(); arlen = 0; arsize = 3; arburst = 2'b01; wait until arready == true; wait 1 cycle; arvalid = false; // Row hit/miss determined by actual open_row register state // — no annotation; real cycles consumed by real logic wait until rvalid == true and rid == arid; let data: UInt<64> = rdata; rready = true; wait 1 cycle; rready = false; free_id(arid); return data; end rtl end implement axi.read end module DramController // ── Simulation commands ─────────────────────────────────────────── // arch sim --refine DramController:lt SocTb.arch // ~1000× RTL // arch sim --refine DramController:at SocTb.arch // ~50× RTL // arch sim --refine DramController:rtl SocTb.arch // 1× RTL // arch sim --check-refinement DramController SocTb.arch // verify equivalence

22.12  Cycle-Accurate TLM — RTL Signal Fidelity Behind Method Interfaces

A TLM method call can be backed by a full RTL signal-level implementation.  When this mode is active, calling axi.read(addr) executes the complete AXI AR and R channel handshake cycle by cycle — every valid, ready, and data signal toggles exactly as it would in hardware — while the caller still uses the clean method call API.  This is cycle-accurate TLM: the initiator sees an abstracted interface; the implementation has full signal fidelity.

22.12.1  Three Implementation Strategies per Method

| Strategy | Keyword | Timing Model | Signal Fidelity | Use When |
| --- | --- | --- | --- | --- |
| Functional model | implement ... timing: N cycles | AT/LT — N cycles per call | None — no signals toggled | Architectural exploration, SW bring-up, max speed |
| Cycle-accurate model | implement ... rtl_accurate | Cycle-exact — runs real FSM | Full — every signal toggles per clock | Protocol verification, interface testing, latency-sensitive models |
| Automatic from RTL ports | implement ... from_rtl | Cycle-exact — derived from port declarations | Full — compiler generates the FSM from port bundle | When the RTL port bundle fully describes the protocol |

22.12.2  rtl_accurate Implementation

An implement block marked rtl_accurate replaces the timing: N annotation with a full RTL description of the handshake.  The method call launches the RTL FSM, which drives signals cycle by cycle and completes the Future or Token when the protocol completes.  The caller's sequence body is suspended exactly as long as the real hardware would take.

rtl_accurate_impl.arch // AXI4 read method — rtl_accurate implementation // The caller writes: let f = axi.read(addr); await f; // Underneath, this FSM runs, driving every AXI AR and R signal: implement axi.read(addr: UInt<32>) -> Future<UInt<64>> rtl_accurate; // Internal state for this transaction reg ar_sent: Bool init false; reg data_out: UInt<64> init 0; // Phase 1: drive AR channel until arready asserted comb arvalid = not ar_sent; araddr = addr; arid = next_id(); // allocate next available AXI ID arlen = 0; // single beat arsize = 3'b011; // 8 bytes arburst = 2'b01; // INCR end comb reg on clk rising, rst high if rst ar_sent <= false; end if else if arvalid and arready ar_sent <= true; // AR channel handshake complete end else end reg // Phase 2: wait for R channel response comb rready = ar_sent; // assert rready once AR is accepted end comb reg on clk rising, rst high if rvalid and rready and rid == arid data_out <= rdata; complete future with data_out; // resolve the Future — caller unblocks end if end reg end implement axi.read // The compiler verifies: // 1. Every RTL port of the AXI interface is driven in exactly one implement block // or declared as driven by another implement block (e.g. write drives awvalid) // 2. The 'complete future' statement is reachable on all non-reset paths // 3. No signal driven here conflicts with another implement block in the same cycle

◈  complete future with value is the statement that resolves the pending Future and unblocks the caller's await expression.  The compiler tracks that every implement block with a non-void return type contains exactly one reachable complete statement — a missing complete is a compile error, not a simulation hang.

22.12.3  from_rtl — Compiler-Generated Cycle-Accurate Implementation

When the RTL port bundle fully expresses the protocol — standard valid/ready with named fields matching AXI, AHB, APB, or TileLink conventions — the compiler can generate the cycle-accurate implement block automatically.  The designer writes only the interface declaration; the compiler synthesises the FSM.

from_rtl_impl.arch // Interface with from_rtl directive — compiler generates the rtl_accurate FSM interface Axi4Lite param ADDR_W: const = 32; param DATA_W: const = 32; // RTL ports — standard AXI4-Lite signal names port awvalid: out Bool; port awready: in Bool; port awaddr: out UInt<ADDR_W>; port wvalid: out Bool; port wready: in Bool; port wdata: out UInt<DATA_W>; port wstrb: out UInt<DATA_W/8>; port bvalid: in Bool; port bready: out Bool; port bresp: in UInt<2>; port arvalid: out Bool; port arready: in Bool; port araddr: out UInt<ADDR_W>; port rvalid: in Bool; port rready: out Bool; port rdata: in UInt<DATA_W>; port rresp: in UInt<2>; methods pipelined method read(addr: UInt<ADDR_W>) -> Future<UInt<DATA_W>> timing: 2 cycles; max_outstanding: 1; // AXI4-Lite: no pipelining implement: from_rtl // compiler generates AXI4-Lite AR+R FSM protocol: Axi4Lite; end implement end method read pipelined method write( addr: UInt<ADDR_W>, data: UInt<DATA_W>, strobe: UInt<DATA_W/8> ) -> Future<AxiResp> timing: 2 cycles; max_outstanding: 1; implement: from_rtl protocol: Axi4Lite; end implement end method write end methods end interface Axi4Lite // Built-in from_rtl protocol drivers: // Axi4, Axi4Lite, Axi4Stream, AHB, AHBLite, APB, TileLink, Wishbone // Custom protocols can be registered as Arch interface modules.

22.12.4  Selecting Implementation Strategy at Simulation Time

The implementation strategy is selectable per-interface at simulation invocation, without changing source.  This enables a three-tier simulation flow from a single design source:

sim_tiers.arch # Tier 1: Functional TLM — maximum speed, no signal fidelity # Use: SW bring-up, firmware, architectural exploration # Speed: 100–1000× RTL baseline arch sim --tlm-lt AttnUnitTb.arch # Tier 2: Approximate TLM — nanosecond timing, no signal fidelity # Use: latency budgeting, throughput analysis, performance estimation # Speed: 10–50× RTL baseline arch sim --tlm-at AttnUnitTb.arch # Tier 3: Cycle-accurate TLM — full RTL signal fidelity behind method calls # Use: protocol correctness, AXI compliance testing, interface bring-up # Speed: 1–2× RTL baseline (small overhead from method call bookkeeping) arch sim --tlm-rtl AttnUnitTb.arch # Mixed: some interfaces at RTL, others at TLM # Use: when one subsystem needs signal-level verification but others do not arch sim --tlm-rtl dut.axi --tlm-at dut.hbm --tlm-lt dut.pcie AttnUnitTb.arch # Full RTL — no TLM abstraction anywhere, all ports driven as signals # Use: regression, formal, gate-level verification arch sim AttnUnitTb.arch

22.12.5  What Cycle-Accurate TLM Catches That AT/LT Cannot

Running a testbench with --tlm-rtl surfaces protocol-level bugs that the approximate timing model cannot detect, because the full signal handshake executes:

| Bug Class | AT/LT | Cycle-Accurate TLM | RTL Full Sim |
| --- | --- | --- | --- |
| arready deasserted mid-burst by target | Not visible — method returns after timing:N | Caught — FSM waits for real arready | Caught |
| rvalid without matching arid (ID mismatch) | Not visible | Caught — rid check in FSM | Caught |
| wvalid/awvalid issued in wrong order | Not visible | Caught — AW/W channel ordering verified | Caught |
| bvalid not asserted within N cycles | Not visible | Caught — timeout assert in FSM | Caught |
| Initiator violates max_outstanding | Compile-time warning only | Caught — ready stalls FSM correctly | Caught |
| Wrong ARLEN for burst length | Not visible — burst method hides length | Caught — arlen drives wire directly | Caught |
| Back-to-back transactions with no gap | Not visible — LT has no time | Caught — real cycle timing enforced | Caught |

22.12.6  The Refinement Workflow

The practical flow for an AI accelerator or SoC team proceeds through three stages, each using a different simulation tier from the same Arch source:

refinement_workflow.arch // ── Stage 1: Architecture (weeks 1–4) ──────────────────────────────── // All interfaces are LT. Simulation speed: ~500× RTL. // Goal: verify SW stack, API correctness, memory map, interrupt model. // arch sim --tlm-lt SystemTb.arch // ── Stage 2: Performance (weeks 5–8) ───────────────────────────────── // Memory interfaces switch to AT. Compute stays LT. // Goal: verify latency budgets, bandwidth, bottleneck identification. // arch sim --tlm-at dut.hbm --tlm-at dut.noc --tlm-lt dut.cpu SystemTb.arch // ── Stage 3: Protocol Verification (weeks 9–12) ─────────────────────── // Critical interfaces switch to rtl_accurate. Others stay AT. // Goal: AXI compliance, CDC correctness, handshake protocol fidelity. // arch sim --tlm-rtl dut.axi --tlm-at dut.hbm --tlm-lt dut.cpu SystemTb.arch // ── Stage 4: Full RTL Regression ────────────────────────────────────── // All TLM abstraction removed. Parallel simulation active. // Goal: final gate-level sign-off, coverage closure. // arch sim --parallel --cores 32 SystemTb.arch // The testbench source is identical across all four stages. // The design source is identical across all four stages. // Only the arch sim flags change.

◈  The same testbench sequence — tb_master.read(addr), check result, report — runs unchanged across all four stages.  The caller never knows whether the implementation is functional, approximate, or cycle-accurate.  This is the abstraction boundary working correctly.

22.12.7  Correctness Guarantee Across Abstraction Levels

The Arch compiler enforces that an rtl_accurate implement block is a correct refinement of its method signature:

| Property Checked | What It Means | How Enforced |
| --- | --- | --- |
| Completion reachability | complete future is reachable on all non-reset paths | Compiler performs reachability analysis; missing complete is a compile error |
| Port coverage | every RTL port in the interface is driven by exactly one implement block | Compiler checks port-to-implement assignment; undriven ports are errors |
| No cross-implement conflicts | two implement blocks for different methods do not drive the same port in the same cycle | Compiler inserts priority mux and warns if simultaneous drive is possible |
| Timing monotonicity | an rtl_accurate method takes at least as many cycles as its timing: annotation | Compiler emits warning if FSM can complete faster than timing: declares — signals a spec mismatch |
| max_outstanding enforcement | no more than N transactions simultaneously in the FSM | Compiler generates saturating counter; (N+1)th call blocks automatically |

22.13  Cycle Accuracy of Pipelined Method Calls

A pipelined method call returning Future<T> correctly models transaction concurrency — multiple calls in-flight, responses in order.  However, the degree of timing accuracy depends on the implementation strategy behind the method.  This section makes the distinction precise, because the difference matters for latency-sensitive designs.

22.13.1  What timing: N Gets Right and Wrong

The timing: N declaration models a fixed-latency response with no backpressure.  It correctly models concurrency semantics — a pipelined caller can issue multiple calls before any respond — but deviates from RTL in three specific ways:

timing_n_gap.arch // RTL actual waveform — arready deasserted for 2 cycles (target busy): // // cycle: 0 1 2 3 4 5 6 7 // arvalid: 1 1 1 0 0 0 0 0 // arready: 0 0 1 0 0 1 0 0 // target busy cycles 0–1, 4–5 // rvalid: 0 0 0 0 1 0 0 1 // // Call 0: issued c0, accepted c2, response c4. Future 0 resolves: cycle 4 // Call 1: issued c3 (after acceptance), accepted c5, response c7. Future 1 resolves: cycle 7 // pipelined + timing: 4 cycles — what the model produces: // // Call 0: issued cycle 0. Future 0 resolves: cycle 4 ✓ coincidentally correct // Call 1: issued cycle 0. Future 1 resolves: cycle 4 ✗ RTL says cycle 7 // // Three failures: // 1. Call 1 issued cycle 0 — wrong. RTL: cycle 3 (arready held call 0 for 2 cycles) // 2. Future 1 resolves cycle 4 — wrong. RTL: cycle 7 // 3. Backpressure invisible — both calls overlap fully regardless of target load

22.13.2  Backpressure — The Critical Failure Mode

The most consequential divergence is backpressure.  When a target deasserts arready because it is busy, the RTL initiator must hold arvalid and stall.  The timing: N model has no arready signal to observe — it issues all calls immediately at the declared rate, making the target appear always-ready.  This produces optimistic throughput estimates proportional to how congested the real target is.

backpressure_gap.arch // 8 pipelined reads, target has 50% arready availability: // RTL: calls issue every 2 cycles (arready throttles) // total time: 18 cycles // timing: 4 cycles: all 8 calls issue cycle 0, all Futures resolve cycle 4 // total time modeled: 4 cycles ← 4.5× optimistic // rtl_accurate: calls issue every 2 cycles (arready modeled) // total time: 18 cycles ← matches RTL exactly

⚑  ⚠ timing: N for pipelined methods can be severely optimistic when target arready availability is limited.  For accurate bandwidth modeling, use rtl_accurate on both sides.

22.13.3  Concurrency vs Timing Fidelity — Two Orthogonal Axes

|  | timing: N (approximate) | rtl_accurate (cycle-exact) |
| --- | --- | --- |
| blocking | Serial, fixed N-cycle latency, no backpressure | Serial, real FSM, backpressure modeled |
| pipelined | Concurrent issue, fixed N latency, no backpressure — overlap is optimistic | Concurrent issue, real handshake, backpressure propagated — cycle-accurate |
| out_of_order | Concurrent, fixed latency range, ID routing modeled, no backpressure | Concurrent, real FSM per ID, backpressure and ID interleaving — cycle-accurate |
| burst | One AR issue, N beats at fixed rate, no per-beat backpressure | One AR issue, real INCR burst, wlast/rlast modeled — cycle-accurate |

◈  Concurrency mode (pipelined, out_of_order, burst) describes the caller API contract — how many calls can be in-flight and what order responses arrive.  Timing fidelity (timing: N vs rtl_accurate) describes how accurately the implementation models real hardware.  These are independent choices.

22.13.4  Making Pipelined Calls Fully Cycle-Accurate

Cycle accuracy requires rtl_accurate on both the initiator and target implement blocks.  A cycle-accurate initiator driving a functional target achieves partial accuracy — issue timing is correct, but completion timing still reflects the target's fixed-latency response rather than real backpressure.

rtl_accurate_both.arch // Initiator — rtl_accurate: drives real AR/R signals implement axi.read(addr: UInt<32>) -> Future<UInt<64>> rtl_accurate; reg ar_sent: Bool init false; comb arvalid = not ar_sent; araddr = addr; end comb reg on clk rising, rst high if arvalid and arready ar_sent <= true; end if // waits for real arready end reg comb rready = ar_sent; end comb reg on clk rising, rst high if rvalid and rready complete future with rdata; // resolves at real rvalid cycle — not N end if end reg end implement axi.read // Target — rtl_accurate: asserts arready based on internal state implement mem.read(addr) -> UInt<64> rtl_accurate; reg busy: Bool init false; comb arready = not busy; end comb // deasserts when pipeline full reg on clk rising, rst high if arvalid and arready busy <= true; after 3 cycles // real SRAM latency rvalid <= true; rdata <= Storage.data[addr[27:3]]; after 1 cycle rvalid <= false; busy <= false; end after end after end if end reg end implement mem.read // With both rtl_accurate: the pipelined call sequence is cycle-for-cycle // identical to full RTL simulation of the same design.

22.13.5  When is timing: N Sufficient?

| Simulation Goal | timing: N sufficient? | Reason |
| --- | --- | --- |
| SW API correctness — does the driver issue the right call sequence? | Yes | Call ordering and response routing modeled; cycle count irrelevant |
| Functional data correctness — does the right data reach the right place? | Yes | Data flow modeled completely regardless of timing fidelity |
| Worst-case latency estimation | No | Backpressure absent — timing: N gives best-case only |
| Throughput under load | No | All calls overlap fully — optimistic by design |
| AXI protocol compliance (arvalid held, wlast on last beat) | No | Signal-level rules require signal transitions |
| Performance regression detection | Sometimes | Only catches regressions larger than N-cycle variance |

22.13.6  The Honest Summary

A pipelined method call with timing: N cycles is not cycle-accurate compared to the eventual RTL implementation.  It correctly models that multiple transactions are in-flight simultaneously and that responses arrive in order — the concurrency contract — but does not model arready backpressure, variable-latency responses, or the exact cycle at which each Future resolves under load.

Cycle accuracy requires rtl_accurate on both initiator and target implement blocks.  At that point the pipelined call API — issue Future, continue, await — becomes cycle-for-cycle identical to RTL simulation.  The abstraction stays clean; the fidelity is complete.

The timing: N mode is not intended to approximate RTL timing — it is intended to provide maximum simulation speed for use cases where cycle accuracy is not the goal.  The arch sim flags --tlm-lt, --tlm-at, and --tlm-rtl exist to make this tradeoff explicit per simulation run.

25.  AI-Assisted Hardware Design Workflow

Arch is designed for AI-assisted hardware generation.  This section describes how the workflow operates in practice — how a designer interacts with an AI assistant to produce correct Arch, why the full language specification does not need to be in the AI's context window, and what a minimal, effective AI context looks like.

25.1  Why the Full Spec Does Not Need to Be in Context

A large language model assisting with Arch hardware design does not need the complete specification in its context window.  Three properties of the language make this possible:

- Uniform construct schema: every Arch construct — fifo, pipeline, cam, crossbar, scoreboard — has identical structure: param / port / body / verification / end keyword Name.  An AI that knows the schema for one construct knows the shape for all of them.  Novel constructs do not require novel syntax.

- Compiler as verifier: the Arch compiler produces precise, structured error messages that name the construct, the signal, the type mismatch, and the line.  When generated code contains an error, the compiler output is sufficient context for the AI to self-correct — without consulting the spec.  The compiler replaces the spec in the feedback loop.

- Intent-to-construct mapping: hardware intent maps unambiguously to an Arch construct.  A FIFO is always fifo.  A state machine is always fsm.  A pipelined datapath is always pipeline.  The AI does not need to choose between implementation strategies — it needs to identify the right construct and fill in the schema.

25.2  The Core Workflow

The practical AI-assisted Arch design loop has four steps, typically completing in two to four iterations:

workflow_steps Step 1 — Designer states intent in natural language 'I need a 4-stage AXI4 read pipeline with a 16-entry scoreboard that handles out-of-order responses and stalls on structural hazards.' Step 2 — AI generates Arch The AI identifies the constructs (pipeline, scoreboard, interface Axi4), applies the uniform schema to each, and emits Arch source. It uses todo! for any body it is uncertain about. Step 3 — Designer runs the compiler arch check ReadPipeline.arch The compiler emits zero errors, or precise typed errors. Step 4 — AI self-corrects from compiler output Compiler error: 'ReadPipeline: port rid has type UInt<4> but scoreboard token expects UInt<ID_W>; declare param ID_W: const = 4 or widen token.' AI fixes the param declaration. No spec lookup required.

25.3  What Goes in the AI Context Window

The effective AI context for Arch hardware design has three components, all compact:

| Component | Size | Content | Purpose |
| --- | --- | --- | --- |
| Arch AI Reference Card | ~400 lines | Construct catalog, universal schema, type table, one example per construct | Activates construct knowledge — fits in one context window |
| Design intent | 5–20 lines | Natural language description of the block to build | Specifies what to generate |
| Compiler output (when iterating) | 5–30 lines | arch check error messages from the previous iteration | Replaces spec lookup for error correction |

◈  The full specification is a reference document for human designers learning the language.  The AI Reference Card is what an AI assistant actually uses.  It is structured differently — optimised for construct lookup and schema recall, not for human reading.

25.4  Effective Prompting Patterns

Several prompting patterns consistently produce high-quality Arch output:

Pattern 1 — Construct-First

construct_first_pattern // Tell the AI which construct to use. // The schema fills itself in from the construct name. // Instead of: // 'Design a hardware component that stores data in a ring buffer...' // Write: // 'Generate an Arch fifo construct named InstrQueue with: // depth 64, element type struct InstrPacket, single clock SysDomain. // Expose push_valid/push_ready and pop_valid/pop_ready ports. // Add a cover property for push_when_full.' // The construct name resolves all structural ambiguity. // The AI fills param / port / body / verification.

Pattern 2 — Interface-First

interface_first_pattern // Describe the interface contract before the implementation. // The AI generates the port bundle and TLM methods first, // then fills in the body to satisfy the contract. // 'The AttentionUnit module has: // - a TLM pipelined socket (initiator) on MemBusTlm for weight reads // - a blocking socket (target) on HostBus for register-mapped control // - ports in_valid/in_token (QKVToken)/out_valid/out_score (SInt<16>) // - parameter D_K: const (key dimension) // - parameter SEQ_LEN: const (max sequence length)', // The AI generates the full module skeleton with todo! bodies. // The designer fills in or iterates on the todo! blocks.

Pattern 3 — todo! Scaffolding

todo_scaffold_pattern // Use todo! to get a compilable skeleton quickly. // Ask the AI to generate the structure, not the full implementation. // 'Generate the Arch skeleton for a 5-stage RISC-V integer pipeline. // Use todo! for all stage bodies and hazard logic. // I will fill in the stages myself.' // The AI generates: pipeline RiscVPipe param XLEN: const = 32; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; // ... ports ... stage Fetch todo! end stage Fetch stage Decode todo! end stage Decode // ... 3 more stages ... stall when todo!; forward todo! from todo! when todo!; end pipeline RiscVPipe // This compiles. The designer iterates on each todo! independently. // Each stage can be handed back to the AI with its own focused prompt.

Pattern 4 — Compiler-Error-as-Prompt

compiler_error_pattern // Paste the full compiler output as the next prompt. // No spec lookup needed — the error is self-contained. // Designer prompt: // 'Fix this Arch compiler error: // // ERROR AttentionUnit.arch:47 // cdc_crossing: signal score_raw crosses from ComputeDomain to OutputDomain // without a declared crossing block. // Add: crossing compute_to_output { from ComputeDomain to OutputDomain // data score_raw -> score_sync } or register score_raw in OutputDomain.' // The AI reads the error, identifies the missing crossing block, // and inserts it at the correct location. // It does not need to look up CDC rules — the error contains them.

25.5  The Self-Correction Loop

When the AI generates Arch with errors, the compiler output is sufficient for self-correction in the majority of cases.  Arch error messages are designed with AI consumers in mind — they name the construct, the exact signal, the type mismatch, and suggest the fix.

compiler_errors // Examples of Arch compiler errors that are self-sufficient for AI correction: // Type width mismatch: // ERROR SystolicPE.arch:23 // reg acc_reg: SInt<32> += SInt<8> * SInt<8> // Product SInt<16> cannot be assigned to SInt<32> without explicit extension. // Use: (a.sext<32>() * b.sext<32>()).trunc<32>() or sext both operands first. // Missing port connection: // ERROR SystolicArray.arch:41 // inst pe[3]: SystolicPE — port sum_in not connected. // Expected: connect sum_in <- pe[2].sum_out (or boundary value for i=0) // CDC violation: // ERROR DmaEngine.arch:78 // Signal req_valid: domain WriteDomain drives register in ReadDomain. // Add a crossing block or use a fifo with wr_clk / rd_clk ports. // Unreachable complete in rtl_accurate implement block: // WARNING Axi4Transactor.arch:112 // implement axi.read — 'complete future' not reachable when rvalid is never asserted. // Ensure complete future with rdata appears on all non-reset paths. // Each error contains: location, construct, signal names, type information, fix suggestion. // The AI can apply the fix without accessing the specification.

25.6  Scaling Complexity — One Construct at a Time

Large hardware systems in Arch are built by composing constructs.  An AI assistant is most effective when each prompt targets one construct.  This matches the language's structure and keeps each prompt focused and verifiable.

incremental_prompts // Building a full AI accelerator — prompt sequence: // Prompt 1: 'Generate the QKVToken struct and ScoreResult struct' // Prompt 2: 'Generate the WeightCache ram construct, 256 entries, UInt<8>' // Prompt 3: 'Generate the SystolicPE module — single MAC, SInt<8> inputs' // Prompt 4: 'Generate the SystolicArray pipeline using SystolicPE with // generate for SIZE PEs' // Prompt 5: 'Generate the ScoreScaler pipeline — dot product + RECIP_SQRT' // Prompt 6: 'Generate the AttentionUnit module connecting the above' // Prompt 7: 'Generate the testbench for AttentionUnit with 3 test sequences' // Each prompt generates one construct. // Each construct is compiled independently before the next is started. // Errors are caught and fixed at the smallest possible scope. // The AI reference card stays in context throughout.

◈  This one-construct-per-prompt discipline is not a limitation — it is the correct way to design hardware.  Each construct is a verified unit.  Composing verified units produces a correct system faster than generating the entire system at once and debugging globally.

25.7  Abstraction-Level Progression

For complex blocks, the AI workflow proceeds through abstraction levels — from TLM behavioral to cycle-accurate RTL — using the same prompting discipline:

abstraction_progression // Level 1 — TLM skeleton (fastest to generate, fastest to simulate) // Prompt: 'Generate the AttentionUnit module with TLM pipelined sockets. // Use todo! for all internal logic.' // Purpose: verify the API contract and testbench before building internals. // Level 2 — Behavioral RTL (correct function, no timing) // Prompt: 'Replace the todo! in the Compute stage with a behavioral // implementation of scaled dot-product. Timing does not matter yet.' // Purpose: verify functional correctness with fast simulation. // Level 3 — Pipelined RTL (correct timing, pipeline structure) // Prompt: 'Rework the Compute stage as a 3-stage pipeline: // stage 1: dot product accumulate, stage 2: scale, stage 3: output.' // Purpose: verify cycle-level behaviour and latency. // Level 4 — Resource-optimised (FPGA/ASIC target) // Prompt: 'Optimise the dot product stage for Xilinx UltraScale DSP48E2 blocks. // Map the SInt<8>*SInt<8> multiplies to DSP cascade chains.' // Purpose: target-specific optimisation from verified baseline. // Each level uses arch sim to verify before proceeding. // The testbench is identical at all four levels.

24.  Interfaces

An interface is a named, typed port bundle reusable across module, pipeline, fifo, and arbiter boundaries.  Interfaces eliminate the repetitive port declarations that make large designs brittle.  Because the schema is identical to all other constructs, the AI applies the same template.

11.1  Declaring and Using Interfaces

interface.arch /// AXI4-Lite manager interface interface Axi4Lite param DWIDTH: const = 32; port awvalid: out Bool; port awready: in Bool; port awaddr: out UInt<32>; port wvalid: out Bool; port wready: in Bool; port wdata: out UInt<DWIDTH>; port wstrb: out UInt<DWIDTH/8>; port bvalid: in Bool; port bready: out Bool; port bresp: in UInt<2>; end interface Axi4Lite // Declare as a single port — compiler flips directions for subordinate side module DmaController port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port mgr: out Axi4Lite param DWIDTH = 64; end port mgr // ... end module DmaController

⚑  Declaring a port as out InterfaceName makes all interface outputs drive outward and all interface inputs expect inward.  Directions flip automatically for the subordinate side.

25.  Lightweight Verification Built-ins

Arch includes three verification constructs built directly into the language.  They use the same types, signals, and naming as the design — there is no separate verification sublanguage to learn.

12.1  assert — Invariant Checking

assert.arch module SafeAdder port a: in UInt<8>; port b: in UInt<8>; port sum: out UInt<9>; comb sum = a.zext<9>() + b.zext<9>(); end comb // Fires in simulation; formal tools prove it statically assert no_overflow: a.zext<9>() + b.zext<9>() <= 9'h0FF; end module SafeAdder

12.2  cover — Reachability

cover.arch fifo TxQueue // ... ports as before ... cover full_seen: full == true; cover empty_seen: empty == true; end fifo TxQueue

12.3  assume — Formal Constraints

assume.arch module FormalWrapper port req: in UInt<4>; // Constrain the formal tool: only consider one-hot inputs assume one_hot: req == 0 or (req & (req - 1)) == 0; end module FormalWrapper

| Keyword | Simulation | Formal Tool | Meaning |
| --- | --- | --- | --- |
| assert name: expr | Error if false | Property to prove | Invariant — must always hold |
| cover name: expr | Log when true | Reachability check | Confirms a scenario is reachable |
| assume name: expr | Ignored | Input constraint | Restricts the input space for formal analysis |

26.  AI-Generatability Design Rationale

This section documents the explicit language decisions made so that any LLM — without hardware-specific fine-tuning — can produce correct Arch from a plain-English description.

13.1  Uniform Declaration Schema

Every first-class construct uses the same four-section layout.  The AI applies one template universally:

universal_template.arch // Valid for: module, pipeline, fsm, fifo, arbiter, regfile, interface keyword Name param NAME: const = value; // section 1: compile-time parameters param NAME: type = SomeType; port name: in Type; // section 2: ports (direction always explicit) port name: out Type; port name: inout Type; // section 3: construct-specific body assert name: expr; // section 4: verification (optional) cover name: expr; end keyword Name

13.2  Named Endings Prevent Structural Hallucination

The most common failure mode in LLM-generated code is incorrect nesting — closing the wrong block or leaving one open.  Named endings make this a hard compiler error:

named_endings.arch // The compiler matches each ending to its opener by keyword AND name. // Mismatches are errors. The AI always knows exactly where it is. end stage Fetch end pipeline Decode end module Core // Compare with traditional HDLs where all closers look identical: // end end end endmodule ← which block does each close?

13.3  Directional Connect Arrows Encode Data Flow

In Verilog, signal assignment direction is determined by context — LLMs frequently confuse this.  In Arch, every connect statement encodes direction explicitly:

connect_arrows.arch connect data_in <- local_signal; // <- drives an input FROM a local signal connect data_out -> local_signal; // -> reads an output INTO a local signal // Direction is visible in the syntax itself. // An AI cannot silently reverse data flow.

13.4  todo! Enables Incremental AI-Assisted Design

A practical AI workflow: generate a correct skeleton with todo! for all logic, then fill in logic section by section.  Every intermediate state compiles and type-checks.

todo_workflow.arch // Step 1: AI generates skeleton — compiles immediately pipeline RiscVCore param XLEN: const = 32; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port imem_addr: out UInt<XLEN>; port imem_data: in UInt<32>; stage Fetch comb imem_addr = todo!; end comb end stage Fetch stage Decode comb todo!; end comb end stage Decode stage Execute comb todo!; end comb end stage Execute end pipeline RiscVCore // Step 2: fill in Fetch — recompile, types checked // Step 3: fill in Decode — recompile, etc.

13.5  No Implicit Behaviour — Full Explicitness Table

| Traditional HDL implicit | Arch explicit equivalent |
| --- | --- |
| Undriven output defaults to high-Z | Compile error — every output must have exactly one driver |
| reg with no reset holds unknown state | Reset value required: reg x: UInt<8> init 0; |
| Wire driven by last assignment wins | Compile error — single-driver rule enforced statically |
| Unsigned/signed determined by context | Explicit: .zext<N>() .sext<N>() as SInt<N> |
| Clock inferred from sensitivity list | Explicit: reg on clk rising, rst high |
| Module port width from implicit param math | Explicit: port sum: out UInt<WIDTH+1>; |
| CDC implicitly allowed across assignments | Compile error — crossing block required |

27.  Compilation and Output

14.1  Compiler Pipeline

| Phase | Key Checks and Actions |
| --- | --- |
| Parse | Syntax, named-block matching, naming convention enforcement |
| Elaboration | Parameter resolution, generic instantiation, type expansion |
| Type Check | Bit-width safety, clock domain tracking, direction safety, single-driver rule |
| Micro-Arch Lowering | Pipeline hazard generation, FIFO implementation, arbiter logic, FSM encoding |
| Verification Emit | assert/cover/assume converted to SystemVerilog Assertions (SVA) |
| SV Emit | One deterministic, lint-clean SystemVerilog file per Arch top-level module |

14.2  Output Guarantee

The generated SystemVerilog is guaranteed to contain none of the following:

- Latches — all state is explicit reg with init.

- Multiply-driven nets — enforced by the single-driver rule.

- Unresolved high-Z outputs — every output port has exactly one driver.

- X-propagation from uninitialised state — all reg declarations require init.

- Implicit clock-domain crossings — all CDCs are declared and synchroniser-wrapped.

⚑  The output guarantee means synthesis tools receive RTL that passes lint clean with no attributes, no translate_off pragmas, and no synthesis workarounds required.

14.3  Toolchain Targets

| Target | Command | Notes |
| --- | --- | --- |
| SystemVerilog (ASIC) | arch build --target asic | IEEE 1800-2017 output, no vendor primitives |
| SystemVerilog (FPGA Xilinx) | arch build --target fpga --vendor xilinx | Inserts BRAM/DSP primitives where appropriate |
| SystemVerilog (FPGA Intel) | arch build --target fpga --vendor intel | Inserts Intel BRAM/DSP primitives |
| Formal verification | arch build --target formal | SVA + SymbiYosys script |
| Simulation | arch sim --tb MyTb | Compiles with Verilator or ModelSim/Questa |
| Documentation | arch doc | HTML reference from /// doc comments |

28.  Complete Example: 3-Stage RISC-V Integer Pipeline

The following example shows all major Arch constructs working together — module, pipeline with stall and forward, ram with multi-variable mapping, fifo, arbiter, and verification.  No braces appear anywhere.  Every block is opened by its keyword and name, and closed by end keyword name.

Design: a 3-stage in-order RISC-V integer pipeline with a unified register + CSR RAM (multi-variable mapping), a shared memory bus arbitrated between instruction fetch and data memory, and a 4-entry instruction queue between fetch and decode.

28.1  Shared Types and Domains

types.arch domain SysDomain freq_mhz: 100, end domain SysDomain struct DecodedInstr opcode: UInt<7>, rd: UInt<5>, rs1: UInt<5>, rs2: UInt<5>, funct3: UInt<3>, imm: SInt<32>, alu_op: AluOp, end struct DecodedInstr enum AluOp Add, Sub, And, Or, Xor, Sll, Srl, Sra, Slt, Sltu, end enum AluOp

28.2  Unified Register and CSR RAM

unified_regs.arch // A single physical RAM holds integer registers and CSRs. // The compiler assigns non-overlapping address ranges automatically: // int_regs → 0x000 .. 0x01F (32 entries × 32 bits) // csr_regs → 0x020 .. 0x41F (1024 entries × 32 bits) // Total: depth = 1056, word width = 32 bits // // Designers access int_regs[rs1] and csr_regs[addr] by name. // The compiler emits physical_base + index in the generated SystemVerilog. ram UnifiedRegs port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; kind simple_dual; read: async; // combinational read — register-file behaviour store int_regs: Vec<UInt<32>, 32>; csr_regs: Vec<UInt<32>, 1024>; end store port read_port en: in Bool; addr: in UInt<$clog2(1056)>; data: out UInt<32>; end port read_port port write_port en: in Bool; addr: in UInt<$clog2(1056)>; data: in UInt<32>; end port write_port init: zero; // x0 and all CSRs initialise to zero on reset end ram UnifiedRegs

28.3  Memory Bus Interface and Supporting Constructs

support.arch interface MemBus port valid: out Bool; port ready: in Bool; port addr: out UInt<32>; port rdata: in UInt<32>; port wdata: out UInt<32>; port wen: out Bool; end interface MemBus // Priority arbiter — Fetch (port 0) beats LSU (port 1) arbiter MemArbiter param NUM_REQ: const = 2; param NUM_RSRC: const = 1; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; ports[NUM_REQ] request valid: in Bool; ready: out Bool; end ports ports[NUM_RSRC] grant valid: out Bool; requester: out UInt<1>; end ports policy priority; end arbiter MemArbiter // 4-entry instruction queue between Fetch and Decode stages fifo InstrQueue param DEPTH: const = 4; param WIDTH: type = UInt<32>; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port push_valid: in Bool; port push_ready: out Bool; port push_data: in UInt<32>; port pop_valid: out Bool; port pop_ready: in Bool; port pop_data: out UInt<32>; port full: out Bool; port empty: out Bool; end fifo InstrQueue

28.4  The 3-Stage Pipeline

pipeline.arch pipeline RiscVPipe param XLEN: const = 32; port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port imem: out MemBus end port imem port dmem: out MemBus end port dmem // Register file read results arrive combinationally (async RAM) port rs1_val: in UInt<XLEN>; port rs2_val: in UInt<XLEN>; port rd_wen: out Bool; port rd_addr: out UInt<5>; port rd_data: out UInt<XLEN>; stage Fetch reg pc: UInt<32> init 0; reg on clk rising, rst high if rst pc <= 0; end if else pc <= pc + 4; end else end reg comb imem.addr = pc; imem.valid = true; imem.wen = false; imem.wdata = 0; end comb end stage Fetch stage Decode let raw: UInt<32> = imem.rdata; comb todo!; // instruction decode — fills DecodedInstr fields end comb end stage Decode stage Execute comb todo!; // ALU, branch resolution, LSU address generation end comb end stage Execute stall when iq_full == true; forward Decode.rs1_val from Execute.rd_data when Execute.instr.rd == Decode.instr.rs1 and Execute.instr.rd != 0; forward Decode.rs2_val from Execute.rd_data when Execute.instr.rd == Decode.instr.rs2 and Execute.instr.rd != 0; assert pc_aligned: Fetch.pc[1:0] == 2'b00; cover fetch_stall: iq_full == true; end pipeline RiscVPipe

28.5  Top-Level Module

top.arch module RiscVCore port clk: in Clock<SysDomain>; port rst: in Reset<Sync>; port imem: out MemBus end port imem port dmem: out MemBus end port dmem // ── Unified register + CSR RAM ─────────────────────────────── inst regs: UnifiedRegs connect clk <- clk; connect rst <- rst; // Read port: address is either int_regs[rs1] or csr_regs[csr_idx] // Compiler translates logical names to physical addresses connect read_port.en <- true; connect read_port.addr <- int_regs[rs1_addr]; connect read_port.data -> rs1_val; // Write port: writeback from execute stage connect write_port.en <- rd_wen; connect write_port.addr <- int_regs[rd_addr]; connect write_port.data <- rd_data; end inst regs // ── Instruction queue ──────────────────────────────────────── inst iq: InstrQueue param DEPTH = 4; param WIDTH = UInt<32>; connect clk <- clk; connect rst <- rst; connect push_valid <- imem.valid; connect push_data <- imem.rdata; connect push_ready -> imem.ready; connect pop_ready <- true; connect full -> iq_full; end inst iq // ── Memory bus arbiter ─────────────────────────────────────── inst arb: MemArbiter param NUM_REQ = 2; param NUM_RSRC = 1; connect clk <- clk; connect rst <- rst; connect request[0].valid <- imem.valid; connect request[0].ready -> imem.ready; connect request[1].valid <- dmem.valid; connect request[1].ready -> dmem.ready; connect grant[0].valid -> bus_grant; connect grant[0].requester -> bus_winner; end inst arb // ── Pipeline ───────────────────────────────────────────────── inst pipe: RiscVPipe param XLEN = 32; connect clk <- clk; connect rst <- rst; connect imem -> imem; connect dmem -> dmem; connect rs1_val <- rs1_val; connect rs2_val <- rs2_val; connect rd_wen -> rd_wen; connect rd_addr -> rd_addr; connect rd_data -> rd_data; end inst pipe end module RiscVCore

◈  The entire design compiles with two todo! sites as the only incomplete logic.  All types are checked, clock domains verified, all ports driven, pipeline hazard annotations structurally correct, and the RAM's logical variable mapping fully resolved.  An AI fills in the two todo! sites independently without touching anything else.

29. Packages and Imports

29.1 The package Construct

A package groups related type definitions, constants, and functions into a reusable namespace. Packages follow the universal block grammar:

package PkgName // enum, struct, function, and param declarations end package PkgName

What can go inside a package:

| Item | Example |
| --- | --- |
| enum | enum BusOp Read, Write, Idle end enum BusOp |
| struct | struct BusReq op: BusOp; addr: UInt<32>; end struct BusReq |
| function | function max(a: UInt<32>, b: UInt<32>) -> UInt<32> ... end function max |
| param | param BUS_WIDTH: const = 64; |
| domain | domain FastClk freq_mhz: 500 end domain FastClk |

Packages may not contain modules, pipelines, FSMs, or any other construct that produces hardware. They are purely compile-time organizational units.

29.2 The use Import

A consumer file imports a package with the use statement at file scope (before any construct declaration):

use PkgName;

This makes all names defined inside the package available unqualified in the importing file. Multiple use statements are allowed.

29.3 File Resolution

The compiler resolves use PkgName; by searching for PkgName.arch in the same directory as the importing file. The package file must contain exactly one package PkgName ... end package PkgName construct whose name matches the file name.

When using multi-file compilation (arch build a.arch b.arch), packages included on the command line are also resolved.

29.4 Complete Example

Source file BusPkg.arch:

package BusPkg

enum BusOp

Read, Write, Idle

end enum BusOp

struct BusReq

op: BusOp;

addr: UInt<32>;

data: UInt<32>;

end struct BusReq

function max(a: UInt<32>, b: UInt<32>) -> UInt<32>

return a > b ? a : b;

end function max

end package BusPkg

Consumer file Consumer.arch:

use BusPkg;

module Consumer

port clk: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port req: in BusReq;

port addr_out: out UInt<32>;

comb addr_out = req.addr;

end module Consumer

29.5 Generated SystemVerilog

The compiler emits a standard SV package / import pair:

SystemVerilog

package BusPkg;

typedef enum logic [1:0] { READ, WRITE, IDLE } BusOp;

typedef struct packed {

BusOp op;

logic [31:0] addr;

logic [31:0] data;

} BusReq;

function automatic logic [31:0] max(input logic [31:0] a, input logic [31:0] b);

return (a > b) ? a : b;

endfunction

endpackage

import BusPkg::*;

module Consumer (

input wire clk,

input wire rst,

input BusReq req,

output logic [31:0] addr_out

);

assign addr_out = req.addr;

endmodule

Each use PkgName; in Arch emits one import PkgName::*; in SV, placed immediately before the consuming module.

29. Packages and Imports

29.1 The package Construct

A package groups related type definitions, constants, and functions into a reusable namespace. Packages follow the universal block grammar:

package PkgName // enum, struct, function, and param declarations end package PkgName

What can go inside a package:

| Item | Example |
| --- | --- |
| enum | enum BusOp Read, Write, Idle end enum BusOp |
| struct | struct BusReq op: BusOp; addr: UInt<32>; end struct BusReq |
| function | function max(a: UInt<32>, b: UInt<32>) -> UInt<32> ... end function max |
| param | param BUS_WIDTH: const = 64; |
| domain | domain FastClk freq_mhz: 500 end domain FastClk |

Packages may not contain modules, pipelines, FSMs, or any other construct that produces hardware. They are purely compile-time organizational units.

29.2 The use Import

A consumer file imports a package with the use statement at file scope (before any construct declaration):

use PkgName;

This makes all names defined inside the package available unqualified in the importing file. Multiple use statements are allowed.

29.3 File Resolution

The compiler resolves use PkgName; by searching for PkgName.arch in the same directory as the importing file. The package file must contain exactly one package PkgName ... end package PkgName construct whose name matches the file name.

When using multi-file compilation (arch build a.arch b.arch), packages included on the command line are also resolved.

29.4 Complete Example

Source file BusPkg.arch:

package BusPkg

enum BusOp

Read, Write, Idle

end enum BusOp

struct BusReq

op: BusOp;

addr: UInt<32>;

data: UInt<32>;

end struct BusReq

function max(a: UInt<32>, b: UInt<32>) -> UInt<32>

return a > b ? a : b;

end function max

end package BusPkg

Consumer file Consumer.arch:

use BusPkg;

module Consumer

port clk: in Clock<SysDomain>;

port rst: in Reset<Sync>;

port req: in BusReq;

port addr_out: out UInt<32>;

comb addr_out = req.addr;

end module Consumer

29.5 Generated SystemVerilog

The compiler emits a standard SV package / import pair:

SystemVerilog

package BusPkg;

typedef enum logic [1:0] { READ, WRITE, IDLE } BusOp;

typedef struct packed {

BusOp op;

logic [31:0] addr;

logic [31:0] data;

} BusReq;

function automatic logic [31:0] max(input logic [31:0] a, input logic [31:0] b);

return (a > b) ? a : b;

endfunction

endpackage

import BusPkg::*;

module Consumer (

input wire clk,

input wire rst,

input BusReq req,

output logic [31:0] addr_out

);

assign addr_out = req.addr;

endmodule

Each use PkgName; in Arch emits one import PkgName::*; in SV, placed immediately before the consuming module.

ARCH Language Specification v0.1  ·  March 2026  ·  Draft for Review